| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Change request for network coding zone |  |
| Date Submitted | 2009-2-27 |  |
| Source(s) | Kanchei(Ken) Loa, Jiun-Je Jian, Chun-Yen Hsu, Tsung-Yu Tsai, Youn-Tai Lee, Yi-Ting Lin, Chiu-Wen Chen Institute for Information Industry (III) | Voice: +886-2-66000100 E-mail: loa@iii.org.tw |
| Re: | Change request for IEEE 802.16m System Description Document (802.16m-08/003r7) |  |
| Abstract | This contribution proposes modifications on network coding zone |  |
| Purpose | For discussion and approval by IEEE 802.16 Working Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Change request for network coding zone

Kanchei(Ken) Loa, Jiun-Je Jian, Chun-Yen Hsu, Tsung-Yu Tsai,

Youn-Tai Lee, Yi-Ting Lin, Chiu-Wen Chen

Institute for Information Industry (III)

# Introduction

The value of network coding is that it can simultaneously transmit same processed information to a super-ordinate station and sub-ordinate stations, which can reduce the relaying overhead.

The power control is a critical issue for network coding schemes. In SDD (IEEE802.16m_08/003r7), the ARS directly attached to the ABS can simultaneously transmit data to ABS(Up-link) and sub-ordinate ARS (Down-link) in the “Network Coding Transmit Zone”. The power level by which the ARS transmit data in the “Network Coding Transmit Zone” needs to be determined.

If the network coding transmitting power follows the Up-link power control instruction from the ABS (see Fig.1) , the sub-ordinate ARS may not be able to decode the data when the channel conditions of the ARS-ABS link and the ARS-ARS link are different. On the contrary, if the network coding transmitting power follows the Down-link power level (see Fig.2), it may cause interference to the ABS.

Therefore, it is not adequate to include the network coding zone in the relay frame structure without a clear description of the power control scheme and the subframe structure within the network coding zone. We propose to remove the network coding zone from the relay frame structure.

Fig.1 , Network coding transmit zone following       Fig.2 , Network coding transmit zone following

Up-link power control                       Down-link power level

# Proposed Changes

[remove the 16m Network Coding Receive Zone and Network Coding Transmit Zone from Figure31,Page68]

[remove the following text from Line 13 to 24,Page69 as indicated:]

.

- Network Coding Transmit Zone: An integer multiple of subframes located in the DL of the frame of the Odd Hop ARS which is directly attached to the ABS, where an Odd Hop ARS can transmit network coded transmissions to the ABS and Even Hop ARS. Transmissions to the AMS in this zone are FFS.

- Network Coding Receive Zone: An integer multiple of subframes located in the DL of the ABS or Even Hop ARS frame, where an ABS or Even Hop ARS can receive network coded transmissions from the Odd Hop ARS.

If the ABS supports network coding, the presence of the aforementioned zones is determined by the ABS depending on the number of hops and the ARS capabilities. The Network Coding Transmit Zone may be present in a ARS frame if the ARS supports network coding. If the Network Coding Transmit Zone is present, it appears only in the frame of a ARS which is directly attached to the ABS. The Network Coding Receive Zone may be present only in the frames of the ABS and the even hop ARS that is two hops away from the ABS, if the ARS and the ABS support network coding.

[remove the following text from Line 12 to 13,Page132 as indicated:]

ARS may transmit data to the super-ordinate and sub-ordinate station(s) using the same LRU (e.g., MIMO, network coding, etc)

[Insert the following text in Line 25,Page69 as indicated:]

ARS may simultaneously transmit data to the super-ordinate and sub-ordinate station(s). The details are FFS.