| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Performance evaluation of MIMO Midamble |  |
| Date Submitted | 2009-02-25 |  |
| Source(s) | Woosuk Kwon Wookbong Lee Jinyoung Chun Sukwoo Lee Bin-Chul Ihm LG Electronics | kytloze@lge.com wbong@lge.com jychun03@lge.com sugoo@lge.com bcihm@lge.com |
| Re: | Call for Comments on Amendment Working Document, IEEE 802.16m-09/0012 |  |
| Abstract | The contribution proposes the text of downlink reference signal section to be included in the 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Performance evaluation of MIMO Midamble

Woosuk Kwon, Wookbong Lee, Jinyoung Chun, Sukwoo Lee and Bin-Chul Ihm

LG Electronics

- Introduction

The contribution proposes the MIMO midamble structure for channel estimation of TX antennas. And it also can be used for estimating channel states of neighbor cells for interference mitigation techniques. (See the session 20.2 in SDD.)

- Considerations for MIMO midamble structure

The demodulate pilots are used for data detection. To estimate CQI or PMI properly, the downlink reference signal shall not be precoded and the AMS shall know exactly which pilot pattern was used. Since A-MAP is dedicated to a certain AMS, AMS does not know which pilot pattern was used or even the pilots are precoded or not. So, we need to have a downlink reference signal to estimate CQI or PMI. We can consider reuse the demodulated pilot pattern for downlink reference signal by using a whole subframe’s pilots as common pilot. However, for sounding based precoding or non-unitary codebook based precoding, it is difficult to indicate which precoder was used in the subframe. Moreover, it increases pilot overhead.

The key requirements for MIMO midamble structure are as follows.

- Midamble shall avoid existing demodulate pilots in order not to damage channel estimation performance.

- In pair-tone based DRU, the remaining data tones shall be able to be paired for supporting SFBC.

- AMS shall be able to estimate channel of all physical transmit antennas in the serving and neighbor cells.

- The structure of MIMO midamble

In SDD, measurement pilot (midamble) section is specified. In order to construct midamble pattern, pilot position should be considered. Figure 1 (a) is a pilot pattern for 2 streams and (b) is a pilot pattern for 4 streams. In figure 1 (c), ‘P’ shows pilot location to gather all pilot including (a), (b) and interlaced pattern.

(a) 2 stream pilot pattern	(b) 4 stream pilot pattern	(c) all pilot location

Figure 1 Pilot patterns of a RU in SDD text

Figure 2 (a) shows a proposed midamble pattern for 2Tx antennas. Each antenna has 3 midambles in a RU. Figure 2 (b) shows a proposed midamble pattern for 4Tx antennas. Midamble pattern for 8 Tx antennas is FFS. In these figures, ‘Mk’ shows the position of midamble for antenna k.

(a) Midamble pattern for 2Tx		(b) Midamble pattern for 4Tx

Figure 2 Proposed midamble patterns in a RU

- The performance of proposed MIMO midamble pattern

OL-SU-MIMO was evaluated in noise limited and interference limited scenario. In each case, AMS estimates the channels of antennas with MIMO midamble and calculates the values of MCS, Rank and PMI. In order to adjust on target error rate, CQI modification is performed using ACK/NAK.

Figure 3 2Tx midamble performance in OL-MIMO

We observe the performance of proposed midamble patterns are more close to perfect channel measurement than 16e midamble. Using proposed midamble, data channel estimation performance is better than 16e midamble and overhead is lower than 16e midamble. As a result, we recommend the above patterns to measurement pilot.

- Appendix

There are simulation parameters.

| Parameter | Values |
| --- | --- |
| Channel Bandwidth | 10MHz |
| Over-sampling Factor | 28/25 |
| FFT Size | 1024 |
| Cyclic Prefix (CP) ratio | 1/8 |
| Resource Unit | CRU (4x18x6) |
| Channel condition | mPED_B 3km/h |
| The number of antennas | 2Tx, 2Rx |
| Channel estimation for measurement | 2D-MMSE |
| Channel estimation for demodulation | 2D-MMSE |
| Receiver | MMSE receiver |
| Report period | 5ms |
| OL-SU-MIMO schemes | SFBC with non adaptive precoder, SM with non adaptive precoder (rank adaptation) |

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

- Text proposal for inclusion in the 802.16m SDD

-------------------------------  Text Start  ---------------------------------------------------

Line 34, page 34, insert 15.3.5.4.2 Measurement Pilot section

15.3.5.4.2 Measurement Pilot

MIMO midamble is used for measurement of channel quality indicators, precoding matrix index and so on.

There are midamble patterns in Figure xxx.

(a) Midamble pattern for 2Tx		(b) Midamble pattern for 4Tx

Figure xxx. Midamble patterns

-------------------------------  Text End  ---------------------------------------------------