| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Rate Compatible LDPC-Convolutional Codes for the Change Request to 802.16m SDD |  |
| Date Submitted | 2009-02-27 |  |
| Source(s) | Takaaki KISHIGAMI, Yutaka MURAKAMI, Isamu YOSHII Panasonic Corporation | Voice: +81-46-840-5123 E-mail: kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | Change Request to 802.16m SDD (802.16m-08/003r7) - Target topic: “11.13 Channel Coding and HARQ ”. |  |
| Abstract | This contribution is detail description for supporting SDD Change Request (C802.16m-09/0415) that propose LDPC as an optional 16m FEC encoder. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Rate Compatible LDPC-Convolutional Codes

for the Change Request to 802.16m SDD

Takaaki KISHIGAMI, Yutaka MURAKAMI, Isamu YOSHII

Panasonic Corporation

# Introduction

This contribution is the detail description for supporting SDD Change Request (C802.16m-09/0415) that proposes to support LDPC codes as an optional 16m FEC encoder.

# Proposed Rate Compatible LDPC-CC

LDPC codes enable not only to achieve as good performance as Turbo codes, but also to achieve high-speed decoding, thanks to parallel processing with message-passing decoding. However, current 802.16e LDPC [1] does not support IR (incremental redundancy) type HARQ which offers potential for better performance than Chase combining. We describe on rate compatible LDPC-CC which have ability to support IR type HARQ[2][3] for the 16m FEC.

# 2.1 FEC Encoding based on LDPC-CC

The FEC Scheme block diagram is shown in Fig.1. The block is composed of LDPC-CC Encoder and a Puncture block, where payload data length is M bits and code bit length after Puncture is n bits.

M

2M

to modulator

Fig. 1:  FEC encoder.

# 2.2 LDPC-CC Encoding

Payload data is coded by LDPC-CC encoder. LDPC-CC Encoder is shown in Fig. 2.

LDPC-CC Encoder: - Payload input M bits then outputs are “Systematic Bits (M bits)” and “Parity Bits (also M bits)”. LDPC-CC Encoder coding rate is 1/2.

LDPC Convolutional Encoding process is as follows:

- Information Bits (M bits) are streamed to two blocks. One is for Systematic Bits (M bits) and another is for Constituent Encoder input.

- Constituent Encoder performs coding process for “Information Bits (M bits)”, then outputs “Parity Bits (M bits)”.

LDPC-CC Encoder outputs “Code Bits (a pair bits)”, sequentially {d0,p0}, {d1,p1}, {d2,p2}, …,{dM-1,pM-1}.

M

M

M

ut

dt

pt

Fig. 2:  Structure of LDPC-CC encoder.

LDPC-CC is defined by a parity check matrix H. A general description of parity check matrix H is shown below.

H=hd(0)(ms)hp(0)(ms)hd(0)(ms−1)hp(0)(ms−1)⋯⋯hd(0)(0)hp(0)(0)0000hd(1)(ms)hp(1)(ms)⋯⋯hd(1)(1)hd(1)(1)hd(1)(0)hd(1)(0)0000⋱⋮⋮⋮⋮00⋯⋯00⋮⋮⋱⋯hd(M−2)(0)hd(M−2)(0)⋮⋮⋯hd(M−1)(1)hd(M−1)(1)hd(M−1)(0)hd(M−1)(0) --- (1)

The parity check matrix H is an M×N binary matrix. Each column corresponds to “Systematic Bits (d-ms,…, d0,…, dM-1)” and “Parity Bits(p-ms,…, p0,…, pM-1)”, where the order is : d-ms, p-ms,…, d0, p0, …, dM-1, pM-1, and ms is LDPC-CC memory length.

Each row represents a parity-check polynomial. hd(i)(t)(t=0,…,ms) is the Systematic Bit weight (“1(one)” or “0(zero)”) at i-th order parity-check  polynomial, and hp(i)(t)(t=0,…, ms) is the Parity Bit weight (“1(one)” or “0(zero)”) at i-th order parity-check polynomial.

In the matrix H, all elements are “0(zero)” other than hd(i)(t) and hp(i)(t). As shown in equation (1), “1”s are placed around a diagonal line in the LDPC-CC matrix H.

We consider convolutional codes with the coding rate R=1/2 which have generation matrix G=[1 G1(D)/G0(D)]. Note that G0 is a feed back polynomial and G1 is a feed forward polynomial. Information and parity using polynomial are expressed as X(D) and P(D). A parity check polynomial with X(D) and P(D) is expressed as:

Check-equations for proposed FEC scheme are as follows:

G1DXD+G0DPD=0                                                                  (2)

Based on the parity check polynomial of Eq. (2), we consider a parity check polynomial of Eq. (3).

Da1+Da2+⋯+Dar+1XD+Db1+Db2+⋯+Dbs+1PD=0                                                                           (3)

Note that ap and bq (p=1,2,•••,r ; q=1,2,•••,s) are natural numbers, and a1a2•••ar and b1b2•••bs are satisfied. The codes which are defined by the check matrix based on the parity check polynomial of Eq. (3), are time-invariant LDPC-CCs.We prepare m pieces of different parity check polynomials based on Eq. (3), where m is a natural number with m2. These parity check polynomials are expressed as:

AiDXD+BiDPD=0                                         (4)

where i=0,1,•••,m−1. Information and parity bits at time j are expressed as Xj, Pj. For Xj and Pj at time j, a parity check polynomial of Eq. (5) is satisfied.

AkDXD+BkDPD=0 (k=j mod m)                                                    (5)

The codes which are defined by a check matrix based on the parity check polynomial of Eq. (5), are time-varying LDPC-CCs with period m.

The proposed LDPC-CC Encoder is time-varying convolutional encoder with period m=3 and memory length 628, which uses Check-equations (6), (7) and (8) depending on time slot.

A1(D)X(D)+B1P(D)=( D628 + D143 + 1)X(D) + ( D44 + D34 + 1)P(D) = 0 --- (6)

A2(D)X(D)+B2P(D)= ( D494 + D106 + 1)X(D) + ( D43 + D20 + 1)P(D) = 0 --- (7)

A3(D)X(D)+B3P(D)= ( D565 + D326 + 1)X(D) + ( D97 + D50 + 1)P(D) = 0 --- (8)

,where  X(D) represents ”Systematic Bits (d-ms,…, d0,…, dM-1)” and  P(D) represents ”Parity Bits (p-ms,…, p0,…, pM-1).

LDPC-CC Encoder can be optionally composed, performing equation (9).

dt=utpt=i=0Mhd(i)(t)ut−i+i=1Mhp(i)(t)pt−i		                                            (9)

Initial sate of LDPC-CC Encoder is all zero state which means:

dt=0pt=0, （t≤0）                                                              (10)

LDPC-CCs support “variable length M Information Bits coding” by same encoder composition, and it also supports plural memory length.

2.3 Structure of LDPC-CC Encoder

An LDPC-CC encoder can be implemented by any encoder realizing the equation (9). An structure of LDPC-CC encoder is shown in Fig. 3 [4]. The LDPC-CC encoder is a time-varying systematic convolutional encoder with memory ms. The LDPC-CC encoder consists of M1 shift registers for ut, M2 shift registers for pt ,a weight controller and an adder and weight multipliers as shown in Fig. 3. Since LDPC-CC is a convolutional code, the complexity of an LDPC-CC encoding is less than the complexity of a CTC encoding which requires an internal interleaver. The complexity of LDPC-CC encoding is proportional to the number of row weights of the matrix H.

Fig. 3:  A structure of LDPC-CC encoder.

2.4 Puncturing

Puncturing is a process to discard several systematic bits and/or parity bits from LDPC-CC Encoder outputs, in order to gain higher ( than 1/2) coding rate by a single coding composition. Puncturing patterns by each coding rates are shown in Table 1. Puncturing pattern d and p represent “Systematic Bit” and “Parity Bit” respectively, and when bit value is “0”, that bit will be “punctured”.  LPunc represents “puncturing pattern length”.

In Puncturing stage, “regular rotated puncturing” is used.  “Systematic Bit” and “Parity Bit” are punctured respectively every LPunc bit according to rules of puncturing pattern described in Table 1. In case of coding rate: 2/3，3/4，4/5, 5/6, systematic Bits are also punctured, thus result code will be non-systematic code. In this proposal, we achieve coding rate R=1/3 by adopting a shortening scheme for LDPC-CCs with coding rate R=1/2 [6].

Encoding Termination:

Termination via Zero-tailing will lead to a stable LDPC-CC Encoder state.

Zero-tailing can be done by LDPC-CC encoder by adding Qz-t bits with value set to “0”. Since Tail-bits are known at receiver, Tail-bits are excluded from Systematic Bits transmission but Parity Bits resulted from Tail-bit coding can be transmitted.

Table 1  Puncturing patterns

| Code Rate | Puncturing Pattern | LPUNC |
| --- | --- | --- |
| 1/3 | d: 010101101010 p: 111111111111 | 24 |
| 1/2 | d: 1 p: 1 | 2 |
| 2/3 | d: 001110111111 p: 111111100011 | 24 |
| 3/4 | d: 010111 p: 111010 | 12 |
| 4/5 | d: 1011 p: 0110 | 8 |
| 5/6 | d: 0001111101 p: 1010111010 | 20 |

# Performance Evaluation

An example of performance in the proposed rate compatible LDPC-CCs is shown. Fig. 4 shows BLER (Block error rate) performance comparing with 16e CTC under AWGN channels, BPSK modulation. Note that Normalized BP decoding [7] used as the message passing decoding and information bit length is 2880 bits. We can confirm that the proposed LDPC-CC performance in coding rate 1/2 and 5/6 is as good as 16e CTC and that LDPC-CC performance is better than 16e CTC that in coding rate 2/3 and 3/4.

Fig. 4:  Performance comparison between LDPC-CC and 16e CTC.

(Parameters in LDPC-CCs evaluation: Iteration number=100, Number of the termination bits Qz-t=380bits)

# Conclusion

We proposed rate compatible LDPC-CC as one of the candidates of the IEEE802.16m FEC scheme, because LDPC-CC are most likely to achieve the peak data rate required for 16m, with low complexity and low latency.

# References

[1] IEEE Std 802.16e-2005.
[2] IEEE C802.16m-08/074r1, “LDPC-Convolutional Codes for IEEE 802.16m FEC Scheme”.
[3] IEEE C802.16m-HARQ-08/051, “Rate Compatible LDPC-Convolutional Codes for IEEE 802.16m FEC”. 
[4]Alberto Jimenez Felström and Kamil Sh. Zigangirov, "Time-Varying Periodic Convolutional Codes with Low - Density Parity - Check Matrix," IEEE Transactions on Information Theory, vol.45, no.6, pp.2181-2191, Sep. 1999. 
[5] IEEE C802.16m-08/806r1, “Feasibility verification of LDPC codes for HARQ operation”.
[6]U. Dammer, E. Naroska, S. Schemermbeck, and Schwiegelshohn, “A data puncturing IR-scheme for typ-II hybrid ARQ protocols using LDPC codes,” Proc. of IEEE Globecom 2004, pp.3012-3016, Nov. 2004.
[7]J. Chen, and M. P. C. Fossorier, “Near optimum universal belief propagation based decoding of low-density parity check codes,” IEEE Transactions on communications, vol.50, no.3, pp.406-414, March 2002.