| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Change Request for the 16m FEC encoding in 802.16m SDD |  |
| Date submitted | 2009-02-27 |  |
| Source(s) | Takaaki KISHIGAMI, Isamu YOSHII Panasonic Corporation Wataru Matsumoto, Toshiyuki Kuze Mitsubishi Electric Corporation Philip V. Orlik, Zhifeng (Jeff) Tao, Jinyun Zhang Mitsubishi Electric Research Laboratories Jin Xu, Bo Sun ZTE Corporation Du Ying, Luo Zhendong CATR Wu Zhanji Beijing University of post and telecommunication (BUPT) | E-mail: kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com Matsumoto.Wataru@aj.MitsubishiElectric.co.jp Kuze.Toshiyuki@ah.MitsubishiElectric.co.jp {porlik, tao, jzhang}@merl.com xu.jin7@zte.com.cn duying@mail.ritt.com.cn luozhendong@mail.ritt.com.cn wuzhanji@bupt.edu.cn |
| Re: | Change Request to 802.16m SDD (802.16m-08/003r7) - Target topic: “11.13 Channel Coding and HARQ ”. |  |
| Abstract | We propose to support rate compatible LDPC codes as an optional 16m FEC encoder. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Change Request for the 16m FEC encoding in 802.16m SDD

Takaaki KISHIGAMI, Isamu YOSHII

Panasonic Corporation

Wataru Matsumoto, Toshiyuki Kuze

Mitsubishi Electric Corporation

Philip V. Orlik, Zhifeng (Jeff) Tao, Jinyun Zhang

Mitsubishi Electric Research Laboratories

Jin Xu, Bo Sun

ZTE Corporation

Du Ying, Luo Zhendong

CATR

Wu Zhanji

Beijing University of post and telecommunication (BUPT)

# Introduction

This contribution is to propose text change on 802.16m SDD(08/003r7) in order to support LDPC codes as an optional 16m FEC, because LDPC codes enable not only to achieve as good performance as Turbo codes, but also to achieve high-speed decoding, thanks to parallel processing with message-passing decoding. In addition, Rate Compatible LDPC codes has already been proposed by several companies as follows. The proposed RC-LDPC codes can provide 1/3 code rate and Hybrid ARQ with incremental redundancy and would be able to provide improved robustness to the BS-MS links.

Proposed Rate Compatible LDPC codes:

Alternative 1: The proposed RC-LDPC codes uses the current 802.16 LDPC as the baseline matrix.

The detail is discussed in [1] or its latest version

Alternative 2: The proposed RC-LDPC codes uses LDPC-Convolutional Codes as the baseline matrix.

The detail is discussed in [2] or its latest version

# Proposed Change

-------------------------------  Text Start  ---------------------------------------------------

11.13 Channel coding and HARQ

11.13.1 Channel coding

11.13.1.3 FEC encoding

IEEE 802.16m uses the CTC (convolutional turbo code) of code rate 1/3 defined in the IEEE 802.16e standard

where the CTC inner interleaver parameters for additional FEC block sizes are FFS while maintaining IEEE

802.16e CTC interleaver. The code rate of the “FEC Encoder” block in Figure 50 is termed mother code rate

(RMC). The use of other coding schemes like CC and LDPC are FFS. IEEE 802.16m also uses LDPC codes as optional coding schemes. The details on LDPC codes are FFS.

The CTC scheme is extended to support additional FEC block sizes. FEC block sizes larger than the legacy

ones are supported. The FEC block sizes are FFS and they are independent of the transmission format,

including

code rate, modulation order, and resource allocation. Further, the FEC block sizes are regularly increased with

pre-determined block size resolutions. The FEC block sizes which are multiple of 7 is removed for the

tail-biting encoding structure.

The encoder block depicted in Figure 50 includes the sub-block interleavers. The interleaving details

are FFS.

-------------------------------  Text End  ---------------------------------------------------

# References

[1]IEEE C80216m-09/0102r4, “Proposed Text of Enhanced Structured LDPC coding with rate matching for the IEEE 802.16m Amendment”.

[2]IEEE C802.16m-09/0412, “Rate Compatible LDPC-Convolutional Codes for the Change Request to 802.16m SDD”.