| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on SDD text for IEEE 802.16m generic MAC header |  |
| Date Submitted | 2009-02-28 |  |
| Source(s) | Eunjong Lee, Kiseon Ryu, Jeongki Kim and Doo-hyun Sung LG Electronics | Voice: +82-31-450-1910 E-mail: mehappy3@lge.com ksryu@lge.com chegal@lge.com dh.sung@lge.com |
| Re: | “IEEE 802.16m Internal Documents Configuration Control Procedure (CCP)”, IEEE 802.16m-09/0008 |  |
| Abstract | Comments on SDD text for PDU formats |  |
| Purpose | Discussion, review, and inclusion into IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on SDD text for IEEE 802.16m generic MAC header

Eunjong Lee, Kiseon Ryu, Jeongki Kim and Doo-hyun Sung

LG Electronics

# Introduction

At the last meeting, the 2 bytes GMH containing EH, Flow ID and Length field was accepted in SDD. In here, Flow ID field can be used to distinguish different header types or connections. However, since Flow ID in current SDD always presents following EH field, all MAC headers should contain EH and Flow ID field although some MAC headers (e.g., multiplexing MAC header, signaling MAC header, etc.) may do not need to contain the EH field. For such MAC header, EH field is an unnecessary overhead. Therefore, we propose that the Flow ID field is always located in the first 4 bits of MAC header.

# Text Proposal

============================ Start of Proposed Text ==================================

Modify the chapter 10.12.1.1 to the following text

10.12.1 MAC header formats

4 bits Flow IDs are used to distinguish different connections or header types.

10.12.1.1 Generic MAC Header

Figure 22 Generic MAC Header Format

- EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header is present following this GMH.

- Flow ID (Flow Identifier): This field indicates the flow that is addressed. This field is 4bits long.

- EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header is present following this GMH.

- Length: Length of the payload. This field is 11bits long.

============================== End of Proposed Text ================================