| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on SDD text for IEEE 802.16m Generic MAC header |  |
| Date Submitted | 2009-03-05 |  |
| Source(s) | Eunjong Lee, Kiseon Ryu, Jeongki Kim, Doo-hyun Sung, Ronny Yongho Kim, Jin Sam Kwak LG Electronics | Voice: +82-31-450-1910 E-mail: mehappy3@lge.com ksryu@lge.com |
| Re: | “IEEE 802.16m Internal Documents Configuration Control Procedure (CCP)”, IEEE 802.16m-09/0008 |  |
| Abstract | Comments on SDD text for PDU formats |  |
| Purpose | Discussion, review, and inclusion into IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on SDD text for IEEE 802.16m Generic MAC header

Eunjong Lee, Kiseon Ryu, Jeongki Kim, Doo-hyun Sung, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics

# Introduction

At the last meeting, the 2 bytes GMH containing EH, Flow ID and Length field was accepted in SDD. In here, Flow ID field can be used to distinguish different header types or connections. However, since Flow ID in current SDD always presents following EH field, all MAC headers should contain EH and Flow ID field although some MAC headers (e.g., multiplexing MAC header, signaling MAC header, etc.) may do not need to contain the EH field. For such MAC header, EH field is an unnecessary overhead. Therefore, we propose that the Flow ID field is always located in the first 4 bits of MAC header, as illustrated in Figure 1. And the 4-bit Flow ID is used to distinguish different connections or header types (e.g., for transport data, management, signaling and multiplexing).

Figure 1 Proposed Generic MAC Header Formats

In addition to GMH, current SDD has described that multiple MAC SDUs and/or SDU fragments from different unicast connections belonging to the same AMS can be multiplexed into a single MAC PDU. However, the multiplexing method is not described in SDD. Therefore, we propose to define the 1-byte multiplexing MAC Header (MMH) to support the multiplexed flows, as shown in Figure 2. If the Flow ID indicates that this header is MMH (e,g., Flow ID = 0b0010), the NOF (Number Of Flows) field indicating the number of multiplexed flows should present following the Flow ID field. And then, as shown in Figure 3, as many GMHs as NOF indicates exist following this MMH. In order to support all operations for a MAC PDU with a single flow, the MAC PDU for multiple flows should also have GMH per flow.

Figure 2 Proposed Multiplexing MAC Header Format

Figure 3 An example of MMH usage

# Text Proposal

============================ Start of Proposed Text ==================================

[Modify the chapter 10.12. to the following text.]

10.12 MAC PDU Formats

Each MAC PDU contains a MAC header. The MAC PDU may contain payload. The MAC PDU may contain one or more extended headers.

Multiple MAC SDUs and/or SDU fragments from different unicast connections belonging to the same AMS can be multiplexed into a single MAC PDU.

10.12.1 MAC header formats

Flow IDs are used to distinguish different connections or header types.

10.12.1.1 Generic MAC Header

Figure 22 Generic MAC Header Format

- EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header is present following this GMH.

- Flow ID (Flow Identifier): This field indicates the flow that is addressed. This field is 4bits long.

- EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header is present following this GMH.

- Length: Length of the payload. This field is 11bits long.

10.12.1.2 Multiplexing MAC Header

Multiplexing MAC Header is used to transmit the multiple MAC SDUs and/or SDU fragments from different unicast connections belonging to the same AMS into a single MAC PDU.

Figure xx Multiplexing MAC Header Format

- Flow ID: This field indicates that this header is Multiplexing MAC Header. This field is 4bits long.

- NOF (Num of Flows): This field indicates the number of multiplexed flows. This field is 4bits long. As shown in below Figure yy, as many GMHs as NOF field indicates will present following 1-byte MMH.

============================== End of Proposed Text ================================