| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on MAC header formats in IEEE 802.16m SDD |  |
| Date Submitted | 2009-02-27 |  |
| Source(s) | Jeongki Kim, Kiseon Ryu, Youngsoo Yuk, and Ronny Yongho Kim LG Electronics | chegal@lge.com ksryu@lge.com |
| Re: | “IEEE 802.16m Internal Documents Configuration Control Procedure (CCP)”, IEEE 802.16m-09/0008 |  |
| Abstract | Comments on MAC header formats in IEEE 802.16m SDD |  |
| Purpose | Discussion, review, and inclusion into IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on MAC header formats in IEEE 802.16m SDD

Jeongki Kim, Kiseon Ryu, Youngsoo Yuk, and Ronny Yongho Kim

LG Electronics

# Introduction

The VoIP capacity is currently defined as “> 60 users/sector/MHz” in SRD[1]. To meet this system requirement, we need to minimize the unnecessary MAC overhead.

So, we propose compact header for persistent allocation (PA) and group resource allocation (GRA) with smaller size than that of GMH. Since the resource region allocated by PA/GRA is the special resource region pre-negotiated between BS and MS, the compact header doesn’t need to be distinguished from other MAC headers (e.g. GMH, signaling MAC header, etc.).

Figure 1 shows an example of compact header.

Figure 1. Compact header format

- EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header is present following this header.

- Length (7): Length of the payload.

# References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

# Text Proposal

================= Start of Proposed Text ========================

10.12.1 MAC header formats

10.12.1.1 Generic MAC Header

10.12.1.2 Extended header

10.12.1.3 Compact header

Compact header is used for connections with persistent allocation and group allocation.

Figure xx. Compact header format

- EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header is present following this header.

- Length (7): Length of the payload.

============================== End of Proposed Text ===============