| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on SDD text for IEEE 802.16m Extended Header |  |
| Date Submitted | 2009-02-28 |  |
| Source(s) | Eunjong Lee, Kiseon Ryu, Jeongki Kim and Doo-hyun Sung LG Electronics | Voice: +82-31-450-1910 E-mail: mehappy3@lge.com ksryu@lge.com chegal@lge.com dh.sung@lge.com |
| Re: | “IEEE 802.16m Internal Documents Configuration Control Procedure (CCP)”, IEEE 802.16m-09/0008 |  |
| Abstract | Comments on SDD text for PDU formats |  |
| Purpose | Discussion, review, and inclusion into IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on SDD text for IEEE 802.16m Extended Header

Eunjong Lee, Kiseon Ryu, Jeongki Kim and Doo-hyun Sung

LG Electronics

# Introduction

In IEEE 802.16, when the fragmentation/packing for a MAC SDU occurs and a MAC PDU is transmitted with ARQ-enabled connection, the transmitter should attach the Fragmentation & Packing Extended Header (FPEH) following the Generic MAC Header (GMH). This means that most of the MAC PDUs are transmitted with FPEH. That is, the overheads of FPEH can directly affect the overhead of total header. Therefore, we propose to define the Fragmentation and Packing Indicator (FPI) field in Extended Header (EH) to reduce the overhead of FPEH.

The proposed FPI field indicates whether the current EH is FPEH or not. If this field is set, the FPEH can be indicated by the FPI field without specific type field and the remaining fields for the FPEH can be defined following this FPI, as shown in Figure 1. If this FPI field is not set, the EH should be used with the type field, as shown in Figure 2. By using the FPI field, the total size of the FPEH can be reduced shorter than the one using the EH type field. In the next section, we compare the MAC header overhead for some formats and the result shows that the header with FPI field has the lowest overhead.

Figure 1 An example of Fragmentation and Packing Extended Header format

Figure 2 General Extended Header format

# Overhead Analysis

This section compares the MAC header overhead for some formats based on the assumptions/parameters as shown in the Table 1.

Table 1 Assumptions and parameters

| Parameter | Value | Note |
| --- | --- | --- |
| % of CMH Usage | 30% | % of MAC Header usage for persistent allocation |
| % of GMH Usage | 100 - % of CMH Usage |  |
| % of MPDU with Fragmentation Header | 70% |  |
| % of MPDU with Packing Header | 20% |  |
| Avg # of packed SDUs in one MPDU | 3 |  |

Following table describes the bytes required for each header and the overhead per MPDU for a single connection.

|  | GMH | CMH | Fragmentation overhead(FSH) | Packing overhead (PSH) | *Total overhead per MPDU (Bytes) |
| --- | --- | --- | --- | --- | --- |
| 16e | 6 | 6 | 2 | 9 | 8.2 |
| SDD + Samsung’s FPEH | 2 | 2 | 3 | 5.625 | 4.3 |
| LGE | 2 | 1 | 2 | 4.875 | 3.4 |

In the above table, the bytes of packing overhead (PSH) and total overhead per MPDU can be derived as follows.

* Total overhead per MPDU = GMH* % of GMH Usage + CMH * %of CMH Usage +

% of GMH Usage * (FSH * % of MDU with FSH + PSH * % of MPDU with PSH)

Packing overhead for 16e

= 3* Avg # of packed SDUs in one MPDU = 9

Packing overhead for SDD +SS’s FPEH

= 1 byte EH + 2 bits FC + 10bits SN + 1 bit END + (11bits LEN + 1 bit END)*(n-1)
= 8/8 + 13/8 + 12/8 * (Avg # of packed SDUs in one MPDU -1)= 5.625

Packing overhead for LGE

= 1 bit LAST+1 bit FPI+2 bits FC+10 bits SN+1bit END+(11bits LEN+1 bit END)*(n-1)
= 2/8 + 13/8 + 12/8 * (Avg # of packed SDUs in one MPDU -1)= 4.875

# Text Proposal

============================ Start of Proposed Text ==================================

Modify the chapter 10.12.2 to the following text

10.12.2 Extended header

The inclusion of extended header is indicated by EH indicator bit in MAC Header. The EH format is shown in Figure 23 and will be used unless specified otherwise.

Figure 23 Extended Header Format

- Last (1): When the “Last” bit is set, another extended header will follow the current extended header. If this bit is not set this extended header is the last one.

- FPI (1): This field indicates whether the current extended header is fragmentation/packing extended header or not. If this field is set to ‘1’, the extended header as shown in chapter 10.12.2.1 is used for fragmentation and packing.

- Type (TBD): indicates the type of extended header. The length is TBD.

- Body Contents (variable length): Type-dependent contents.

10.12.2.1 Fragmentation and packing extended header

Fragmentation and packing extended header format is FFS. The Fragmentation and packing extended header is used to support the fragmentation of MAC SDU into one or more MAC PDUs and allow the packing of one or more SDUs/SDU fragments in a single MAC PDU. The fragmentation and packing extended header is shown in figure xx.

Figure xx Fragmentation and Packing Extended Header Format

- Last (1): When the “Last” bit is set, another extended header will follow the current extended header. If this bit is not set, this extended header is the last one.

- FPI (1): This field indicates whether the current extended header is fragmentation/packing extended header or not. This field shall be set to ‘1’.

- FPEH contents (variable length): Contents to support fragmentation and packing.

============================== End of Proposed Text ================================