| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CR on SDD: Modifications on PHY Level Cell Identifier of Femtocell |  |
| Date Submitted | 2009-03-12 |  |
| Source(s) | Zheng Yan-Xiu, Yung-Han Chen ITRI | zhengyanxiu@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-09/0008, “IEEE 802.16m Internal Documents Configurations Control Procedure (CCP)” |  |
| Abstract | This contribution proposes two modifications on physical layer identifier for femtocell ABS. The first modification is applying PA-PREAMBLE to indicate the existence of CSG and OSG femtocell ABS. The second modification recommends applying A-PREAMBLE to distinguish femtocell ABS and macro ABS. |  |
| Purpose | For discussion and adoption in 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

CR on SDD: Modifications on PHY Level Cell Identifier of Femtocell

Zheng Yan-Xiu, Yung-Han Chen

ITRI

Introduction

The existence of CSG and OSG femtocell BSs shall be indicated by PA-Preamble. Physical layer identifier reduces frequent handover to femtocell BS. However CSG femtocell BS would block non-CSG femtocell AMSs. If there is no prior Cell ID information on CSG femtocell BS, AMS performs exhausted Cell ID search and wastes computation power. If PA-Preamble indicates the existence of CSG and OSG femtocell BSs, AMS can ignore CSG femtocell BSs to avoid redundant Cell ID search procedure.

Femtocell BS and macro BS are recommended to be distinguished by A-Preamble. Cell ID can only be obtained by SA-Preamble. If we distinguish femtocell BSs and macro BSs by Cell ID, it implies femtocell BS and macro BS are distinguished by SA-Preamble and does not allow a composite solution to identify femtocell BS and macro BS. For example: we can allocate SA-Preamble for femtocell BS and the SA-Preamble uses different frequency interleaved SA-Preamble. Using A-Preamble to identify femtocell BS and macro BS does not preclude existing Cell ID but also allow possible simpler method to identify femtocell BS.

Proposed SDD Text Modification

[Modify section 17.2.1 as follows]

--------------------------------------------------------  Text Start  ------------------------------------------------------------

### 17.2.1 PHY level cell identifier

Femtocell ABSs and macro ABSs are differentiated using Cell IDs, which shall be obtained from the A-PREAMBLE. PA-PREAMBE indicates the existence of CSG femto BS. It enables AMSs to quickly identify cells types, avoid too frequent handover attempts into and out of a Ffemtocell, and avoid performing unnecessary network entry/re-entry, and eliminate redundant Cell ID search.

--------------------------------------------------------  Text End  ------------------------------------------------------------