| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Network Entry procedure for 16m Relay |  |
| Date Submitted | 2009-03-09 |  |
| Source(s) | Youngbin Chang, Hyunjeong Kang, Mingxia Xu, Rakesh Taori, Jungje Son Samsung Electronics Yousuf Saifullah, Shashikant Maheshwari, Adrian Boariu, Haihong Zheng Nokia Siemens Networks | yb.chang@samsung.com, yousuf.saifullah@nsn.com |
| Re: | Change request to the 802.16m SDD (section 15) |  |
| Abstract | This contribution proposes a high level description on network entry procedure for 16m Relay |  |
| Purpose | For adopting proposed text into 16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Network Entry procedure for 16m Relay

Youngbin Chang, Hyunjeong Kang, Mingxia Xu, Rakesh Taori, Jungje Son

Samsung Electronics

Yousuf Saifullah, Shashikant Maheshwari, Adrian Boariu, Haihong Zheng

Nokia Siemens Networks

# Introduction

When relaying is applied to IEEE 802.16m system, Network entry procedure is a basic operation in 16m Relay. Network entry procedure of 16m Relay can follow the basic mechanisms defined in IEEE 802.16j [1].

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Insert the texts in subclause 15.4 as follows:]

15.4.x. ARS Network entry and Initialization

ARS shall follow network entry and initialization procedure of AMS. Additionally, ARS may perform interference measurement of neighbor stations, path creation, and tunnel connection establishment with ABS. ARS operation parameters are obtained from access station by configuration signaling.

15.4.y AMS Network Entry support in ARS

AMS Network entry procedure could be distributed between ARS and ABS. ARS should handle the initial link

adjustment with AMS. The remaining AMS network entry procedures such as capability negotiation, connection establishment, authentication, registration are processed between AMS and ABS.

---------------------------------------------------------End of the Text----------------------------------------------------------

# Reference

[1] IEEE P802.16j/D9, “Draft Amendment to IEEE Standard for Local and metropolitan area networks: Air

Interface for Fixed and Mobile Broadband Wireless Access Systems, Multihop Relay Specification,” Feb.

2009.