| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Change Request for Section 11.8.4.1: Multi-BS MIMO in 802.16m SDD |  |
| Date Submitted | 2009-02-27 |  |
| Source(s) | Philip V. Orlik, Ramesh Annavajjala, Amine Maaref, Zhifeng (Jeff) Tao, Simon Pun, Jinyun Zhang Mitsubishi Electric Research Lab Toshiyuki Kuze Mitsubishi Electric Corp | Voice: 617-621-{7558, 7518, 7570, 7595} Fax: 617-621-7550 {porlik, annavajjala, maaref, tao,mpun ,jzhang}@merl.com Voice: +81-467-41-2885 Fax: +81-467-41-2486 Kuze.Toshiyuki@ah.MitsubishiElectric.co.jp |
| Re: | Change Request to 802.16m SDD (802.16m-08/003r7) - Target topic: “11.8.4.1 Multi-BS MIMO ”. |  |
| Abstract | This contribution proposes pilot patterns for Section 11.8.4.1 Multi-BS MIMO in the current 802.16m SDD (802.16m-08/003r7) |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Change Request for

Section 11.8.4.1: Multi-BS MIMO in 802.16m SDD

Phil Orlik, Ramesh Annavajjala, Amine Maaref, Zhifeng (Jeff) Tao, Simon Pun, Jinyun Zhang

Mitsubishi Electric Research Labs

Toshiyuki Kuze

Mitsubishi Electric Corp

# Introduction

In Section 11.8.4.1 of the IEEE 802.16m Systems Description Document (SDD) [1], Multi Base-Station MIMO approaches are considered to improve the sector throughput as well as cell-edge throughput. However, for both open-loop as well as closed-loop multi-BS MIMO approaches mentioned in [1], very little information is present on the resource blocks, pilot designs, pre-coder selection etc. In this contribution we present pilot patterns for two collaborating base-stations each with two or four transmit antennas.

# Proposed Scheme

Similar to the single-BS MU-MIMO, we assume that our resource block size is 18 consecutive subcarriers over sub-frame duration of 6 consecutive OFDM symbols. In [1], pilot patterns for up to 4 transmit antennas are presented only in the context of single-user MIMO with up to 4 streams. In our contribution, we consider two collaborating base-stations under Multi-BS MIMO each with either 2 or 4 transmit antennas. We assume that the number of streams per BS under cooperation is equal to the number of transmit antennas. This configuration is highly useful to enhance the multi-user sector throughput and cell-edge user performance.

In Fig. 1, a pilot pattern is presented for two collaborating base-stations under multi-BS MIMO configuration where each BS has 2 transmit antennas for transmission of up to 2 spatial streams. From Fig. 1, the pilot overhead per stream per BS is 3.7%. Here, Pj,k denotes the pilot for stream j from BS k. In a similar manner, our pilot pattern for four transmit antenna Multi-BS MIMO is shown in Fig. 2 with the same per-stream overhead per BS as 3.7%.

Figure 1: Pilot pattern for Multi-BS MIMO. Two collaborating base-stations each having 2 transmit antennas

Figure 2: Pilot pattern for Multi-BS MIMO. Two collaborating base-stations each having 4 transmit antennas

# Proposed Text

Modify the existing text in [1] as follows: Existing text is shown in black, whereas the proposed text is shown in red.

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

11.8.4.1 Multi-BS MIMO

Multi-BS MIMO techniques are supported for improving sector throughput and cell-edge throughput throughmulti-BS collaborative precoding, network coordinated beamforming, or inter-cell interference nulling. Both open-loop and closed-loop multi-BS MIMO techniques can be considered. For closed-loop multi-BS MIMO, CSI feedback via codebook based feedback or sounding channel will be used. The feedback information may be shared by neighboring base stations via network interface. Mode adaptation between single-BS MIMO and multi-BS MIMO is utilized.

11.8.4.1.1 Pilot patterns for Multi-BS MIMO

11.8.4.1.1.1 Pilot patterns for two collaborating ABSs each with 2 antennas

With two antennas at each advanced base-station (ABS) for transmission of two spatial streams, the following pilot pattern shall be used.

Figure 3: Pilot pattern for Multi-BS MIMO. Two collaborating base-stations each having 2 transmit antennas

In Figure 1, Pj,k is the pilot subcarrier for the j-th stream on k-th ABS, with j=1,2 and k=1,2. The pilot subcarrier groups {P1,1, P2,1} and {P1,2,P2,2} are distinguished based on the  Cell-ID.

11.8.4.1.1.2 Pilot patterns for two collaborating ABSs each with 4 antennas

With four antennas at each advanced base-station (ABS) for transmission of four spatial streams, the following pilot pattern shall be used.

Figure 4: Pilot pattern for Multi-BS MIMO. Two collaborating base-stations each having 4 transmit antennas

In Figure 2, Pj,k is the pilot subcarrier for the j-th stream on k-th ABS, with j=1,2,3,4 and k=1,2. The pilot subcarrier groups {P1,1,P2,1,P3,1,P4,1,} and {P1,2,P2,2,P3,2,P4,2} are distinguished based on the Cell-ID.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# Reference

[1] Shkumbin Hamiti, “The Draft IEEE 802.16m System Description Document”, IEEE 802.16m-08/003r7