| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed header format to support Multiplexing scheme in 16m |  |
| Date Submitted | 2009-03-10 |  |
| Source(s) | Haihong Zheng, Shashikant Maheshwari, Adrian Boariu, Yousuf Saifullah Nokia Siemens Networks Andrea Bacioccola Nokia Anil Agiwal,Youngbin Chang, Sungjin Lee, Rakesh Taori, Jungje Son Samsung Yihshen Chen MediaTek Hua Xu Motorola Ken Lao III | E-mail: haihong.zheng@nsn.com andrea.bacioccola@nokia.com |
| Re: | TGm SDD: 10.12.2 Extended Headers |  |
| Abstract | SDD text proposal on 16m Headers |  |
| Purpose | For discussion and adoption in 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Extended Header Format to support Multiplexing

# Introduction

SDUs or SDUs fragments belonging to different connections but share the same security association can be multiplexed into one MPDU. With multiplexing, single security information (e.g, PN and ICV) can be used to protect the entire MPDU instead of having the security information for each MPDU. The overhead can be significantly reduced. This contribution proposes to use Multiplexing Extended Header to realize the multiplexing feature.

# Text Proposal

================= Start of Proposed Text ========================

10.12.2.2 Multiplexing Extended Header (MEH)

Multiplexing Extended Header (MEH) is used when SDUs or SDUs fragments from different connections are included in the same MPDU.

As shown in the figure x, Multiplexing Extended Header contains multiple Multiplexing Extended Header Blocks (MEHBs). The SDUs or SDU fragments belonging to the same connection are packed together and the information related to these SDUs or SDU fragments is included in one MEHB. The M bit in MEHB indicates if there is more MEHB followed. If the SDUs or SDU fragment(s) included belong to the same connection, only one MEHB is present.

Figure x: Format of Multiplexing Extended Header (MEH)

The format of MEHB is shown in Figure y, except the first MEHB. The first MEHB doesn’t contain the Flow ID and the length for the first SDU or SDU fragment associated with the Flow ID. The Flow ID and the Length fields in the generic MAC header represent the flow ID and the length of the first SDU or SDU fragment associated with the first MEHB.

Figure y: Format of Multiplexing Extended Header Block (MEHB)

M (1bit): indicate if there is more MEHB follows

Flow ID (4 bit): flow id of the SDUs/SDU fragments identified in the MEHB

SN (10): ARQ BSN for ARQ enabled connection or Fragment SN for non-ARQ enabled connection.

L (1bit): indicate if there is more length field follows

Length: length for each SDU identified in the MEHB

FC (2bit): Multiplexing Control Information (as shown in Table z).

============================== End of Proposed Text ===============