| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of E-MBS region indication |  |
| Date Submitted | 2009-02-27 |  |
| Source(s) | Ke Yao, Kaiying Lv, Li Wang ZTE Corporation | Voice: E-mail: lv.kaiying@zte.com.cn |
| Re: |  |  |
| Abstract | This contribution proposes modifications to the text in Section 14.4.1.3 (“Frame and control channel structure”) of TGm SDD (IEEE 802.16m-08/003r7) to clarify the E-MBS region indication. |  |
| Purpose | For review and discussion by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of E-MBS region indication

Ke Yao, Kaiying Lv, Li Wang

ZTE

# 1. Introduction

In the current SDD [1] text, it has been agreed that S-SFH indicates E-MBS region which may span over multiple subframes for each E-MBS zone in unicast/multicast mixed carrier, but the details are indeterminate.

Essential system parameters and system configuration information belonging to the S-SFH are categorized into multiple S-SFH IEs. The S-SFH IEs are transmitted in different timing and periodicity. E-MBS region information could be carried on one of S-SFH IEs and be transmitted periodically. Assuming the periodicity of the S-SFH IE carrying the E-MBS region information is M superframes, the E-MBS region may span over M superframes which is the interval of two adjacent S-SFHs.

If a superframe contains MBS subframes, MBS subframes are allocated with fixed pattern within superframe. In order to facilitate the signaling of E-MBS region information and, a hierarchical scheme is suggested to reduce the signaling overhead. The hierarchical indication may consist of the indication on superframe, frame and subframe level respectively.

# 2. References

[1] IEEE 802.16m-08/003r7, “IEEE 802.16m System Description Document”

# Text proposal for the 802.16m SDD

---------------------------------------------------- Start of the Text Proposal ------------------------------------------------

[Modify the following text in Section 14.4.1.3]

14.4.1.3 Frame and Control Channel Structure

In unicast/multicast mixed carrier, E-MBS uses the same frame structure used for unicast carrier. The E-MBS data is multiplexed with Unicast traffic. The S-SFH indicates E-MBS region which may span over M superframes which is the periodicity of corresponding S-SFH IEs for each E-MBS zone. The value of M is FFS. The hierarchical indication scheme may be used. It may consist of the indication on superframe, frame and subframe level respectively. If a superframe contains MBS subframes, MBS subframes are allocated with fixed pattern within superframe. The pattern may vary between superframes. The figure below illustrates the frame structure when MBS subframes are present in superframes.

-----------------------------------------------------End of the Text Proposal ---------------------------------------------------