| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Discussion on femtocell low-duty operation mode |  |
| Date Submitted | 2009-03-08 |  |
| Source(s) | Guang Han Stavros Tzavidas Hua Xu Motorola | guang.han@motorla.com stavros.tzavidas@motorola.com Hua.Xu@motorola.com |
| Re: | Change request to Project 802.16m System Description Document (SDD) (IEEE 802.16m-08/003r7) |  |
| Abstract | This contribution discusses Femtocell low-duty operation mode |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Discussion on femtocell low-duty operation mode

# Guang Han, Stavros Tzavidas, Hua Xu

# Motorola

# Discussion

The current text (the third paragraph of section 17.7) proposes that the handover signaling or location update signaling is used to trigger the network to signal the CSG Femtocell BS to enter low-duty operation mode or normal operation mode. However, in order to know whether a CSG Femtocell BS needs to be signaled and which CSG Femtocell BS needs to be signaled upon receiving a handover or location update signaling, the network has to maintain the mapping information of the Femtocell BSs and their overlay macro BSs as well as the mapping information of the MSs and the Femtocell BSs to which they are registered. Once any change has occurred, the network has to update this database. Moreover, without special indication, every handover or location update message has to be inspected by either the network or the macro BS in order to determine whether some CSG Femtocell BS needs to be activated. Maintaining femto-related mapping information and inspecting every handover or location update message will introduce significant effort to the macro network.

We thus propose that the MS shall send a message including the Femtocell identification information and an ON/OFF indication bit to the network (or the overlay macro BS) when it enters/leaves the overlay macro cell. The Femtocell identification information will help the network to route the message to the CSG Femtocell BS and the indication bit is used to notify the network whether the CSG Femtocell BS should enter or leave the low-duty operation mode.

# Proposed SON/Femto Harmonized Text Modification

[In IEEE 802.16m-08/003r7, Section 17.7, modify the following text]

-------------------------------  Text Start  ------------------------------------------------------------------------------------

17.6	Low-duty Operation Mode

Besides the normal operation mode, CSG Femtocell BSs may support Low-duty Operation Mode, in order to reduce interference to neighbor cells. The low-duty operation mode consists of available intervals and unavailable intervals. During an available interval, the Femtocell BS may become active on the air interface for synchronization and signaling purposes such as paging, ranging or for data traffic transmission opportunities for the MSs. During an unavailable interval, it does not transmit on the air interface. Unavailable interval may be

used for synchronization with the overlay macro BS or measuring the interference from neighbor cells.

The Femtocell BS may enter low-duty operation mode either if all MSs attached to the Femtocell BS are in idle

or sleep mode, or if no MS is in the service range of the Femtocell BS at all.

When an MS leaves or enters the overlay macro BS of its CSG Femtocell BS, the network may signal the CSG Femtocell BS to enter low-duty operation mode or normal operation mode, respectively. Handover signaling or location update signaling may trigger such operation. The message to trigger such operation shall include an ON/OFF indication bit and the identifier of the CSG Femtocell BS to help facilitate network signaling the CSG Femtocell BS.

The CSG Femtocell BS switches between the low-duty operation mode and the normal operation mode when it receives requests from the overlay macro BS, the core network, or an MS for network entry, HO, or the exit of the sleep mode.

The low-duty operation mode shall not impact the support for emergency service and E911.

-------------------------------  Text End  ------------------------------------------------------------------------------------