| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Issues with femtocell low-duty operation mode |  |
| Date Submitted | 2009-03-08 |  |
| Source(s) | Guang Han Stavros Tzavidas Hua Xu Motorola | guang.han@motorla.com stavros.tzavidas@motorola.com Hua.Xu@motorola.com |
| Re: | Change request to Project 802.16m System Description Document (SDD) (IEEE 802.16m-08/003r7) |  |
| Abstract | This contribution discusses Femtocell low-duty operation mode |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Issues with femtocell low-duty operation mode

# Guang Han, Stavros Tzavidas, Hua Xu

# Motorola

# Introduction

The current proposal (the fourth paragraph of section 17.7) suggests that a CSG Femtocell BS should leave the low-duty operation mode when it gets to know that an MS subscriber enters the overlay macro cell. However, this MS subscriber may still be quite distant from the CSG Femtocell BS and it may keep wandering for a long time before finally arriving at the Femtocell (or not at all!). To prevent a CSG Femtocell BS from generating unnecessary interference, we should keep it in the low-duty operation mode until it is notified that the MS subscriber has entered its coverage.

To help a MS to detect its CSG Femtocell BS in the low-duty operation mode after it enters the overlay macro cell, the network shall notify the MS at what time the CSG Femtocell BS will be active on the air interface. The network (or the overlay macro BS) can get this timing information from the CSG Femtocell BS. Given such timing information, the MS will scan for its CSG Femtocell BS only when it is expected to be active. CSG Femtocell BSs overlaid by the same macro cell shall become active at non-overlapping time slots so that they can effectively hide themselves from non-allowed passing-by MSs.

# Proposed SON/Femto Harmonized Text Modification

[In C80216m-FemtoSON-08_040, Section 17.2, modify the following text]

-------------------------------  Text Start  ------------------------------------------------------------------------------------

17.6	Low-duty Operation Mode

Besides the normal operation mode, CSG Femtocell BSs may support Low-duty Operation Mode, in order to reduce interference to neighbor cells. The low-duty operation mode consists of available intervals and unavailable intervals. During an available interval, the Femtocell BS may become active on the air interface for synchronization and signaling purposes such as paging, ranging or for data traffic transmission opportunities for the MSs. During an unavailable interval, it does not transmit on the air interface. Unavailable interval may be

used for synchronization with the overlay macro BS or measuring the interference from neighbor cells.

The Femtocell BS may enter low-duty operation mode either if all MSs attached to the Femtocell BS are in idle

or sleep mode, or if no MS is in the service range of the Femtocell BS at all.

When an MS leaves or enters the overlay macro BS of its CSG Femtocell BS, the network may signal the CSG Femtocell BS to enter low-duty operation mode or normal operation mode, respectively. Handover signaling or location update signaling may trigger such operation.

The CSG Femtocell BS shall remain in low-duty operation mode when it receives requests from the overlay macro BS or the core network. The network shall notify the MS at what time the CSG Femtocell BS will be active on the air interface. Given such timing information, the MS will scan for its CSG Femtocell BS only when it is expected to be active. CSG Femtocell BSs overlaid by the same macro cell shall become active at non-overlapping time slots so that they can hide themselves from non-allowed passing-by MSs. The CSG Femtocell BS switches to between the low-duty operation mode and the normal operation mode when it receives requests from the overlay macro BS, the core network, or an MS for network entry, HO, or the exit of the sleep mode.

The low-duty operation mode shall not impact the support for emergency service and E911.

-------------------------------  Text End  ------------------------------------------------------------------------------------