| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Change Request to IEEE 802.16m SDD - Selective Protection Changes |  |
| Date Submitted | 2009-02-27 |  |
| Source(s) | Shashikant Maheshwari, Haihong Zheng, Yousuf Saifullah Nokia Siemens Networks Jan Suumaki Nokia | E-mail: shashi.maheshwari@nsn.com E-mail: jan.suumaki@nokia.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | CR for IEEE 802.16m System Description Document (SDD) - Section 10.6.5.2.1 Management Message Protection |  |
| Abstract | This contribution proposes changes to the Section 10.6.5.2.1 Management Message Protection of the 802.16m SDD. |  |
| Purpose | For review and adoption into 802.16m System Description Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Change Request to IEEE 802.16m SDD - Selective Protection Changes Shashikant Maheshwari, Haihong Zheng, Yousuf Saifullah

Nokia Siemens Networks

Jan Suumäki

Nokia

# Introduction

This contribution proposes changes for the section “Section 10.6.5.2.1 Management Message Protection” in IEEE 802.16m System Description Document [1].

Currently information required for selective protection is not defined for management connections.

This contribution proposes "strongest protection" method which does not require any explicit identification in the MAC header. Thus no bits from GMH or dedicated flow IDs are not needed to reserve for security method identification.

Instead using explicit identification, security state and validity of AK / TEK is used to select the protection method;

- No protection

- CMAC protection

- AES-CCM protection

Validity of AK means that EAP authentication is successfully executed and AK lifetime is not elapsed

Validity of TEK means that Nonce is exchanged and TEKs are derived locally in ABS and AMS. Also TEK lifetime is not elapsed (Lifetime of TEK generation which can be used for encryption)

For example in initial network entry the following protection methods are used.

- Initial ranging and pre-authentication capability negotiation - no protection

- EAP authentication - no protection

- Key Agreement - CMAC based protection

- Basic capability negotiation and Registration - AES-CCM based protection

- Further management procedures - AES-CCM based protection

# Proposed Changes to SDD

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

10.6.5.2.1 Management Message Protection

IEEE 802.16m supports the selective confidentiality protection over MAC management messages. Through capability negotiation, AMS and ABS know whether the selective confidentiality protection is applied or not. If the selective confidentiality protection is activated, the negotiated keying materials and cipher suites are used to encrypt the management messages.

Figure 21 presents three levels of selective confidentiality protection over management messages in IEEE 802.16m. Strongest applicable protection method is used thus the explicit identification of the used protection method is not needed.

• No protection: If AMS and ABS have no shared security context (no valid AK) or protection is not required, then the management messages are neither encrypted nor authenticated. Management messages before the successful completion of the authentication phase also fall into this category.

• CMAC based integrity protection; CMAC Tuple TLV is included to the end of management message as a last TLV. CMAC integrity protects only payload, not header part. Actual management message is plain text. CMAC integrity protection is used when AMS and ABS shares security context (valid AK), but not valid TEK. Key agreement management messages fall into this category.

• AES-CCM based authenticated encryption; ICV field is included after encrypted payload and this ICV integrity protects both payload and MAC header part. AES-CCM based protection is used when AMS and ABS shares security context (valid AK) and valid TEK.

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

# References

[1] The Draft IEEE 802.16m System Description Document, IEEE 802.16m-08/003r7