| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support for ARS |  |
| Date Submitted | 2009-3-8 |  |
| Source(s) | Yousuf Saifullah, Adrian Boariu, Shashikant Maheshwari, Haihong Zheng Nokia Siemens Networks Zexian Li Nokia Hyunjeong Kang, Mingxia Xu, Youngbin Chang, Rakesh Taori, Jungje Son | adrian.boariu@nsn.com zexian.li@nokia.com Hyunjeong.Kang@samsung.com |
| Re: | “802.16m SDD text”: IEEE 802.16m-08/003r7, “CCP”. Target topic: “15. Support for Multihop Relay”. |  |
| Abstract | Proposes data control functions for ARS |  |
| Purpose | Review and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Support for ARS

Yousuf Saifullah et al.

NSN

# Introduction

The contribution proposes text for ARS section in the SDD.

# Proposed text

15.4.1 MAC PDU construction

One or more tunnels may be established between the ABS and the access ARS after the network entry is performed. Each tunnel between an ARS and ABS is identified by a unique Flow ID. Connections of an AMS may be mapped to one or multiple tunnels.

The mode for constructing and forwarding MPDUs through a tunnel is called tunnel mode. In the tunnel mode, MAC PDUs that traverse a tunnel shall be encapsulated in a relay MAC PDU with the relay MAC header carrying a tunnel identifier. Multiple MAC PDUs from connections that traverse the same tunnel can be concatenated into a relay MAC PDU for transmission.

15.4.6 Topology Discovery

An ABS determines that an AMS/an ARS sending initial ranging is directly accessing the ABS, or through an ARS. The ABS discovers topology information of all the ARS and AMS connected through it during the initial ranging.