| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Mandatory Fragmentation on transport and control connections |  |
| Date Submitted | 2009-02-27 |  |
| Source(s) | Thomas Schneider Broadband Mobile Technologies, Inc. | E-mail: tschneider@broadbandmobiletech.com |
| Re: | Change Request on 802.16m SDD (802.16m-08/003r7) |  |
| Abstract | This contribution proposes the mandatory requirement of Fragmentation on the transport and control connections |  |
| Purpose | To be discussed and adopted for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Mandatory Fragmentation on control and transport connections

Thomas Schneider

Broadband Mobile Technologies, Inc.

- Introduction

Fragmentation is the process by which a MAC SDU (or MAC management message) is divided into one or more MAC PDUs. This process is undertaken to allow efficient use of available bandwidth relative to the QoS requirements of a connection’s service flow. Capabilities of fragmentation and reassembly are mandatory.

The authority to fragment traffic on a connection is defined when the connection is created by the MAC SAP. Fragmentation may be initiated by an ABS for DL connections and by a AMS for UL connections, but is not required.

# 2.  Problem Definition

Currently, fragmentation is supported on transport connections and may be supported on unicast management connections.  The unavailability of fragmentation as a feature on MAC management messages during Initial Network Entry and on key MAC management messages is a substantial weakness. Inability to fragment what can be substantial sized messages limits the ability for terminals to perform at cell edge. This was a problem in 16e, and now we are perpetuating the problem into 16m.

- Proposed Solution

Require all terminals and base stations to include fragmentation as a standard for transport connections and enabled for unicast management connections.  With fragmentation as a default standard, AMS network entry may be simplified and improved.

Minimum agreed upon initial parameters will need to be defined.

- Proposed Text

Modify  Section 10.9 Connection Management

Page 56, Lines 25-26 replace existing text to read:

Fragmentation shall be mandatory on all transport connections and shall be enabled on all unicast management connections.  A set of minimum MAC SDU fragmentation and performance parameters is FFS and shall be mandatory for AMS and ABS.