| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Change Request to IEEE 802.16m SDD: Data regions and Subcarrier Mapping for Multi-carrier Operation |  |
| Date Submitted | 2009-03-09 |  |
| Source(s) | Kuk-Jin Song Kyounghwan Lee Jungnam Yun POSDATA | Voice: +1-408-986-1143 E-mail: song@posdata.co.kr khlee@posdata-usa.com jnyun@posdata-usa.com |
| Re: | Change Request for IEEE 802.16m System Description Document (SDD) – Section 19.3 |  |
| Abstract | This contribution proposes changes to the Section 19.3 of IEEE 802.16m SDD. |  |
| Purpose | To discuss and adopt the proposed text in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Change Request to IEEE 802.16m SDD: Data regions and Subcarrier Mapping for Multi-carrier Operation

Kuk-Jin Song, Kyounghwan Lee, Jungnam Yun

POSDATA

- Introduction

This contribution proposes a UL data regions and sub-carrier mapping scheme for OFDMA multi-carrier operation in IEEE 802.16m. One mapping scheme is captured in the current SDD [1]. The existing scheme is applied to both UL and DL.

AMC allows AMS (Advanced Mobile Station) to match modulation scheme and coding rate to the time-varying channel conditions. This leads to optimize the overall network resources and enhance system throughput. To maximize the efficiency of AMC, a careful design should be considered for UL data region mapping scheme since the allocated data region for each carrier can be overlapped in time (subframe) and an AMS has limited transmitted power per symbol. To avoid this problem, we propose an efficient UL data region mapping scheme for multi-carrier operation.

- Mapping Algorithm in SDD [1]

Section 19.3.5 defines the mapping algorithm as following

- Segment the modulated symbol sequence into blocks sized to fit into a single LRU.

- Map each segmented block onto one LRU from the lowest LRU index in the OFDMA data region of the carrier with the lowest logical carrier index.

- Continue the mapping so that the LRU index increases. When the edge of the data region is reached, continue the mapping from the lowest LRU index in the OFDMA data region of the carrier with the next available logical carrier index.

- Continue the mapping until the all modulated data symbols are mapped.

Figure 1 shows an example of the above algorithm [1].

Figure 1 Example of mapping in SDD

The mapping algorithm is good for DL allocation. However, we note that this mapping scheme is not efficient for UL allocation if MCS is applied for UL transmission. Due to the limited transmitted power per symbol for an AMS, the overlapped data regions in a subframe prevent the AMS from maintaining MCS level especially for AMSs in near cell edge. This leads to performance degradation.

- UL Data Regions and Sub-carriers Mapping

The key idea is to transmit data such that each transmission in carrier is not overlapped in a subframe. By doing so, AMS can maintain the chosen MCS level for whole data region. One example of the proposed UL data mapping scheme is shown in Figure 2. The detail description of the proposed UL mapping scheme is as following:

- Segment the modulated symbol sequence into blocks sized to fit into a single LRU.

- Map each segmented block onto one LRU from the lowest LRU index in the OFDMA data region of the carrier with the lowest logical carrier index.

- Continue the mapping so that the LRU index increases. When the edge of the data region is reached, continue the mapping from the lowest LRU index in the OFDMA data region of the carrier with the next available logical carrier index (not necessarily increasing order) in the next subframe.

- Continue the mapping until the all modulated data symbols are mapped.

Figure 2 shows an example of the proposed UL data region mapping for multi-carrier operation.

Figure 2 UL Data Region Mapping for Multicarrier Operation

- Proposed Text for the 802.16m SDD

Delete “at the same subframe” in line 10 and 11 on page 143.

---------------------------  Start of the proposed text  --------------------------------------------------------------

B. Transmitting the modulated symbol sequence on DRUs across several RF carriers at the same

subframe, via PHY burst segmentation and mapping to different RF carriers, by using the same

MCS and MIMO scheme.

---------------------------  End of the proposed text   ---------------------------------------------------------------

References

- IEEE 802.16m-08/003r7. The Draft IEEE 802.16m System Description Document, February 2008.