| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Frame Structure for a System Bandwidth of 8.75 MHz |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Sungho Moon, Jin Sam Kwak, Minseok Noh LG Electronics | Voice : +82-31-450-1852 E-mail: msungho@lge.com, samji@lge.com |
| Re: | Re: IEEE 802.16m-09/0012 Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content - Call for Comments on IEEE 802.16m-09/0010 Amendment Working Document |  |
| Abstract | The contribution proposes the text of frame structure section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Frame Structure for a System Bandwidth of 8.75 MHz

Sungho Moon, Jin Sam Kwak, Minseok Noh

LG Electronics

# Introduction

This contribution includes the proposed text for the frame structure section in the IEEE 802.16m Amendment. The technical text is inherited from the 802.16m SRD [1], the IEEE 802.16m SDD [2], and the IEEE P802.16 Rev2/D7 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4]. In this contribution, amendment text including several illustrations is proposed for the section in which 16m members have enough consensus.

# Proposed Frame Structure for the 8.75MHz bandwidth

### 4.1 OFDMA parameters

Table I shows the current OFDMA parameters included in the amendment text [5]. The [TBD] fields in the bandwidth of 8.75 MHz and G=1/16 are modified into detailed values for defining a frame structure of 8.75 MHz.

Table I – OFDMA parameters

| The nominal channel bandwidth, BW (MHz) |  | 5 | 7 | 8.75 | 10 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| Sampling factor, n |  | 28/25 | 8/7 | 8/7 | 28/25 | 28/25 |
| Sampling frequency, Fs (MHz) |  | 5.6 | 8 | 10 | 11.2 | 22.4 |
| FFT size, NFFT |  | 512 | 1024 | 1024 | 1024 | 2048 |
| Subcarrier spacing, Δf(kHz) |  | 10.94 | 7.81 | 9.77 | 10.94 | 10.94 |
| Useful symbol time, Tb (us) |  | 91.4 | 128 | 102.4 | 91.4 | 91.4 |
| CP ratio, G = 1/8 | OFDMA symbol time, Ts (us) | 102.82 | 144 | 115.2 | 102.82 | 102.82 |
|  | Number of OFDMA symbols per 5ms frame | 48 | 34 | 43 | 48 | 48 |
|  | Idle time (us) | 62.86 | 104 | 46.40 | 62.86 | 62.86 |
| CP ratio, G = 1/16 | OFDMA symbol time, Ts (us) | 97.143 | [TBD] | 108.8 | 97.143 | 97.143 |
|  | FDD Number of OFDMA symbols per 5ms frame | 51 | [TBD] | 45 | 51 | 51 |
|  | Idle time (us) | 45.71 | [TBD] | 104 | 45.71 | 45.71 |

### 4.2 Design Criteria

In the current SDD and amendment text, three types of subframes have been defined, i.e., type-1 with 6 symbols, type-2 with 7 symbols, and type-3 with 5 symbols. Therefore, the main rule of our design is to use these subframe types to define a new frame structure of 8.75 MHz. A new subframe type like 3 symbol subframe or 9 symbol subframe should be avoided to reduce a burden to design a new PHY structure. Here are our design criteria:

- A new frame structure should be designed by type-1, type-2, and type-3 subframes.

- New subframe type definitions should be avoided to reduce a burden PHY design.

- Type-1 subframes should be utilized in a frame as many as possible since it is a default subframe type optimized in a PHY design.

- There should be commonalities between TDD and FDD.

- The switching point of TDD frame structure having a CP length of 1/16 Tb should be aligned with that of the 16m system with a CP length of 1/8 Tb.

### 4.3 Frame structure for CP = 1/8 Tb

Since there are 43 OFMDA symbols for CP = 1/8 Tb, seven type-1 subframes can be defined within a TDD frame. One remaining OFDMA symbol could be used for TTG and RTG durations.

Figure 1 shows examples of the proposed TDD and FDD frame structures. In TDD, all subframes are type-1 subframe with a TTG of 87.2 us and a RTG of 74.4 us, which are exactly the same as those in the WiMAX profile. The FDD frame structure has a commonality with the TDD frame structure in the number of subframes, and six subframes can be configured as the type-1 subframe, and one subframe should be configured as the type-2 subframe with 7 OFDMA symbols. The location of the type-2 subframe is FFS.

Figure 1 Examples of TDD and FDD Frame Structures with a CP of 1/8 Tb

Table II shows the number of OFDM symbols in DL and UL in the WiMAX profile [6]. In case of the system bandwidth of 8.75MHz, there are 7 possibilities in the number of UL OFDM symbols, which is from 12 to 18. Therefore, the proposed frame structure has no problem to support the backward compatibility when the WirelessMAN-OFDMA system operates with 12 or 18 OFDM symbols.

Table II. Number of DL and UL OFDM symbols in WiMAX profile

| Item | Description | Reference | Status | BS Required | BS Values | MS Required | MS Values | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Number of OFDM Symbols in DL and UL for 5 and 10 MHz BW | 8.4.4.2 | oi | Y | (35, 12), (34, 13), (33, 14), (32, 15), (31, 16), (30, 17), (29, 18), (28, 19), (27, 20), (26, 21) | Y | The same as BS values |  |
| 2 | Number of OFDM Symbols in DL and UL for 8.75 MHz BW | 8.4.4.2 | oi | Y | (30, 12), (29, 13), (28, 14), (27, 15), (26, 16), (25, 17), (24, 18) | Y | The same as BS values |  |
| 3 | Number of OFDM Symbols in DL and UL for 7 and 3.5 MHz BW | 8.4.4.2 | oi | Y | (24, 09), (23, 10), (22, 11), (21, 12), (20, 13), (19, 14), (18, 15) | Y | The same as BS values |  |

However, if the WirelessMAN-OFDMA system should operate with 15 UL OFDM symbols in the legacy support mode, other frame structures for legacy-supports should be defined in order not to define a new subframe type for the AAIF.

Figure 2 shows an example of the proposed frame structure to support the WirelessMAN-OFDMA with 27 DL and 15 UL OFDM symbols. There exist two frame structures depending on the multiplexing in UL. For both cases, the first DL subframe is consisting of 3 OFDM symbols, and it can be only used for the WirelessMAN-OFDMA DL and UL Zones. All subframes in the AAIF DL Zone are type-1 subframes. The number of symbols in the WirelessMAN-OFDMA DL Zone is 3+6·(FRAME_OFFSET-1). In UL TDM mode, all subframes in the AAIF UL Zone are type-1 subframes, and in UL FDM mode, all subframes in the AAIF UL Zone are type-3 subframes. Type-3 subframes in UL may cause the performance degradation, and thus, two type-2 subframes can be used instead of three type-3 subframes.

Figure 2 Example of TDD frame to support the WirelessMAN-OFDMA with 15 UL OFDM symbols

### 4.4 Frame structure for CP = 1/16 Tb

For a CP length of 1/16 Tb, the switching point alignment with the legacy or the 16m system with a CP length of 1/8 Tb should be one of the important design criteria due to the DL and UL interference problem in a TDD mode. Moreover, the proposed frame structure should have a commonality with those of 5, 10, and 20 MHz.

For nominal channel bandwidths of 5, 10, and 20 MHz, Each FDD frame have five type-1 subframes and three type-2 subframes, and each TDD frame should have six type-1 subframes and two type-2 subframes.

Here, we propose TDD and FDD frame structures not only allowing the coexistence with the legacy and the 16m systems operating with a CP length of 1/8 Tb, but also having a commonality of subframe configurations.

In TDD, two type-2 subframes should be located in DL and UL region separately, and the last one OFDMA symbols in another type-2 subframe is an idle symbol, which is the same as those of 5, 10, 20 MHz bandwidth cases. All other DL/UL ratio cases, i.e., (2:5), (3:4), (5:2), and (6:1) can satisfy the coexistence with the 16m systems operating with a CP length of 1/8 Tb. In the FDD frame, there exists an additional type-2 subframe, and the location of this subframe is FFS.

Figure 3 Examples of TDD and FDD Frame Structures with a CP of 1/16 Tb

# References

[1] IEEE 802.16m-07/002r4, “802.16m System Requirements.”

[2] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document.”

[3] IEEE P802.16 Rev2/D7, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment.”

[5] IEEE 802.16m-08/050, “IEEE 802.16m Amendment Working Document.”

[6] WiMAX Forum™ Mobile System Profile Release 1.0 Approved Specification (Revision 1.7.1: 2008-11-07).

# Text proposal for the 802.16m amendment

============================== Start of Proposed Text =================================

[Insert the following text and illustration after Figure 390 on page 16:]

Figure 390-1 illustrates an example FDD frame structure, which is applicable to the nominal channel bandwidth of 8.75 MHz with G = 1/8. The fourth subframe among seven subframes is the type-2 subframe including 7 OFDM symbols.

Figure 390-1 Frame structure in FDD for the 8.75 MHz channel bandwidth (CP = 1/8 Tb)

[Insert the following text after “as shown in Figure 393 and Figure 394.” in line 43 on page 18]

When the Advanced Air Interface frames support the WirelessMAN-OFDMA for the 8.75 MHz channel bandwidth, all subframes in the Advanced Air Interface DL are type-1 subframes. The number of symbols in the WirelessMAN-OFDMA DL Zone is 6·FRAME_OFFSET.

When the Advanced Air Interface frames support the WirelessMAN-OFDMA for the 8.75 MHz channel bandwidth with 15 UL OFDM symbols, all subframes in the Advanced Air Interface DL Zone are type-1 subframe. The number of symbols in the WirelessMAN-OFDMA DL Zone is 3+6·(FRAME_OFFSET – 1).

[Insert the following text after “when FDM mode is used.” in line 2 on page 19]

In case of 5/10/20 MHz and 8.75 MHz with 12, 18 UL OFDM symbols, all UL subframes are type-1 subframes. In case of 8.75 MHz with 15 UL OFDM symbols, all UL subframes are type-3 subframes.

[Insert the following text after “when TDM mode is used.” in line 46 on page 19]

In case of 5/10/20 MHz and 8.75 MHz, all UL subframes are type-1 subframes.

[Insert the following text and illustration after Figure 391 on page 17:]

Figure 391-1 illustrates an example TDD frame structure with D:U = 4:3, which is applicable to the nominal channel bandwidths of 8.75 MHz with G = 1/8.

Figure 391-1—Frame structure in TDD for 8.75 MHz channel bandwidth (CP=1/8 Tb)

[Insert the following text and illustration after Figure 392 on page 18:]

For a channel bandwidth of 8.75 MHz, a frame shall have four type-1 subframes and three type-2 subframes for FDD, and five type-1 subframes and two type-2 subframes for TDD.

In the TDD frame, the first and last subframes within each frame shall be type-2 subframes. In the FDD frame, the first, forth, and last subframe within each frame shall be type-2 subframes.

Figure 392-1 illustrates an example of TDD and FDD frame structure for the 8.75 MHz channel bandwidth with a CP of 1/16 Tb. Assuming OFDMA symbol duration of 108.8 μs and a CP length of 1/16 Tb, the length of type-1 and type-2 subframes are 0.6528 ms and 0.7616 ms, respectively.

Figure 392-1— TDD and FDD frame structure for 8.75 MHz channel bandwidth with a CP of 1/16 Tb (DL to UL ratio of 4:3)

============================== End of Proposed Text =================================