| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Frame Structure for a System Bandwidth of 7 MHz |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Sungho Moon, Jin Sam Kwak LG Electronics Jaeweon Cho Samsung Electronics Co., Ltd. | Voice : +82-31-450-1852 E-mail: msungho@lge.com, samji@lge.com E-mail: jaeweon.cho@samsung.com |
| Re: | Re: IEEE 802.16m-09/0012 Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content - Call for Comments on IEEE 802.16m-09/0010 Amendment Working Document |  |
| Abstract | The contribution proposes the text of frame structure section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Frame Structure for a System Bandwidth of 7 MHz

Sungho Moon, Jin Sam Kwak

LG Electronics

Jaeweon Cho

Samsung Electronics

# Introduction

This contribution includes the proposed text for the frame structure section in the IEEE 802.16m Amendment. The technical text is inherited from the 802.16m SRD [1], the IEEE 802.16m SDD [2], and the IEEE P802.16 Rev2/D7 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4]. In this contribution, amendment text including several illustrations is proposed for the section in which 16m members have enough consensus.

# Proposed Frame Structure for the 7 MHz bandwidth

### 4.1 OFDMA parameters

Table I shows the current OFDMA parameters included in the amendment text [5]. The [TBD] fields in the bandwidth of 7 MHz and G=1/16 are modified into detailed values for defining a frame structure of 7 MHz.

Table I – OFDMA parameters

| The nominal channel bandwidth, BW (MHz) |  | 5 | 7 | 8.75 | 10 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| Sampling factor, n |  | 28/25 | 8/7 | 8/7 | 28/25 | 28/25 |
| Sampling frequency, Fs (MHz) |  | 5.6 | 8 | 10 | 11.2 | 22.4 |
| FFT size, NFFT |  | 512 | 1024 | 1024 | 1024 | 2048 |
| Subcarrier spacing, Δf(kHz) |  | 10.94 | 7.81 | 9.77 | 10.94 | 10.94 |
| Useful symbol time, Tb (us) |  | 91.4 | 128 | 102.4 | 91.4 | 91.4 |
| CP ratio, G = 1/8 | OFDMA symbol time, Ts (us) | 102.82 | 144 | 115.2 | 102.82 | 102.82 |
|  | Number of OFDMA symbols per 5ms frame | 48 | 34 | 43 | 48 | 48 |
|  | Idle time (us) | 62.86 | 104 | 46.40 | 62.86 | 62.86 |
| CP ratio, G = 1/16 | OFDMA symbol time, Ts (us) | 97.143 | 136 | [TBD] | 97.143 | 97.143 |
|  | FDD Number of OFDMA symbols per 5ms frame | 51 | 36 | [TBD] | 51 | 51 |
|  | Idle time (us) | 45.71 | 104 | [TBD] | 45.71 | 45.71 |

### 4.2 Design Criteria

In the current SDD, three types of subframes have been defined, i.e., type-1 with 6 symbols, type-2 with 7 symbols, and type-3 with 5 symbols. Therefore, the main rule of our design is to use these subframe types to define a new frame structure of 7 MHz. A completely new subframe type such as 3 symbol subframe or 9 symbol subframe should be avoided to reduce a burden of PHY structure designs. Here are our summarized design criteria:

- A new frame structure should be designed by type-1, type-2, and type-3 subframes.

- New subframe type definitions should be avoided to reduce a burden PHY design.

- The type-1 subframe should be utilized more in a frame since it is a default subframe type optimized in a PHY design.

- There should be commonalities between TDD and FDD.

- The switching point of TDD frame structure having a CP length of 1/16 Tb should be aligned with that of the 16m system with a CP length of 1/8 Tb.

### 4.3 Frame structure for CP = 1/8 Tb

Figure 1 shows examples of the proposed TDD and FDD frame structure. In TDD, there are three type-1 subframes and three type-3 subframes with 5 OFDM symbols with a TTG of 188 us and a RTG of 60 us, which are exactly the same as those in the WiMAX profile. The FDD frame structure has a commonality with the TDD frame structure in the number of subframes, and four subframes can be configured as the type-1 subframe, and two subframe should be configured as the type-3 subframe with 5 OFDMA symbols. The location of the type-1 and type-3 subframes is FFS.

Figure 1 Examples of TDD and FDD Frame Structures with a CP of 1/8 Tb

### 4.4 Frame structure for CP = 1/16 Tb

For a CP length of 1/16 Tb, the switching point alignment with the legacy or the 16m system with a CP length of 1/8 Tb should be one of the important design criteria due to the DL and UL interference problem in a TDD mode. Moreover, the proposed frame structure should have a commonality with those of 5, 10, and 20 MHz.

Figure 2 shows examples of the proposed frame structure. The proposed TDD and FDD frame structures allows the coexistence with the legacy and the 16m systems operating with a CP length of 1/8 Tb. With a CP length of 1/8Tb, the proposed frame structure satisfies a DL/UL alignment in TDD. In the FDD frame, all of the subframes are type-1 subframes since 36 OFDMA symbols can exist within a frame. In TDD, one OFDMA symbol should be an idle symbol and thus, there is one type-3 subframe instead of one type-1 subframe, which is the same as those of 5, 10, 20 MHz bandwidth cases.

Figure 2 Examples of TDD and FDD Frame Structures with a CP of 1/16 Tb

# References

[1] IEEE 802.16m-07/002r4, “802.16m System Requirements.”

[2] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document.”

[3] IEEE P802.16 Rev2/D7, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment.”

[5] IEEE 802.16m-08/050, “IEEE 802.16m Amendment Working Document.”

# Text proposal for the 802.16m amendment

============================== Start of Proposed Text =================================

[Insert the following text and illustration after Figure 390 on page 16:]

Figure 390-2 illustrates an example FDD frame structure, which is applicable to the nominal channel bandwidth of 7 MHz with G = 1/8. Four subframes among six subframes are the type-1 subframes, and the other two subframes are the type-3 subframe. The [third] and [forth] subframes are the type-3 subframe.

Figure 390-2 — Frame structure in FDD for the 7 MHz channel bandwidth (CP = 1/8 Tb)

[Insert the following text and illustration after Figure 391 on page 17:]

Figure 391-2 illustrates an example TDD frame structure with D:U = 4:2, which is applicable to the nominal channel bandwidths of 7 MHz with G = 1/8. Three subframes among six subframes are the type-1 subframes, and the other three subframes are the type-3. The [second], [third] and [forth] DL subframes are the type-3 subframe. TTG and RTG are 188μs and 60μs, respectively.

Figure 391-2—Frame structure in TDD duplex mode for 7 MHz channel bandwidth (CP=1/8 Tb)

[Insert the following text and illustration after Figure 392 on page 18:]

For a channel bandwidth of 7 MHz, a frame shall have seven type-1 subframes for FDD, and six type-1 subframes and one type-3 subframes for TDD. In the TDD frame, the subframe preceding a DL to UL switching point is a type-3 subframe.

Figure 392-2 illustrates an example of TDD and FDD frame structure for the 7 MHz channel bandwidth with a CP of 1/16 Tb. Assuming OFDMA symbol duration of 136 μs and a CP length of 1/16 Tb, the length of type-1 and type-3 subframes are 0.816 ms and 0.680 ms, respectively. TTG and RTG are 180μs and 60μs, respectively.

Figure 392-2— TDD and FDD frame structure for 7 MHz channel bandwidth with a CP of 1/16 Tb (DL to UL ratio of 4:3)

============================== End of Proposed Text =================================