| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications on the Carrier Management |  |
| Date Submitted | 2009-03-11 |  |
| Source(s) | Nan Li,Yuqin Chen ZTE Corporation Xiaolu Dong, Ying Du, Daning Gong CATR I-Kang Fu Mediatek Inc. | Voice: E-mail: chen.yuqin@zte.com.cn li.nan25@zte.com.cn IK.Fu@mediatek.com |
| Re: | IEEE 802.16m-09/0008 “IEEE 802.16m Internal Documents Configuration Control Procedure (CCP)” |  |
| Abstract | Proposes text to multicarrier management. |  |
| Purpose | Discussion and Approval |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications on the carrier management

Nan Li, Yuqin Chen

ZTE Corporation

Xiaolu Dong, Ying Du, Daning Gong

CATR

I-Kang Fu

Mediatek Inc.

# 1. Introduction

The current SDD draft has listed out principle of multicarrier operation. However, besides primary carrier management, details about secondary carrier management need to be clarified, including secondary carrier update and release. The secondary carrier update and release are required especially during AMS HO. The chapter of the text is also revised slightly to make the structure clearer.

# 2 Text Proposal for SDD

The following text is proposed to be adopted in the IEEE 802.16m system description document.

---------------------------------------------------- Start of the Text Proposal ----------------------------------------------

19.4.7 Primary Carrier Change

The following steps summarize the high level sequence of procedures involved in the MC operation:

…

Based on AMS RF capabilities, loading of available carriers or other factors, the ABS may provide more detailed configuration information on subset of available carrier designated as Assigned Secondary Carriers to AMS. The ABS may update and release the assigned secondary carriers based on QoS requirement, loading and other factors.

The AMS does not perform any PHY/MAC processing on Assigned Secondary Carriers until directed by the ABS.

- The ABS allocates a subset of assigned secondary carriers to be ready for the potential use for MC data transmission based on QoS requirement, loading and other factors. This subset is called the Active Secondary Carriers.

- AMS performs PHY/MAC processing on those active carriers. The ABS may update and release the active secondary carriers based on QoS requirement, loading and other factors.

…

-----------------------------------------------------End of the Text Proposal ----------------------------------------------