| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Femtocell Idle Mode |  |
| Date Submitted | 2009-02-27 |  |
| Source(s) | Baowei Ji, Hyunjeong Kang, Jungje Son, Rakesh Taori, Anshuman Nigam Samsung Electronics | baowei.ji@samsung.com |
| Re: | CR to tgmsdd |  |
| Abstract |  |  |
| Purpose | For discussion and adoption into SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Harmonized text on Femto Idle State

Baowei Ji, Hyunjeong Kang, Jungje Son, Rakesh Taori, Anshuman Nigam

Samsung Electronics

SDD Text Proposal

Black text: existing text in SDD (C802.16m-08/003r7)

Red Strikethrough text: deleted text

Blue text: new text

Introduction

To broadcast paging message occupies a huge amount overhead for all the BSs in a paging area. When the network is aware of that an MS is inside a CSG Femtocell, it is desirable to have only the Femtocell BS(s) belong to that CSG to page this MS, so that the paging overhead is saved for all the other BSs in the paging area.

To fulfill this purpose, the only requirement in the air link specification is the requirement for an MS to do location update whenever it enters or leaves a CSG Femtocell. Please see the suggested text in this document.

Moreover, the suggested text also specifies the MS behavior in idle mode, which is missing in the existing SDD text in Section 17.6 Idle Mode.

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

17.6	Idle Mode

The OSG Femtocell BSs operate like macro BSs when paging an MS.

Femtocell BS shall support idle mode. The CSG Femtocell BSs may broadcast the paging messages that are

related to only the MSs of this CSG. Whether the Femto BS and the Overlay BS share the same set of PGID or use different set of PGIDs is out of scope of the 16m specification. When the MS detects a change in the PGID it will perform location update. When an MS enters or leaves a CSG coverage area to which it is registered, the MS may do location update so that the MS may be paged only by the Femtocell BS(s) of this CSG when it is inside the CSG Femtocell(s).

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++