| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Change Request to IEEE 802.16m SDD - Bandwidth Request Channel |  |
| Date Submitted | 2009-02-27 |  |
| Source(s) | Xiangying Yang Yuan Zhu Jong-Kae Fwu Roshni M Srinivasan Intel Corporation | xiangying.yang@intel.com |
| Re: | CR for IEEE 802.16m System Description Document (SDD) - Section 11.9.2.5 Bandwidth Request Channel |  |
| Abstract | This contribution proposes changes to Section 11.9.2.5 Bandwidth Request Channel in the IEEE 802.16m SDD document |  |
| Purpose | For discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Change Request to IEEE 802.16m SDD - Bandwidth Request Channel

Xiangying Yang, Yuan Zhu, Roshni Srinivasan, Jong-kae Fwu

Intel Corporation

# Introduction

The current bandwidth request procedure diagram leads to confusion in operations in LZone and MZone. Based on the different SDD Bandwidth Request Channel PHY structures for LZone PUSC and other cases (MZone or LZone AMC), we propose text change to clarify the different procedures used in LZone and MZone.

# Text Proposal

----------------------------------------------Start of the text proposal ------------------------------------------------------

Modify section 11.9.2.5 as follows:

----------------------------------------------Text Change #1 ------------------------------------------------------

#### 11.9.2.5 Bandwidth Request Channel

Contention based or non-contention based random access is used to transmit bandwidth request information on this control channel. Prioritized bandwidth requests are supported on the bandwidth request channel. The mechanism for such prioritization is TBD.

The random access based bandwidth request procedure for MZone or LZone with AMC is described in Figure 1. In these cases, Aa 5-step regular procedure (step 1 to 5) or an optional 3-step quick access procedure (step 1,4 and 5) may be supported concurrently. Step 2 and 3 are used only in 5-step regular procedure. In step 1, AMS sends a bandwidth request indicator and a message for quick access that may indicate information such as AMS addressing and/or request size (FFS) and/or uplink transmit power report (FFS), and/or QoS identifiers (FFS), and the ABS may allocate uplink grant based on certain policy. The 5-step regular procedure is used independently or as a fallback mode for the 3-step bandwidth request quick access procedure. The AMS may piggyback additional BW REQ information along with user data during uplink transmission (step 5). Following Step 1 and Step 3, ABS may acknowledge the reception of bandwidth request. If AMS does not receive any acknowledgement or UL grant, it waits until the expiration of a pre-defined period and restarts the bandwidth request. The pre-defined period may be differentiated by factors such as QoS parameters (e.g. scheduling type, priority, etc). In case BW is granted immediately, there is no need for ABS to send explicit Ack.

Figure 1 Bandwidth Request Procedure in MZone or LZone with AMC

The bandwidth request procedure for LZone with PUSC is described in Figure 49. In LZone with PUSC, only a 5-step regular procedure is supported. In step 1, AMS sends a bandwidth request indicator only. The rest of LZone with PUSC bandwidth request procedure shall be the same as the 5-step procedure in Figure 48.

Figure 49 Bandwidth Request Procedure in LZone with PUSC

----------------------------------------------Text Change #2 ------------------------------------------------------

11.9.2.5.2	PHY structure

In the LZone with PUSC, a BW REQ tile is defined as 4 contiguous subcarriers by 6 OFDM symbols. The number of BW REQ tiles per BW REQ channel is 3 or 6. Each BW REQ tile carries a BW REQ access sequence.

-------------------------------------------------- End of the text --------------------------------------------------