| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HARQ Timing and Signaling for Persistent Allocation for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-09 |  |
| Source(s) | Wang Junhu, SunBo ZTE Corporation | Voice: [Telephone Number (optional)]] E-mail: {wang.junhu, sun.bo1}@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: “HARQ functions” |  |
| Abstract | The contribution proposes text for HARQ functions to be included in the 802.16m amendment. In particular, HARQ signaling and timing for persistent allocation is discussed herein. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

HARQ Timing and Signaling for Persistent Allocation for the IEEE 802.16m Amendment

Wang Junhu, Sun Bo

ZTE Corporation

# 1 Introduction

Persistent allocation is a technique used to reduce A-MAP overhead for connections with periodic traffic pattern and with relatively fixed payload size. In 802.16e, persistent allocated sub-bursts are grouped into a predefined semi-static region or a dynamically allocated region. However, there is no specific description on the HARQ of the sub-burst transmitted in the Persistent allocation (PA ) region. And this would result in much overhead if full signal indication is needed for every sub-burst retransmission.

To reduce the overhead of retransmission of the sub-burst in the PA region, an HARQ method for persistent scheduling is provided in this contribution.

- Proposed Scheme

PA regions are allocated in downlink and uplink. Sub-packets in the PA regions have periodic traffic, e.g. 20ms. The HARQ sub-packets for the PA region will be transmitted in every radio frame if necessary. Based on PA region, the related Retransmission Regions are allocated in the same or the next frame. The allocation of retransmission for downlink is illustrated in Fig1.

Fig 1

P1 is PA region, and r1, r2 and r3 in the Radio Frame 2 are 1st retransmission region,2nd retransmission region and 3rd retransmission region respectively to P1. P2 is another allocated PA region which can be used for the other group of persistent scheduling users.

The attributes of all the sub-packets transmitted in P1 will be organized in a table including the duration of each sub-packet and its reduced CID, which is called the Sub-packets Attribute Table. ABS will update the table in case there is change in P1. ABS informs all AMSs which have traffic in P1 the updated table. The Sub-packets Attribute Table can be illustrated as following.

Table 1: The Sub-packets Attribute Table

| Sub-burst | RCID | Duration |
| --- | --- | --- |
| 1 | xbit | xbit |
| 2 | xbit | xbit |
| 3 | xbit | xbit |
| 4 | xbit | xbit |
| 5 | xbit | xbit |
| … |  | … |

In one radio frame, the retransmission regions can be concatenated to PA region. The start of the PA region is fixed. ABS only needs to notify the offset of each retransmission regions to the start of PA region.

The HARQ process is illustrated in Fig 2 for example. Sub-packets transmitted in P1 will be retransmitted in r1 when NACK is fed back. And the second retransmission will be allocated in r2. The third retransmission will be allocated in r3.

Fig 2

There’re three Retransmission Index Bitmaps for each retransmission region respectively. Each sub-packet in the retransmission regions are concatenated to reduce resource holes caused by early terminated HARQ process. Each sub-packet in the retransmission region is corresponding to one bit in corresponding Retransmission Index Bitmap. When the bit is set to 0 (assuming NACK as 1, and ACK as 0), it means the sub-packet will not be retransmitted in the retransmission region. Thus, the following retransmitted sub-packets will be shift forward to cover the resource hole.

Take Fig 3 for example, the first retransmission ACK/NACK Index Bitmap is 11001. The ACK/NACK feedback of the second retransmission is 0,1,0, the ACK/NACK Index Bitmap is 01000. Each bit is corresponding to each item in the Persistent Allocation Sub-packets Table. And for each sub-packet, its duration can be obtained by checking the Sub-packets Attribute Table to determine the offset of each sub-burst in the retransmission region. If the ACK/NACK Index Bitmap is all zero, the corresponding retransmission region is cancelled.

Fig 3

4 Conclusion

The proposed method can reduce signal overhead of retransmissions dramatically comparing the full signal indication required by asynchronous HARQ. The only overhead includes offsets of each retransmission regions and the ACK/NACK Index Bitmap, the size of which is the number of sub-bursts in the PA region. The Sub-bursts Attribute Table will be saved by MSs and updated in the case that there are changes in the PA region.

5  Text proposal for inclusion in the 802.16m amendment

=====================Start of Proposed Text =================================

15.2.x.3 Persistent allocation HARQ signaling and timing

15.2.x.3.2 Uplink

Persistent allocation (PA) is a technique used to reduce MAP overhead for connections with periodic traffic pattern and with relatively fixed payload size.

ABS allocates a region in a radio frame as PA region and the corresponding retransmission regions in the frames. The Retransmission region can be divided into several regions, which are respectively called 1st retransmission region, 2nd retransmission region, 3rd retransmission region. The offset of each retransmission region to the PA region is notified to the AMSs.

ABS must inform all AMSs involved in the PA region the latest Sub-packets Attribute. The Sub-packets Attribute Table includes information of reduced CID of each sub-burst and its duration in the PA region.

The Retransmission Index Bitmap reflects retransmission information of each involved sub-packets in PA region and Retransmission Regions. The offset of each sub-packet is determined by the Retransmission Index Bitmap and the Sub-packets Attribute Table. The sub-packets are concatenated thus to enable resource shifting to mitigate resource holes caused by early-terminated HARQ sub-bursts.

=====================end of Proposed Text =================================

6 References

[1] IEEE 802.16m-08/003r7, IEEE 802.16m System Description Document (SDD)

[2] IEEE 802.16m-09/0050r1, IEEE 802.16m Amendment Working Document (AWD)