| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Change to Subband Partitioning and Frequency Partitioning |  |
| Date Submitted | 2009-3-2 |  |
| Source(s) | 1Chang-Lan Tsai, 2Zheng Yan-Xiu, Chung-Lien Ho, Yu-Chuan Fang, Hsi-Min Hsiao, Ren-Jr Chen, Yu-Tao Hsieh ITRI | Voice: 1+886-3-5914686 2+886-3-5914715 E-mail: 1 tsaichangl@itri.org.tw 2 zhengyanxiu@itri.org.tw |
| Re: | IEEE 802.16m-09/0012 “Call for Comments and Contributions on Project 802.16m Amendment Working Document” Topic: “Downlink Physical Structure” |  |
| Abstract | This contribution proposed a modified subband partitioning and frequency partitioning scheme such that the subband locations for different number of subbands are aligned and the variation in subband allocation to each frequency partition is little. |  |
| Purpose | For discussion and approval by IEEE 802.16m TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Change to Subband Partitioning and Frequency Partitioning

Chang-Lan Tsai, Zheng Yan-Xiu, Chung-Lien Ho, Yu-Chuan Fang, His-Min Hsiao, Ren-Jr Chen, Yu-Tao Hsieh

ITRI

# Subband partition

Regarding to the subband partitioning scheme in the AWD [1], for some subband allocation count (SAC) values, the subband locations are almost different. As define in AWD [1], NPRU denotes the number of PRUs and Nsub is the maximum number of subbands. Indexing the possible location of subband from 0 to Nsub – 1, according to the equation (175), the subband locations, SKSB = {s(0), s(1), …, s(KSB – 1)}, where KSB denotes the number of subbands, which is indicated by SAC, are defined by

.

For NPRU = 48 and KSB = 4, 5, 6 as examples, the subbands are located at S4 = {0, 3, 6, 9}, S5 = {0, 1, 3, 6, 9}, S6 = {0, 2, 4, 6, 8}, respectively. They are also indicated in Figure 1. And Figure 2 shows the subband locations for all KSB values for NPRU = 48. The subband allocation rule here is to distributed the subbands in the bandwidth as uniformly as possible. See the cases of KSB = 2, 3, 4, and 6, the subbands are uniformly distributed.

However, the subband locations for different number of subbands are not overlapped. Observing Figure 2, the set of subband location S1 is not a subset of S2, i.e, S1S2. In this case of NPRU = 48, we can observe that SnSn+1, for n = 1, 2, 3, 4, 5. When the number of subbands, KSB, is changed in different superframes, the subband locations may be totally different. As a result, for example, the terminals need to feedback new channel quality indicator (CQI) and/or the base station has to adapt the to the link quality of the terminals.

Figure 1. The subband locations for KSB = 4, 5 and 6 in the case of NPRU = 48.

Figure 2. The subband locations for KSB = 4, 5 and 6 in the case of NPRU = 48.

# The proposed modification to subband partitioning

We propose a modified subband partitioning scheme such that the set of subband locations corresponding to small KSB is a subset of that corresponding to large KSB, i.e., SnSm, for all n < m. This can be achieved by designing a basic sequence with length Nsub, B = {b(0), b(1), …, b(Nsub – 1)} and defining the subband location as s(n) = b(n), for n = 0, 1, …, KSB – 1. Figure 3 is an example achieving this goal. For the case of NPRU = 48, the basic sequence is designed as B = {0, 6, 3, 9, 1, 4, 7, 10, 2, 5, 8, 11}. The subband locations for KSB = 5 and 6 are defined as S5 = {0, 6, 3, 9, 1} and S5 = {0, 6, 3, 9, 1, 4}, respectively.

Figure 3. An example of aligned subband allocations.

The mapping of PRUs to PRUSBs and PRUSBs is represented as

where

where

# Frequency partitioning

If the subband locations are different due to the change of subband number, the allocated subbands to each frequency partition are different. See Figure 4 for reference, where only subband partitioning is illustrated. The subbands partitioned to frequency partitions are totally different when KSB is changed from 5 to 6. The terminals assigned the resource in that frequency partition need to feedback new channel quality indicator (CQI) and/or the base station has to adapt the to the link quality of the terminals.

Figure 4. Frequency partitioning for the cases of KSB = 5 and 6.

# The proposed modification to frequency partitioning

We propose a modified frequency partitioning scheme such that the subbands allocated to frequency partitions nearly invariant when the number of subband is changed. The existing rule allocates the first KSB,FP0 subbands to frequency partition FP0, and the last (FPCT – 1)*FPSC subbands to FPi, for all i > 0. Each FPi, for i > 0, contains FPSC subbands. We propose to allocate the first (FPCT – 1)*FPSC subbands to FPi, for all i > 0. Each FPi, for i > 0, contains FPSC subbands. The remaining KSB,FP0 subbands are allocated to FP0. If the subband locations are aligned by the design in Section 2, the front subbands are invariant when the number of subbands is changed. We allocate the front subbands to frequency partitions {FP1, , FPFPCT-1} first.

# References

- IEEE 802.16m-09/0010, IEEE 802.16m Amendment Working Document, Jan. 29, 2009.

# Text proposal for the 802.16m amendment working document

[Suggested Remedy #1]

---------------------------------------------------  Start of Text  ---------------------------------------------------

[Change the mapping of PRUs to the PRUSB and PRUMB in Section 15.3.5.2.1]

- 15.3.5.2.1  Subband partitioning

[Line 13]

defines the mapping of PRUs to PRUMBs. Figure 399 illustrates the PRU to PRUSB and PRUMB mapping for a 5 MHz bandwidth with SAC equal to 3.

(174)

where

(175)

where GCD(x,y) is the greatest common divisor of x and y.

(175)

where b( j ), for , is a sequence designed to be subband locations with the property andfor all. The sequence for subband allocation is (TBD).

(176)

where

(177)

where GCD(x,y) is the greatest common divisor of x and y.

(177)

---------------------------------------------------  End of Text  ----------------------------------------------------

[Suggested Remedy #2]

---------------------------------------------------  Start of Text  ---------------------------------------------------

[Change the mapping of subband PRUs and miniband PRUs to frequency partitions in Section 15.3.5.2.3]

- 15.3.5.2.3  Frequency partitioning

[Line 27]

The mapping of subband PRUs and miniband PRUs to the frequency partition is given by Equation (181):

(181)

where

and

.

Figure 401 depicts the frequency partitioning BW=5 MHz, SAC=3, FPCT=2, FPS=12, and FPSC=1.

Figure 401 – Frequency partitioning

---------------------------------------------------  End of Text  ----------------------------------------------------

[Suggested Remedy #3]

---------------------------------------------------  Start of Text  ---------------------------------------------------

[Change the mapping of PRUs to the PRUSB and PRUMB in Section 15.3.8.2.1]

- 15.3.5.2.1  Subband partitioning

[Line 20]

Equation (188) defines the mapping of PRUs into PRUSBs. Equation (189) defines the mapping of PRUs to PRUMBs. Figure 407 illustrates the PRU to PRUSBs and PRUMBs mapping for a 5 MHz bandwidth with SAC equal to 3.

(188)

where

where b( j ), for , is a sequence designed to be subband locations with the property andfor all. The sequence for subband allocation is (TBD).

(189)

where

---------------------------------------------------  End of Text  ----------------------------------------------------

[Suggested Remedy #4]

---------------------------------------------------  Start of Text  ---------------------------------------------------

[Change the mapping of subband PRUs and miniband PRUs to the frequency partitions in Section 15.3.8.2.3]

- 15.3.5.2.3  Frequency partitioning

[Line 23]

The mapping of subband PRUs and miniband PRUs to the frequency partition is given by Equation (181):

(181)

where

and

.

Figure 401 depicts the frequency partitioning BW=5 MHz, SAC=3, FPCT=2, FPS=12, and FPSC=1.

Figure 409 – Frequency partitioning

---------------------------------------------------  End of Text  ----------------------------------------------------