| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Amendment Text for Primary Feedback Channel |  |
| Date Submitted | 2009-03-03 |  |
| Source(s) | Zheng Yan-Xiu, Yu-Chuan Fang, Chang-Lan Tsai, Chung-Lien Ho, Hsi-Min Hsiao, Ren-Jr Chen ITRI | zhengyanxiu@itri.org.tw |
| Re: | IEEE 802.16m-09/0012, “Call for Comments on Amendment Working Document”. |  |
| Abstract | This contribution proposes the design of primary feedback channel to the UL Ctrl DG text. |  |
| Purpose | For member’s review and adoption into P802.16m Amendment Working Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Amendment Text for Primary Feedback Channel

Zheng Yan-Xiu, Yu-Chuan Fang, Chang-Lan Tsai, Chung-Lien Ho, Hsi-Min Hsiao, Ren-Jr Chen

ITRI

- Background

This contribution provides primary feedback channel design for uplink control channel.

- Proposed Text Input to P802.16m Amendment Working Document

[Add the texts marked in blue and figures in the IEEE 802.16m-09/0010]

[-------------------AWD Text Proposal #5 of UL Ctrl DG  --------------------]

# Uplink Control Channels Physical Resource Mapping

# Fast Feedback Control Channel

There are two types of UL fast feedback control channels: primary fast feedback channel (PFBCH) and secondary fast feedback channels (SFBCH).

# Primary Fast Feedback Control Channel

The primary fast feedback channels are comprised of 3 distributed FMTs. Figure UL-3 illustrates the mapping of the PFBCH.

Figure UL- , PFBCH comprised of three distributed 2x6 UL FMTs.

Figure UL- , Mapping of information in the PFBCH.

The process of composing the PFBCH is illustrated in Figure UL- . The l PFBCH payload bits are used to generate PFBCH sequence according to Table UL- . The resulting bit sequence is modulated [, repeated] and mapped to uplink PFBCH symbol s[k]. The mapping of primary fast feedback channel symbol s[k] to the UL FMTs is given by Eqn. UL- . This set of sequence can carrier up to 6 information bits.

Ci,j=s[k],fori=0,1,2,0≤j≤,11,and0≤k≤,11, Eqn. UL-

Ci,j=s[ Ki[j] ],fori=0,1,2,0≤j≤11, 		Eqn. UL-6

Where

Ki[j] denotes the jth element of Ki

K0=0,1,2,3,4,5,6,7,8,9,10,11

K1=9,10,11,3,4,5,0,1,2,6,7,8

K2=3,4,5,6,7,8,9,10,11,0,1,2

Table UL- , Sequences for PFBCH.

| Index | Sequence | Index | Sequence |
| --- | --- | --- | --- |
| 0 | 0 0 0 0 0 0 1 1 1 1 1 0 | 32 | 0 1 0 0 0 0 0 1 0 1 0 0 |
| 1 | 0 0 0 0 0 1 0 0 0 1 0 0 | 33 | 0 1 0 0 0 0 1 0 1 1 1 1 |
| 2 | 0 0 0 0 0 1 1 0 1 0 0 0 | 34 | 0 1 0 0 0 1 0 1 1 0 1 1 |
| 3 | 0 0 0 0 1 0 0 1 1 0 0 0 | 35 | 0 1 0 0 0 1 1 1 0 0 1 0 |
| 4 | 0 0 0 0 1 1 0 0 0 0 0 1 | 36 | 0 1 0 0 1 0 0 1 0 0 1 1 |
| 5 | 0 0 0 0 1 1 0 1 0 1 1 0 | 37 | 0 1 0 0 1 0 1 1 1 1 0 1 |
| 6 | 0 0 0 1 0 0 0 1 0 0 0 1 | 38 | 0 1 0 0 1 1 0 0 1 1 1 0 |
| 7 | 0 0 0 1 0 0 1 0 1 0 1 0 | 39 | 0 1 0 0 1 1 1 0 1 0 1 1 |
| 8 | 0 0 0 1 1 0 0 0 0 0 1 0 | 40 | 0 1 0 1 0 0 0 0 1 0 0 0 |
| 9 | 0 0 0 1 1 0 0 1 1 1 1 1 | 41 | 0 1 0 1 0 0 1 1 0 1 1 1 |
| 10 | 0 0 0 1 1 0 1 1 0 1 0 0 | 42 | 0 1 0 1 0 1 0 0 0 1 1 0 |
| 11 | 0 0 0 1 1 1 0 0 1 0 0 0 | 43 | 0 1 0 1 0 1 1 0 0 0 0 1 |
| 12 | 0 0 0 1 1 1 1 0 0 0 1 1 | 44 | 0 1 0 1 0 1 1 1 1 1 0 0 |
| 13 | 0 0 0 1 1 1 1 0 1 1 0 1 | 45 | 0 1 0 1 1 0 0 0 0 1 0 1 |
| 14 | 0 0 0 1 1 1 1 1 1 0 1 0 | 46 | 0 1 0 1 1 1 1 1 1 0 0 1 |
| 15 | 0 0 1 0 0 0 1 0 1 1 0 1 | 47 | 0 1 1 0 0 0 0 0 1 0 1 0 |
| 16 | 0 0 1 0 0 1 0 1 1 0 0 1 | 48 | 0 1 1 0 0 0 1 0 0 0 1 1 |
| 17 | 0 0 1 0 0 1 1 0 0 1 1 0 | 49 | 0 1 1 0 0 1 0 1 0 0 0 0 |
| 18 | 0 0 1 0 0 1 1 1 0 0 1 1 | 50 | 0 1 1 0 1 0 0 0 0 1 0 0 |
| 19 | 0 0 1 0 1 0 0 1 0 1 0 1 | 51 | 0 1 1 0 1 0 0 0 1 0 0 1 |
| 20 | 0 0 1 0 1 0 1 0 1 0 1 0 | 52 | 0 1 1 0 1 0 1 1 0 0 0 0 |
| 21 | 0 0 1 0 1 1 0 0 1 1 1 1 | 53 | 0 1 1 0 1 1 1 0 0 1 0 1 |
| 22 | 0 0 1 0 1 1 1 1 1 1 0 0 | 54 | 0 1 1 0 1 1 1 1 0 1 1 0 |
| 23 | 0 0 1 1 0 0 0 0 1 1 1 1 | 55 | 0 1 1 1 0 0 1 0 0 1 0 0 |
| 24 | 0 0 1 1 0 0 0 1 0 0 1 0 | 56 | 0 1 1 1 0 1 0 0 1 1 0 1 |
| 25 | 0 0 1 1 0 0 1 1 1 0 0 0 | 57 | 0 1 1 1 0 1 0 1 0 0 1 1 |
| 26 | 0 0 1 1 0 1 0 0 0 0 0 1 | 58 | 0 1 1 1 1 0 0 1 1 0 1 0 |
| 27 | 0 0 1 1 0 1 0 1 1 1 0 0 | 59 | 0 1 1 1 1 0 1 0 1 1 1 0 |
| 28 | 0 0 1 1 0 1 1 1 1 1 1 1 | 60 | 0 1 1 1 1 1 0 1 0 1 0 0 |
| 29 | 0 0 1 1 1 0 1 0 0 0 0 1 | 61 | 0 1 1 1 1 1 0 1 1 1 1 1 |
| 30 | 0 0 1 1 1 0 1 1 0 1 1 1 | 62 | 0 1 1 1 1 1 1 0 0 0 1 0 |
| 31 | 0 0 1 1 1 1 0 0 0 1 1 0 | 63 | 0 0 0 0 0 0 0 0 0 1 1 1 |