| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications to the text of “Downlink Physical Structure” section of the IEEE 802.16m AWD to support multiplexing of EMBS & Unicast traffic |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Sudhir Ramakrishna Kaushik Josiam Taeyoung Kim Zhouyue Pi Samsung Electronics | sudhir.r@samsung.com |
| Re: | “802.16m AWD”. |  |
| Abstract | This contribution proposes modifications to the text in Section 15.3.5 (“Downlink Physical Structure”) of the IEEE Amendment Working Document (IEEE 802.16m-09/0010) to support the multiplexing of EMBS and Unicast traffic. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications to the text of “Downlink Physical Structure” Section of the IEEE 802.16m AWD to support multiplexing of EMBS & Unicast traffic

Sudhir Ramakrishna, Kaushik Josiam, Taeyoung Kim & Zhouyue Pi

Samsung Electronics

- Introduction

In contribution [1], IEEE 802.16m-09/0010, “IEEE 802.16m Amendment Working Document”, in addition to other sections, text is proposed for Section 15.3.5, “Downlink Physical Structure”.

In order to allow support for the multiplexing of broadcast and unicast traffic in the downlink frame, this contribution

- Proposes (in Section 2) the addition of a new Section 15.3.5.2.4, “Resource Reservation for EMBS Traffic”.

- Proposes (in Section 3) the addition of a new Section 15.3.5.2.5, “Sub-carrier Permutation for EMBS DRUs”

- Proposes (in Section 4) modifications to the text in Section 15.3.5.3.1 (“CRU/DRU Allocation”).

In this contribution, all text that is proposed to be added to the next version of [1] is in blue. All text that is present in the current version of [1] and which is proposed to be replaced is in red strikethrough.

- Addition of a New Section 15.3.5.2.4: “Resource Reservation for EMBS Traffic”

The following text is proposed to be added to a new section 15.3.5.2.4.

------- Begin text of proposed new section 15.3.5.2.4: Resource Reservation for EMBS Traffic -------

15.3.5.2.4 Resource Reservation for EMBS Traffic

In a carrier that is not dedicated to EMBS traffic, it may be required to multiplex EMBS and unicast traffic.

The multiplexing of EMBS and unicast traffic obeys the following constraints.

- EMBS traffic is only carried on FP0, i.e., the partition with index i = 0.

- Out of the sub-frames allocated to the down-link in a frame, EMBS traffic may be carried on any number of these down-link sub-frames.

- In the sub-frame(s) where EMBS traffic is carried, the EMBS traffic may be FDMed with the unicast traffic, i.e., a set of sub-carriers in each of the OFDM symbols in the sub-frame(s) may be reserved for EMBS traffic.

The following three (3) EMBS-related indicators are carried in the SFH (TBD). They are valid for the set of super-frames corresponding to the SFH duration.

  - A 1-bit EMBS_EXISTS indicator.

    - EMBS_EXISTS = 0 => no EMBS traffic is carried in any of the frames in this super-frame set.

    - EMBS_EXISTS = 1 => EMBS traffic is carried in all the frames in this super-frame set, as further specified by the EMBS_SF_COUNT in item (b) below.

  - A 3-bit EMBS_SF_COUNT (= EMBS sub-frame count). This points to the sub-frames carrying EMBS traffic in each frame of this super-frame set. The value of EMBS_SF_COUNT is relevant only when EMBS_EXISTS = 1. Given a value of EMBS_SF_COUNT, the indices of the sub-frames within the frame where EMBS traffic is present is indicated via a pre-determined look-up table (TBD).

An example of such a table is as below.

| EMBS_SF_COUNT | Indices (from 0 to 7) of sub-frames (in each frame in super-frame set) that will carry EMBS traffic |
| --- | --- |
| 000 | {0} |
| 001 | {0, 1} |
| 010 | {0, 1, 2} |
| 011 | {0, 1, 2, 3} |
| 100 | {0, 1, 2, 3, 4} |
| 101 | {0, 1, 2, 3, 4, 5} |
| 110 | {0, 1, 2, 3, 4, 5, 6} |
| 111 | {0, 1, 2, 3, 4, 5, 6, 7} |

  - A 5-bit (TBD) EMBS_RU_COUNT, which indicates, for FP0, the number of PRUs allocated to EMBS traffic, in units of N1 PRUs. This value is relevant only when EMBS_EXISTS = 1; otherwise, EMBS_RU_COUNT is set to 0.

    - Hence, the number of PRUs allocated to EMBS traffic for FP0, denoted as LEMBS, is given by LEMBS = N1*EMBS_RU_COUNT.

In the following, the term “DRUEMBS” refers to a logical distributed resource unit that is used to serve EMBS traffic, as opposed to a “DRU” which is used to serve unicast traffic. This is purely a distinction in terminology; a “DRUEMBS” has the same size as a “DRU”. This distinction in terminology is made purely to avoid confusion with respect to the operations on DRUs defined in this and later sections.

The mapping of the  (as defined in Equation(181)) to the DRUEMBS is described by the following Equation(181a):

Hence, the lastof theas described in Equation(181) are reserved for EMBS traffic.

------ End text of proposed new section 15.3.5.2.4: Resource Reservation for EMBS Traffic -------

- Addition of New Section 15.3.5.2.5: “Sub-carrier Permutation for EMBS DRUs”

The following text is proposed to be added to a new section 15.3.5.2.5.

----- Begin text of proposed new section 15.3.5.2.5: Sub-carrier Permutation for EMBS DRUs -----

15.3.5.2.5  Sub-carrier Permutation for EMBS DRUs

The sub-carrier permutation for the DL DRUEMBS reserved for the EMBS traffic in FP0 (as defined in Equation(181a)) spreads the subcarriers in the DRUs across the entire EMBS allocation. The granularity of the sub-carrier permutation is equal to a pair of sub-carriers. In order to support SFN (Single Frequency Network) operation, all the sectors in an EMBS zone employ the same permutation rule.

After mapping all the pilots (EMBS demodulation pilots), the remainder of the sub-carriers reserved for EMBS traffic are used to define the distributed LRU for EMBS traffic.

The permutation details are TBD. The permutation formula could be the same as that used for unicast traffic DRUs (as defined in Section 15.3.5.3.3), with the dependency on cell_id removed.

------ End text of proposed new section 15.3.5.2.5: Sub-carrier Permutation for EMBS DRUs -------

- Modifications to text in Section 15.3.5.3.1 “CRU/DRU Allocation”

The text in Section 15.3.5.3.1 (“CRU/DRU Allocation”) is proposed to be modified as follows.

- Line 15 on Page 30 is proposed to be replaced as follows.

where

Replace by

where

- The range for the s[] sequence on Line 27 on Page 30 is proposed to be replaced as follows.

Replace by

- The range of the sc[] sequence is proposed to be defined by adding the following sentence after the sentence on line 37.

The range of the sc[] sequence is the same as the range of the s[] sequence.

- References

[1] IEEE 802.16-09/0010, “IEEE 802.16m Amendment Working Document”, January 29, 2009.