| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | QoS and Connection management in 802.16m |  |
| Date Submitted | 2009-03-10 |  |
| Source(s) | Jie Hui Muthaiah Venkatachalam Xiangying Yang Aran Bergman Jing Zhu Rath Vannithamby Intel Corporation Jeongki Kim Kiseon Ryu Ronny Yongho Kim Jin Sam Kwak LG Electronics | jie.hui@intel.com muthaiah.venkatachalam@intel.com xiangying.yang@intel.com aran.bergman@intel.com chegal@lge.com |
| Re: |  |  |
| Abstract | This proposal describes stage-3 text for QoS and Connection Management Enhancements for the IEEE 802.16m Amendment. |  |
| Purpose | For discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

QoS and Connection Management Enhancements

for the IEEE 802.16m Amendment

Jie Hui, Muthaiah Venkatachalam, Xiangying Yang, Aran Bergman, Jing Zhu, Rath Vannithamby

Intel Corporation

Jeongki Kim, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics

- Introduction

The contribution proposes the text of sections dealing with QoS and connection management enhancements, to be included in the 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------------------------  Text Start  -------------------------------------------------

15.2 Medium access control

15.2.x QoS

The QoS-related concepts such as dynamic service flow management and two-phase activation model follow the procedure defined in section 6.3.14. In addition, complying to 16m SDD [3], a new schedule service - adaptive granting and polling service (aGPS) - is defined in 15.2.x.y.1 to unify the UGS/ertPS/rtPS/nrtPS scheduling services and provide more flexible QoS support.

15.2.x.y Scheduling services

Scheduling services represent the data handling mechanisms supported by the MAC scheduler for data transport on a connection. Each connection is associated with a single scheduling service. A scheduling service is determined by a set of QoS parameters that quantify aspects of its behavior. These parameters are managed using the DSA and DSC message dialogs. A detailed description of each QoS parameter is provided in 11.13.

IEEE 802.16m supports adaptation of service flow QoS parameters. When QoS requirement/traffic characteristics for UL traffic changes, the ABS may autonomously switch the service flow QoS parameters such as grant/polling interval or grant size based on predefined rules. In addition, the AMS may request the ABS to switch the Service Flow QoS parameter set with explicit signaling. The ABS then allocates resource according to the new service flow parameter set.

Well-known scheduling services can be implemented by specifying a specific set of QoS parameters.

Table xxx describes the QoS parameters that would provide Adaptive grant and polling scheduling service (aGPS).

Table xxx—Example of QoS parameters providing aGPS scheduling service

| Information Element | Notes |
| --- | --- |
| Maximum Latency (unsigned int) |  |
| Minimum Reserved Traffic Rate (unsigned int) |  |
| Maximum Sustained Traffic Rate (unsigned int) |  |
| Traffic Priority (unsigned int) |  |
| Request/Transmission Policy (unsigned int) |  |
| Tolerated packet loss rate (unsigned int) |  |
| If (uplink service flow) { (Boolean ) |  |
| Scheduling Type (unsigned int) | aGPS |
| GPImin (unsigned int) | Minimum GPI, used initially |
| GPImax (unsigned int) | Maximum GPI |
| Adaptation Grant Size (unsigned int) | Grant size to be used during GPI adaptation. If grant size=x bytes (the newly defined BW-REQ header size), it is adaptive polling; otherwise, it is adaptive granting. When it is adaptive granting and the adaptation method is implicit, this adaptation grant size indicates the unsolicited grant size to be used by ABS during GPI adaptation. |
| Adaptation Start Time (AST) (unsigned int) | If the adaptation method is “implicit”, upon the AST-th zero sized BW-REQ, polling/grant adaptation from GPImin to GPImax begins. |
| Adaptation Method (unsigned int) | - Implicit, exponential: upon the AST-th zero sized BW-REQ, GPI begins increasing exponentially from GPImin to GPImax; upon non-zero sized BW-REQ or upon the quick access messages for aGPS (details TBD), GPI resumes to GPImin - Implicit, one-step: upon the AST-th zero sized BW-REQ, GPI begins increasing one-step from GPImin to GPImax; upon non-zero sized BW-REQ or upon the quick access messages for aGPS (details TBD), GPI resumes to GPImin - Explicit: new GPI and grant size to use is going to be indicated in BW-REQ |
| } |  |

15.2.x.y.1 Adaptive granting and polling service (aGPS)

The ABS may grant or poll AMS periodically with adaptation. Initially, ABS uses grant or polling interval (GPI) as indicated by GPImin. If the Adaptation Method is implicit, upon the AST-th zero sized BW-REQ from AMS, GPI adaptation starts and ABS increases GPI from GPImin to GPImax either exponentially or one-step; and upon non-zero sized BW-REQ or upon the quick access messages for aGPS (details TBD), BS resumes GPI back to GPImin. If the Adaptation Method is explicit, the GPI adaptation can be triggered by AMS explicitly with the new GPI indicated through field of Running Grant/Polling Interval (see Table yyy) in BW-REQ headers.

If the Grant Size is x bytes (the newly defined BW-REQ header size), it is adaptive polling; otherwise, it is adaptive granting. When it is adaptive granting, the initial size of allocations corresponds to Maximum Sustained Traffic Rate. And the new grant size during GPI adaptation is either indicated by Adaptation Grant Size if the Adaptation Method is implicit or requested by AMS in BW-REQ headers if Adaptation Method is explicit.

Table yyy—6 bits Running GPI field in BW-REQ to support aGPS

| Filed | Bits | Unit | Meaning |
| --- | --- | --- | --- |
| Running Grant/Polling Interval | 6 bits | frames | If it is zero, grant/polling interval has no change If it is non-zero If (scheduling type == aGPS) new GPI to use for future allocation If (scheduling type == BE) minimum grant interval for the next allocation |

15.2.x.z Service Flow MAC Management Messages

AAIF service flow MAC management messages, including DSA/DSC/DSD follow the similar messaging format as specified in 6.3.2.3.10 to 6.3.2.3.17. The primary management connection identifier in the AAIF service flow MAC management message shall be changed from CID to FID.

15.2.x.z.1 FID

The value of this field specifies the FID assigned by the ABS to a service flow of an AMS with a non-null AdmittedQosParamSet or ActiveQosParamSet. The 4-bit value of this field is used in BRs and in MAC PDU headersGMH. This field shall be present in a ABS-initiated DSA-REQ or DSC-REQ message related to establishing an admitted or active service flow. This field shall also be present in DSA-RSP and DSC-RSP messages related to the successful establishment of an admitted or active service flow.

If a service flow has been successfully admitted or activated (i.e., has an assigned FID) the SFID shall NOT be used for subsequent DSx message signaling as FID is the primary handle for a service flow. If a service flow is no longer admitted or active (via DSC-REQ), its FID may be reassigned by the BS.

-------------------------------------------------  Text end  -------------------------------------------------