| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Femtocell Overview |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Joey Chou Muthaiah Venkatachalam Intel | E-mail: joey.chou@intel.com E-mail: Muthaiah.Venkatachalam@intel.com |
| Re: | TGm SDD: Femto |  |
| Abstract | This contribution proposes text for Femtocell Overview. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Femtocell Overview

Joey Chou, Muthaiah Venkatachalam

Intel

- Introduction

This contribution proposes text to provide overview for Femtocell, based on the following reasons:

- A high level Femtocell overview is needed to set the context of section 17.

- Section 17.1 Types of Base Stations includes a couple paragraphs that don’t fit into the scope of section 17.1, and don’t capture the whole picture of Femtocell.

- Proposed text

17   Support for Femtocell

Femtocell BS is a low power BS that is designed for use at home or SOHO in order to lower the operational cost for operators, and to extend service coverage to areas, where services would otherwise be limited or unavailable due to great penetration loss. Femtocell BS typically operates in licensed spectrum, and is connected to the service provider’s network via standard broadband, such as DSL or cable.

Femtocell BS should be low cost, and support plug and play model where a subscriber will be able to purchase Femtocell BS from the local electronic store, and then simply plug it into the broadband

modem to receive services without any operator intervention.

Figure 1: Femtocell Network Architecture

Figure 1 shows the Femtocell Network Architecture. Broadband access technologies, such as DSL and cable, are used to backhaul Femtocell BS traffic through pubic Internet to WiMAX networks via Femtocell Gateway. SON Server is a server application in NMS (Network Management System) that interface with OAM Server to provide SON functions to SS, MS, FAP, and macro BS.

17.1	Types of Base stations

A Femtocell BS is a BS with low transmit power, typically installed by a subscriber in home or SOHO to provide the access to closed or open group of users as configured by the subscriber and/or the access provider. A Femtocell BS is connected to the service provider’s network via broadband (such as DSL, or cable).

Femtocell BSs typically operate in licensed spectrum and may use the same or different frequency as macro-cells. Their coverage may overlap with macro BS.