| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Femtocell Overview |  |
| Date Submitted | 2009-03-10 |  |
| Source(s) | Joey Chou Muthaiah Venkatachalam Xiangying Yang Sassan Ahmadi Intel | E-mail: joey.chou@intel.com E-mail: Muthaiah.Venkatachalam@intel.com E-mail: Xiangying.yang@intel.com |
| Re: | TGm SDD: Femto |  |
| Abstract | This contribution proposes text for Femtocell Overview. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Femtocell Overview

Joey Chou, Muthaiah Venkatachalam, Xiangying Yang, Sassan Ahmadi

Intel

- Introduction

This contribution proposes text to provide the overview of Femtocell in response to the request for information from ITU WP5D.

- Proposed text

17   Support for Femtocell

  - Femtocell Overview

Femtocell BS is a low power BS that is designed for use at home or SOHO in order to lower the operational cost for operators, and to extend service coverage to areas, where services would otherwise be limited or unavailable due to great penetration loss. Femtocell BS typically operates in licensed spectrum, and is connected to the service provider’s core network via standard broadband connection.

Figure 1: Femtocell Network Architecture

Figure 1 shows the Femtocell Network Architecture. Broadband access technologies, such as Fiber optics or cable modem are used to backhaul Femtocell BS traffic, through pubic Internet, to WiMAX core network (or any other core network) via the Femtocell Gateway. SON Server is a server application in NMS (Network Management System) that interface with OAM Server to provide SON functions to SS, MS, FAP, and overlay BS. Femtocell BS will use the same air interface as overlay BS to interface to MS or SS.

The following are high level vision statements for the operation of the Femtocell BS:

Range, capacity and subscriber groups:

The Femtocell BS is expected to be a low cost and readily available BS, possibly with a well defined (aka closed) subscriber group. Femtocell BS’s can be obtained from retail distribution. It should essentially look like an overlay BS with the capacity and range scaled down compared to an overlay BS. The Femtocell BS is expected to have a range compared to that of a Wi-Fi access point.

Airlink design of the femtocell:

The airlink of the Femtocell BS is not different from that of an overlay BS and the same air-interface protocols are applied.

Home Femto cell discovery by the MS:

The Femtocell BS may have MS’s that have been pre-authorized to connect to a given femto cell. The Femtocell BS supports mechanisms to enable quick discovery of the Femtocell BS by its subscribed MS’s.

Frequency Assignment of the femtocell:

The Femtocell operates in the same FA (or the different FA) relative to the overlay BS’s.

Synchronization with overlay BS:

The Femtocell is synchronized with the overlay BS. However, the synchronization cannot always be guaranteed due to the practical complexity.

Authentication, Authorization and security aspects:

Since the backhaul is going through the public Internet, Femtocell BS will need to be authenticated and authorized before being connected to the WiMAX core networks. A MS accessing the Femtocell BS has to be authorized by the service provider, since the MS traffic is always routed to service provider’s core network. The communication between the Femtocell BS and the core networks will be authenticated, integrity protected, and encrypted.

Handover and session continuity

The Femtocell BS should supports handover mechanisms to and from the overlay BS’s. There is session continuity for an MS when it is handed over to and from the Femtocell BS.

Retail model and Self Optimization:

Femtocell BS will support SON (Self-Organizing Networks) to enable plug and play model where a subscriber will be able to purchase Femtocell BS via retail channels, and then simply plug it into the broadband modem to receive services without any operator intervention. In the startup process, Femtocell BS will discover the wireless environment, and automatically configure the RF, PHY, and MAC parameters that should cause minimum interference to neighboring cells. The Femtocell BS will continue monitoring the wireless environment, and reconfiguring system parameters as needed in order to achieve optimal network performance of throughput, coverage, and capacity.

Interference management:

Interference avoidance and interference mitigation techniques can be used in the Femtocell BS. Femtocell BS is synchronized with the overlay BS both in timing and frequency, in order to minimize the interference. Overlay BS and Femtocell BS may use radio resource reservation (e.g., FDM or TDM) to avoid collision that will mitigate the interference between neighboring cells. Femtocell BS can go to the idle mode, when there is not activity, in order to save power and minimize the interference by turning off the Tx power. DL and UL power control will be supported by the Femtocell BS in order to reduce the interference to overlay BS and neighbor Femtocell BS.

Operational modes:

Femtocell BS can operate in 3 modes. First, Femtocell BS can operate like a public hot-spot that can be accessed by any MS that is authorized by the service provider. Second, as Femtocell BS can be owned by home or SOHO subscribers, its owner will grant access exclusively to a group of users, called CSG (Close Subscriber Group), at their will. Third, Femtocell BS can serve an MS who is not in CSG, if its resources are available, and such MS has no access to other BS. The handover process of a MS between Femtocell BS and overlay BS will follow the standard handover procedure. However, the handover from an overlay BS to a Femtocell BS will be subject to mode restriction, as described above.