| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Channel Coding and HARQ for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Jerry Pi Samsung Electronics Co., Ltd. The Channel Coding and HARQ Drafting Group Editor | +1-972-761-7944 zpi@sta.samsung.com |
| Re: | 802.16m AWD |  |
| Abstract | The contribution proposes the text of channel coding and HARQ section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Channel Coding and HARQ 
for the IEEE 802.16m Amendment

Jerry Pi

Samsung Electronics Co., Ltd.

The Channel Coding and HARQ Drafting Group Editor

- Introduction

The contribution proposes the text of channel coding and HARQ section to be included in the 802.16m amendment ([1]). The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 ([2]), it is compliant to the 802.16m SRD ([3]) and the 802.16m SDD ([4]), and it follows the style and format guidelines in ([5]).

- References

[1] IEEE 802.16m-09/0010, “802.16m Amendment Working Document (AWD)”

[2] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[3] IEEE 802.16m-07/002r7, “IEEE 802.16m System Requirements Document”

[4] IEEE 802.16m-08/003r6, “IEEE 802.16m System Description Document”

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

- Abbreviations and acronyms

Insert the following at section 4 in alphabetic order:

CoRe		constellation re-arrangement

HARQ		hybrid-ARQ

IR			incremental redundancy

MCS			Modulation and Coding Scheme

SPID			subpacket ID

Insert the following subsection at a new section 15:

  - Channel coding and HARQ

    - Channel coding

Channel coding procedures for downlink and uplink data channel are shown in Figure 15.x.1-1.

Figure 15.x.1-1. Channel coding procedure

      - Burst CRC encoding

Cyclic Redundancy Code (CRC) bits are used to detect errors in the received packets. A 16-bit burst CRC is appended to the data burst with the following cyclic generator polynomial:

gDB-CRC(D)=D16+D12+D5+1

Denote the bits of the input data burst by d1, d2 ,d3, ⋯, dNPLwith d1 being the MSB and NPL being the size of the input data burst. Denote the parity bits produced by the burst CRC generator by p1, p2 ,p3, ⋯, p16. The burst CRC encoding is performed in a systematic form, which means that in GF(2), the polynomial:

d1DNPL+15+ d2DNPL+14+ ⋯ +dnD16+p1D15+p2D14+⋯+p15D1+p16

yields a remainder equal to 0 when divided by gDB-CRC(D).

The data burst, including the CRC, is further processed by the data randomizer as described in section 15.x.1.2.

      - Randomization

Data randomization shall be performed on the downlink and uplink data channel.

The randomization bits are generated using a PRBS generator as shown in Figure 15.x.1.1-1. The generator polynomial of the PRBS generator is 1+X14+X15. For each data burst, the beginning state of the PRBS is initialized to [s1 s2 … s15] = [0 1 1 0 1 1 1 0 0 0 1 0 1 0 1] with s1 being the LSB and s15 being the MSB. The data burst to be transmitted shall enter sequentially into the randomizer, MSB first. The data bits are XOR-ed with the output of the PRBS generator, with the MSB of the data burst XOR-ed with the first bit of the PRBS generator output.

Figure 15.x.1.1-1.The data randomizer with a PRBS generator

The output of the data randomizer is further processed by burst partition as described in 15.x.1.3.

      - Burst partition

When the burst size including 16 burst CRC bits exceeds maximum FEC block size, NFB_MAX, the burst is partitioned into KFB FEC blocks. The value of KFB is calculated by following equation.

KFB=Nmod⋅RFEC⋅NRE⋅NSMNFB_MAX

where

Nmod 	is the modulation order of the burst transmission, which is specified by MCS index

RFEC 	is the nominal code rate of the burst transmission, which is specified by MCS index

NRE 		is the number of resource elements allocated for burst transmission. The calculation of NRE shall exclude the resource elements used by pilot channels

NSM		is the spatial multiplexing order of the resource elements allocated for burst transmission. The value of NSM depends on the MIMO transmission mode and is defined in section x.x.x.x.

NFB_MAX	is the maximum FEC block size, which equals to 4800 bits.

The nominal MCS used in a data transmission shall be selected from Table 15.x.1-1.

Table 15.x.1-1. MCS table for downlink and uplink data channel

| MCS index | Modulation | Code rate |
| --- | --- | --- |
| ‘0000’ | QPSK | 31/256 |
| ‘0001’ | QPSK | 48/256 |
| ‘0010’ | QPSK | 71/256 |
| ‘0011’ | QPSK | 101/256 |
| ‘0100’ | QPSK | 135/256 |
| ‘0101’ | QPSK | 171/256 |
| ‘0110’ | 16QAM | 102/256 |
| ‘0111’ | 16QAM | 128/256 |
| ‘1000’ | 16QAM | 155/256 |
| ‘1001’ | 16QAM | 184/256 |
| ‘1010’ | 64QAM | 135/256 |
| ‘1011’ | 64QAM | 157/256 |
| ‘1100’ | 64QAM | 181/256 |
| ‘1101’ | 64QAM | 205/256 |
| ‘1110’ | 64QAM | 225/256 |
| ‘1111’ | 64QAM | 237/256 |

The size of each FEC block is NFB, which is calculated by the following equation.

NFB=argmaxX∈NFBX≤(NmodRFECNRENSMKFB)

where {NFB} 	is a set of NFB shown in Table 15.x.1-2.

Table 15.x.1-2. FEC block size table for downlink and uplink data channels

| Index | NFB | Index | NFB | Index | NFB | Index | NFB | Index | NFB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 48 | 30 | 328 | 60 | 720 | 90 | 1424 | 120 | 2752 |
| 1 | 64 | 31 | 344 | 61 | 736 | 91 | 1448 | 121 | 2816 |
| 2 | 72 | 32 | 352 | 62 | 752 | 92 | 1480 | 122 | 2880 |
| 3 | 80 | 33 | 360 | 63 | 768 | 93 | 1504 | 123 | 2944 |
| 4 | 88 | 34 | 368 | 64 | 776 | 94 | 1536 | 124 | 3008 |
| 5 | 96 | 35 | 376 | 65 | 800 | 95 | 1560 | 125 | 3072 |
| 6 | 104 | 36 | 384 | 66 | 824 | 96 | 1600 | 126 | 3200 |
| 7 | 120 | 37 | 400 | 67 | 848 | 97 | 1640 | 127 | 3264 |
| 8 | 128 | 38 | 416 | 68 | 872 | 98 | 1672 | 128 | 3328 |
| 9 | 136 | 39 | 432 | 69 | 888 | 99 | 1712 | 129 | 3392 |
| 10 | 144 | 40 | 440 | 70 | 912 | 100 | 1752 | 130 | 3456 |
| 11 | 152 | 41 | 456 | 71 | 936 | 101 | 1784 | 131 | 3520 |
| 12 | 160 | 42 | 472 | 72 | 960 | 102 | 1824 | 132 | 3648 |
| 13 | 176 | 43 | 480 | 73 | 984 | 103 | 1864 | 133 | 3712 |
| 14 | 184 | 44 | 496 | 74 | 1000 | 104 | 1896 | 134 | 3776 |
| 15 | 192 | 45 | 512 | 75 | 1024 | 105 | 1920 | 135 | 3840 |
| 16 | 200 | 46 | 528 | 76 | 1048 | 106 | 1952 | 136 | 3904 |
| 17 | 208 | 47 | 544 | 77 | 1072 | 107 | 2000 | 137 | 3968 |
| 18 | 216 | 48 | 552 | 78 | 1096 | 108 | 2048 | 138 | 4096 |
| 19 | 232 | 49 | 568 | 79 | 1112 | 109 | 2096 | 139 | 4160 |
| 20 | 240 | 50 | 584 | 80 | 1136 | 110 | 2144 | 140 | 4224 |
| 21 | 248 | 51 | 600 | 81 | 1160 | 111 | 2192 | 141 | 4288 |
| 22 | 256 | 52 | 608 | 82 | 1184 | 112 | 2232 | 142 | 4352 |
| 23 | 264 | 53 | 624 | 83 | 1216 | 113 | 2280 | 143 | 4416 |
| 24 | 272 | 54 | 640 | 84 | 1248 | 114 | 2328 | 144 | 4544 |
| 25 | 288 | 55 | 656 | 85 | 1280 | 115 | 2368 | 145 | 4608 |
| 26 | 296 | 56 | 664 | 86 | 1312 | 116 | 2432 | 146 | 4672 |
| 27 | 304 | 57 | 680 | 87 | 1336 | 117 | 2496 | 147 | 4736 |
| 28 | 312 | 58 | 696 | 88 | 1368 | 118 | 2560 | 148 | 4800 |
| 29 | 320 | 59 | 712 | 89 | 1392 | 119 | 2624 |  |  |

The burst size NDB including burst CRC and FEC block CRC is defined by the following equation:

NDB = KFB × NFB

The payload size excluding burst CRC and FEC block CRC is defined by the following equation:

NPL = KFB × (NFB – IMFB×NFB-CRC) – NDB-CRC

where,

IMFB		equals 0 when KFB =1, 1 when KFB >1,
NFB-CRC	equals 16, which is the size of FEC block CRC,
NDB-CRC	equals 16, which is the size of burst CRC.

The burst partition block generates KFB FEC blocks, with each FEC block processed by the FEC block CRC encoding block as described in 15.x.1.4.

      - FEC block CRC encoding

The burst partition procedure generates KFB FEC blocks for each burst. If KFB > 1, the FEC block CRC generator appends a 16-bit FEC block CRC for each FEC block. The cyclic generator for FEC block CRC encoding is shown as follows:

gFB−CRC(D)=D16+D15+D2+1.

Denote the bits of the input FEC block by d1, d2 ,d3, ⋯, dNFB−16with d1 being the MSB and NFB being the size of the input FEC block, including the 16-bit FEC block CRC. Denote the parity bits produced by the burst CRC generator by p1, p2 ,p3, ⋯, p16. The burst CRC encoding is performed in a systematic form, which means that in GF(2), the polynomial:

d1DNFB−1+ d2DNFB−2+ ⋯ +dNFB−16D16+p1D15+p2D14+⋯+p15D1+p16

yields a remainder equal to 0 when divided by gDB-CRC(D).

      - FEC encoding

Each FEC block shall be encoded using the convolutional turbo codes specified in section 15.x.1.5.1.

        - Convolutional turbo codes (CTCs)

          - CTC encoder

The CTC encoder, including its constituent encoder, is depicted in Figure c.

It uses a double binary CRSC (Circular Recursive Systematic Convolutional) code.

The bits of the data to be encoded are alternatively fed to A and B, starting with the MSB of the first byte being fed to A, followed by the next bit being fed to B. The encoder is fed by blocks of NEP bits (NEP=2Nbits).

The polynomials defining the connections are described in octal and symbol notations as follows:

- For the feedback branch: 15, equivalently 1+D+D3 (in symbolic notation)
- For the Y parity bit: 13, equivalently 1+D2+D3
- For the W parity bit: 11, equivalently 1+D3

Figure ccc ― CTC encoder

First, the encoder (after initialization by the circulation state SC2, see 15.13.1.6.1.3Error: Reference source not found) is fed the sequence in the natural order (switch 1 in Figure ) with incremental address i=1,2,⋯,N. This first encoding is called C1 encoding.

Then the encoder (after initialization by the circulation state SC2, see 15.13.1.6.1.3) is fed by the interleaved sequence (switch 2 in Figure ) with incremental address i=1,2,⋯,N. This second encoding is called C2 encoding.

The order in which the encoded bits shall be fed into the bit separation block (15.x.1.6.1.4) is:

          - CTC interleaver

The CTC interleaver requires the parameters P0, P1, P2, and P3 shown in Table ddd of which NEP set corresponds to that in Table ccc.

The detailed interleaver structures except table for interleaver parameters correspond to 8.4.9.2.3.2.

| Index | NEP | P0 | P1 | P2 | P3 | Index | NEP | P0 | P1 | P2 | P3 | Index | NEP | P0 | P1 | P2 | P3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 48 | 5 | 0 | 0 | 0 | 50 | 584 | 21 | 74 | 20 | 214 | 100 | 1752 | 31 | 314 | 656 | 666 |
| 1 | 64 | 11 | 12 | 0 | 12 | 51 | 600 | 31 | 12 | 272 | 28 | 101 | 1784 | 33 | 886 | 888 | 518 |
| 2 | 72 | 11 | 18 | 0 | 18 | 52 | 608 | 23 | 288 | 244 | 140 | 102 | 1824 | 41 | 774 | 548 | 898 |
| 3 | 80 | 7 | 4 | 32 | 36 | 53 | 624 | 23 | 286 | 220 | 70 | 103 | 1864 | 33 | 504 | 444 | 664 |
| 4 | 88 | 13 | 36 | 36 | 32 | 54 | 640 | 23 | 84 | 296 | 236 | 104 | 1896 | 35 | 936 | 940 | 832 |
| 5 | 96 | 13 | 24 | 0 | 24 | 55 | 656 | 23 | 24 | 300 | 52 | 105 | 1920 | 43 | 318 | 556 | 778 |
| 6 | 104 | 7 | 4 | 8 | 48 | 56 | 664 | 23 | 272 | 220 | 60 | 106 | 1952 | 35 | 94 | 144 | 686 |
| 7 | 120 | 11 | 30 | 0 | 34 | 57 | 680 | 19 | 48 | 240 | 144 | 107 | 2000 | 37 | 290 | 692 | 638 |
| 8 | 128 | 13 | 46 | 44 | 30 | 58 | 696 | 31 | 252 | 216 | 48 | 108 | 2048 | 31 | 2 | 332 | 622 |
| 9 | 136 | 13 | 58 | 4 | 58 | 59 | 712 | 25 | 214 | 180 | 286 | 109 | 2096 | 39 | 400 | 688 | 68 |
| 10 | 144 | 11 | 6 | 0 | 6 | 60 | 720 | 23 | 130 | 156 | 238 | 110 | 2144 | 29 | 298 | 252 | 610 |
| 11 | 152 | 11 | 38 | 12 | 74 | 61 | 736 | 29 | 126 | 208 | 270 | 111 | 2192 | 39 | 1074 | 148 | 710 |
| 12 | 160 | 13 | 68 | 76 | 64 | 62 | 752 | 23 | 26 | 24 | 230 | 112 | 2232 | 29 | 240 | 496 | 1100 |
| 13 | 176 | 17 | 52 | 68 | 32 | 63 | 768 | 29 | 252 | 0 | 88 | 113 | 2280 | 41 | 474 | 376 | 814 |
| 14 | 184 | 13 | 2 | 0 | 2 | 64 | 776 | 29 | 100 | 196 | 140 | 114 | 2328 | 41 | 254 | 884 | 1054 |
| 15 | 192 | 7 | 58 | 48 | 10 | 65 | 800 | 23 | 150 | 216 | 150 | 115 | 2368 | 47 | 228 | 440 | 724 |
| 16 | 200 | 11 | 76 | 0 | 24 | 66 | 824 | 29 | 130 | 332 | 42 | 116 | 2432 | 43 | 452 | 888 | 96 |
| 17 | 208 | 11 | 10 | 32 | 42 | 67 | 848 | 29 | 234 | 388 | 82 | 117 | 2496 | 43 | 0 | 208 | 528 |
| 18 | 216 | 11 | 54 | 56 | 2 | 68 | 872 | 29 | 408 | 300 | 316 | 118 | 2560 | 53 | 264 | 488 | 824 |
| 19 | 232 | 11 | 70 | 60 | 58 | 69 | 888 | 25 | 414 | 84 | 414 | 119 | 2624 | 47 | 378 | 1092 | 1250 |
| 20 | 240 | 13 | 60 | 0 | 60 | 70 | 912 | 29 | 14 | 264 | 94 | 120 | 2752 | 37 | 430 | 880 | 970 |
| 21 | 248 | 13 | 6 | 84 | 46 | 71 | 936 | 25 | 272 | 168 | 400 | 121 | 2816 | 31 | 624 | 704 | 400 |
| 22 | 256 | 11 | 64 | 8 | 8 | 72 | 960 | 53 | 62 | 12 | 2 | 122 | 2880 | 43 | 720 | 360 | 540 |
| 23 | 264 | 13 | 72 | 68 | 8 | 73 | 984 | 31 | 142 | 40 | 342 | 123 | 2944 | 41 | 338 | 660 | 646 |
| 24 | 272 | 13 | 82 | 44 | 38 | 74 | 1000 | 29 | 290 | 148 | 446 | 124 | 3008 | 43 | 916 | 1136 | 912 |
| 25 | 288 | 17 | 74 | 72 | 2 | 75 | 1024 | 29 | 320 | 236 | 324 | 125 | 3072 | 53 | 184 | 824 | 1368 |
| 26 | 296 | 13 | 0 | 84 | 64 | 76 | 1048 | 27 | 424 | 212 | 416 | 126 | 3200 | 43 | 1382 | 632 | 1086 |
| 27 | 304 | 13 | 130 | 112 | 46 | 77 | 1072 | 35 | 290 | 228 | 390 | 127 | 3264 | 49 | 142 | 828 | 1354 |
| 28 | 312 | 11 | 32 | 124 | 108 | 78 | 1096 | 23 | 178 | 392 | 430 | 128 | 3328 | 37 | 258 | 28 | 1522 |
| 29 | 320 | 17 | 84 | 108 | 132 | 79 | 1112 | 33 | 38 | 244 | 550 | 129 | 3392 | 51 | 460 | 56 | 1608 |
| 30 | 328 | 17 | 148 | 160 | 76 | 80 | 1136 | 37 | 170 | 276 | 134 | 130 | 3456 | 43 | 170 | 920 | 1518 |
| 31 | 344 | 17 | 160 | 116 | 52 | 81 | 1160 | 31 | 314 | 348 | 222 | 131 | 3520 | 57 | 776 | 1232 | 1012 |
| 32 | 352 | 17 | 106 | 56 | 50 | 82 | 1184 | 31 | 2 | 568 | 94 | 132 | 3648 | 49 | 132 | 720 | 276 |
| 33 | 360 | 17 | 40 | 132 | 128 | 83 | 1216 | 31 | 368 | 584 | 524 | 133 | 3712 | 41 | 1328 | 772 | 1036 |
| 34 | 368 | 19 | 88 | 0 | 172 | 84 | 1248 | 31 | 88 | 404 | 608 | 134 | 3776 | 53 | 772 | 256 | 408 |
| 35 | 376 | 13 | 110 | 92 | 14 | 85 | 1280 | 29 | 152 | 8 | 24 | 135 | 3840 | 53 | 92 | 1124 | 476 |
| 36 | 384 | 11 | 96 | 48 | 144 | 86 | 1312 | 31 | 214 | 160 | 506 | 136 | 3904 | 51 | 664 | 200 | 64 |
| 37 | 400 | 19 | 142 | 0 | 142 | 87 | 1336 | 39 | 2 | 168 | 646 | 137 | 3968 | 57 | 1296 | 760 | 1360 |
| 38 | 416 | 17 | 102 | 132 | 178 | 88 | 1368 | 29 | 570 | 348 | 574 | 138 | 4096 | 55 | 148 | 808 | 308 |
| 39 | 432 | 17 | 126 | 92 | 74 | 89 | 1392 | 31 | 218 | 484 | 446 | 139 | 4160 | 79 | 214 | 308 | 262 |
| 40 | 440 | 19 | 48 | 20 | 144 | 90 | 1424 | 31 | 676 | 124 | 184 | 140 | 4224 | 59 | 14 | 668 | 1474 |
| 41 | 456 | 17 | 184 | 0 | 48 | 91 | 1448 | 33 | 254 | 372 | 158 | 141 | 4288 | 57 | 662 | 1516 | 42 |
| 42 | 472 | 19 | 40 | 104 | 28 | 92 | 1480 | 31 | 32 | 716 | 736 | 142 | 4352 | 59 | 2052 | 712 | 1804 |
| 43 | 480 | 13 | 120 | 60 | 180 | 93 | 1504 | 31 | 254 | 416 | 474 | 143 | 4416 | 59 | 1342 | 1968 | 1562 |
| 44 | 496 | 17 | 194 | 0 | 58 | 94 | 1536 | 31 | 34 | 564 | 710 | 144 | 4544 | 65 | 1380 | 1068 | 1036 |
| 45 | 512 | 19 | 64 | 52 | 124 | 95 | 1560 | 29 | 300 | 248 | 568 | 145 | 4608 | 67 | 954 | 1140 | 1566 |
| 46 | 528 | 17 | 36 | 196 | 100 | 96 | 1600 | 31 | 454 | 216 | 234 | 146 | 4672 | 67 | 410 | 1020 | 114 |
| 47 | 544 | 19 | 222 | 248 | 134 | 97 | 1640 | 33 | 164 | 432 | 748 | 147 | 4736 | 59 | 2 | 956 | 458 |
| 48 | 552 | 13 | 198 | 180 | 190 | 98 | 1672 | 35 | 164 | 368 | 700 | 148 | 4800 | 53 | 66 | 24 | 2 |
| 49 | 568 | 19 | 102 | 140 | 226 | 99 | 1712 | 41 | 4 | 848 | 332 |  |  |  |  |  |  |

Table ddd ― CTC Interleaver parameters

          - Determination of CTC circulation states

Correspond to 8.4.9.2.3.3.

          - Bit separation

Correspond to 8.4.9.2.3.4.1.

          - Subblock interleaving

The subblock interleaver requires the parameters m and J shown in Table eee of which NEP set corresponds to that in Table ccc.

The detailed subblock interleaver structures except table for subblock interleaver parameters correspond to 8.4.9.2.3.4.2.

| Index | NEP | m | J | Index | NEP | m | J | Index | NEP | m | J | Index | NEP | m | J | Index | NEP | m | J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 48 | 3 | 3 | 30 | 328 | 6 | 3 | 60 | 720 | 7 | 3 | 90 | 1424 | 8 | 3 | 120 | 2752 | 9 | 3 |
| 1 | 64 | 4 | 2 | 31 | 344 | 6 | 3 | 61 | 736 | 7 | 3 | 91 | 1448 | 8 | 3 | 121 | 2816 | 9 | 3 |
| 2 | 72 | 4 | 3 | 32 | 352 | 6 | 3 | 62 | 752 | 7 | 3 | 92 | 1480 | 8 | 3 | 122 | 2880 | 9 | 3 |
| 3 | 80 | 4 | 3 | 33 | 360 | 6 | 3 | 63 | 768 | 7 | 3 | 93 | 1504 | 8 | 3 | 123 | 2944 | 9 | 3 |
| 4 | 88 | 4 | 3 | 34 | 368 | 6 | 3 | 64 | 776 | 7 | 4 | 94 | 1536 | 8 | 3 | 124 | 3008 | 9 | 3 |
| 5 | 96 | 4 | 3 | 35 | 376 | 6 | 3 | 65 | 800 | 7 | 4 | 95 | 1560 | 8 | 4 | 125 | 3072 | 9 | 3 |
| 6 | 104 | 4 | 4 | 36 | 384 | 6 | 3 | 66 | 824 | 7 | 4 | 96 | 1600 | 8 | 4 | 126 | 3200 | 9 | 4 |
| 7 | 120 | 5 | 2 | 37 | 400 | 6 | 4 | 67 | 848 | 7 | 4 | 97 | 1640 | 8 | 4 | 127 | 3264 | 9 | 4 |
| 8 | 128 | 5 | 2 | 38 | 416 | 6 | 4 | 68 | 872 | 7 | 4 | 98 | 1672 | 8 | 4 | 128 | 3328 | 9 | 4 |
| 9 | 136 | 5 | 3 | 39 | 432 | 6 | 4 | 69 | 888 | 7 | 4 | 99 | 1712 | 8 | 4 | 129 | 3392 | 9 | 4 |
| 10 | 144 | 5 | 3 | 40 | 440 | 6 | 4 | 70 | 912 | 8 | 2 | 100 | 1752 | 8 | 4 | 130 | 3456 | 9 | 4 |
| 11 | 152 | 5 | 3 | 41 | 456 | 7 | 2 | 71 | 936 | 8 | 2 | 101 | 1784 | 8 | 4 | 131 | 3520 | 9 | 4 |
| 12 | 160 | 5 | 3 | 42 | 472 | 7 | 2 | 72 | 960 | 8 | 2 | 102 | 1824 | 9 | 2 | 132 | 3648 | 10 | 2 |
| 13 | 176 | 5 | 3 | 43 | 480 | 7 | 2 | 73 | 984 | 8 | 2 | 103 | 1864 | 9 | 2 | 133 | 3712 | 10 | 2 |
| 14 | 184 | 5 | 3 | 44 | 496 | 7 | 2 | 74 | 1000 | 8 | 2 | 104 | 1896 | 9 | 2 | 134 | 3776 | 10 | 2 |
| 15 | 192 | 5 | 3 | 45 | 512 | 7 | 2 | 75 | 1024 | 8 | 2 | 105 | 1920 | 9 | 2 | 135 | 3840 | 10 | 2 |
| 16 | 200 | 5 | 4 | 46 | 528 | 7 | 3 | 76 | 1048 | 8 | 3 | 106 | 1952 | 9 | 2 | 136 | 3904 | 10 | 2 |
| 17 | 208 | 5 | 4 | 47 | 544 | 7 | 3 | 77 | 1072 | 8 | 3 | 107 | 2000 | 9 | 2 | 137 | 3968 | 10 | 2 |
| 18 | 216 | 5 | 4 | 48 | 552 | 7 | 3 | 78 | 1096 | 8 | 3 | 108 | 2048 | 9 | 2 | 138 | 4096 | 10 | 2 |
| 19 | 232 | 6 | 2 | 49 | 568 | 7 | 3 | 79 | 1112 | 8 | 3 | 109 | 2096 | 9 | 3 | 139 | 4160 | 10 | 3 |
| 20 | 240 | 6 | 2 | 50 | 584 | 7 | 3 | 80 | 1136 | 8 | 3 | 110 | 2144 | 9 | 3 | 140 | 4224 | 10 | 3 |
| 21 | 248 | 6 | 2 | 51 | 600 | 7 | 3 | 81 | 1160 | 8 | 3 | 111 | 2192 | 9 | 3 | 141 | 4288 | 10 | 3 |
| 22 | 256 | 6 | 2 | 52 | 608 | 7 | 3 | 82 | 1184 | 8 | 3 | 112 | 2232 | 9 | 3 | 142 | 4352 | 10 | 3 |
| 23 | 264 | 6 | 3 | 53 | 624 | 7 | 3 | 83 | 1216 | 8 | 3 | 113 | 2280 | 9 | 3 | 143 | 4416 | 10 | 3 |
| 24 | 272 | 6 | 3 | 54 | 640 | 7 | 3 | 84 | 1248 | 8 | 3 | 114 | 2328 | 9 | 3 | 144 | 4544 | 10 | 3 |
| 25 | 288 | 6 | 3 | 55 | 656 | 7 | 3 | 85 | 1280 | 8 | 3 | 115 | 2368 | 9 | 3 | 145 | 4608 | 10 | 3 |
| 26 | 296 | 6 | 3 | 56 | 664 | 7 | 3 | 86 | 1312 | 8 | 3 | 116 | 2432 | 9 | 3 | 146 | 4672 | 10 | 3 |
| 27 | 304 | 6 | 3 | 57 | 680 | 7 | 3 | 87 | 1336 | 8 | 3 | 117 | 2496 | 9 | 3 | 147 | 4736 | 10 | 3 |
| 28 | 312 | 6 | 3 | 58 | 696 | 7 | 3 | 88 | 1368 | 8 | 3 | 118 | 2560 | 9 | 3 | 148 | 4800 | 10 | 3 |
| 29 | 320 | 6 | 3 | 59 | 712 | 7 | 3 | 89 | 1392 | 8 | 3 | 119 | 2624 | 9 | 3 |  |  |  |  |

Table eee ― The parameters for the subblock interleavers

          - Bit grouping

Corresponding to section 8.4.9.2.3.4.3.

          - Resource segmentation

If KFB > 1, the NRE data resource elements allocated for the subpacket are segmented into KFB blocks, one for each FEC block. The number of data resource elements for the k-th FEC block is defined by following equation.

NRE, k=2×NRE2+(KFB−k−1)KFB,0≤k<KFB

          - Bit selection and repetition

Bit selection and repetition are performed to generate the subpacket.

Let NCTC, k be the number of coded bits that shall be transmitted for the k-th FEC block. The value of NCTC, k is calculated by the following equation

NCTC, k=NRE, k⋅NSM⋅Nmod.

The index in the HARQ buffer for the j-th bit transmitted for the k-th FEC block shall be:

uk, j=(Pi,k+j)mod (3⋅NFB), for k = 0, …, KFB – 1, and j=0,...,NCTC,k−1,

where i is the subpacket ID of the subpacket (SPID = i), and Pi,k is the starting position for subpacket i of the k-th FEC block as specified in 15.x.2.1.

          - Bit collection

The selected bits from each FEC block are collected in the order of FEC block for the HARQ transmission.

          - Modulation

Correspond to 8.4.9.4.2.

    - HARQ

      - IR HARQ

HARQ IR is used in 802.16m, by changing the starting position, Pi,k, of the bit selection for HARQ retransmissions.

For downlink, it is determined as a function of SPID for the following equation.

Pi,k=24⋅SPIDi⋅3NFB4⋅24

For uplink, it is determined as a function of SPID for the following equation.

Pi,k=SPIDi⋅NCTC,kmod3NFB

-------------------------------  Text End  ---------------------------------------------------