| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AMS Location Privacy |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | John Harris, Hua Xu, Stavros Tzavidas | Hua.xu@motorola.com, John.Harris@motorola.com |
| Re: | IEEE 802.16m-09/0012: Call for Contributions and Comments on Project 802.16m Amendment Working Document (AWD) |  |
| Abstract | This contribution proposes a AMS location privacy scheme. |  |
| Purpose | For review and adoption of proposed text in AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AMS Location Privacy

John Harris, Hua Xu, Stavros Tzavidas

Motorola

# Introduction

From section 10.5.2.2.1 of the 802.16m system description document states.  "In order to reduce signaling overhead and provide a location privacy, the paging controller may assign temporary identifier to uniquely identify the AMS in the idle mode any particular paging group."

In that light we propose to derive this temporary identifier by feeding the following inputs into a hash function: (1) the AMS' MAC address, (2) an 8-bit nonce (which the ABS gives to the AMS during idle mode entry) and (3) the paging group ID (PG-ID).

# Text Proposal

-------------------------------  Text Start  ---------------------------------------------------

[Insert a new section 15: ]

- 15.2.x Power Management

15.2.x.x Idle Mode

15.2.x.x.x Idle Mode Entry/Exit Procedure

15.2.x.x.x.1 Idle Mode Initiation

In order to reduce signaling overheads and provide location privacy, the ABS gives an 8-bit nonce to the AMS during idle mode entry. The nonce assignment may happen during idle mode entry or during location update due to paging group change. The AMS Mac Address hash value shall be the result of feeding following inputs into a hash function: (1) the AMS' MAC address, (2) an 8-bit nonce (which the ABS gives to the AMS during idle mode entry) and (3) the paging group ID (PG-ID).  The AMS Mac Address hash identifier may be used in paging messages or during AMS’s network re-entry procedure from idle mode as response to paging.

- 15.2.x MAC PDU Formats

15.2.x.x MAC management messages

- 15.2.x.x.x ABS broadcast paging MOB_PAG-ADV) message

[modify section 6.3.2.3.51 of 802.16Rev2_D9]

AMSs are identified in the BS Broadcast AMS MAC Address hash. A single BS Broadcast Paging message may include multiple MAC Addresses. For a given BS Broadcast Paging message in a specific BS Paging Interval, the BS shall include only those AMS MAC Address hash particular to

the PAGING_CYCLE.

The MOB_PAG-ADV message shall be sent on the Broadcast CID or Idle mode multicast CID during the

BS Paging Interval.

The MAC Management Message Type for this message is given in Table 14. The format of the message is

shown in Table 1.

Table 1—BS Broadcast Paging (MOB_PAG-ADV) message format

| Syntax | Size | Notes |
| --- | --- | --- |
| MBS_MAP Message format (){ | — | — |
| Management Message Type=62 | 8 bits | — |
| Num_Paging_Group_IDs | 8 bits | Number of Paging Group IDs in this message |
| For (i=0; i<Num_Paging_Group_IDs; i++) { | — | — |
| Paging Group ID | 16 bits | — |
| } | — | — |
| Num_MACs | 8 bits | Number of AMS MAC addresses |
| For (j=0; j<Num_MACs; j++) { | — | — |
| AMS MAC Address hash | 24 bits | The hash is obtained by computing a CRC24 on the AMS 48-bit MAC address. The polynomial for the calculation is 0x1864CFB |
| Action Code | 2 bits | Paging action instruction to AMS 0b00=No Action Required 0b01=Perform Ranging to establish location and acknowledge message 0b10=Enter Network 0b11=Reserved |
| Reserved | 6 bits | — |
| } | — | — |
| Padding | variable | Padding bits to ensure octet aligned |
| TLV Encoded Information | variable | TLV specific |
| } | — | — |

A BS shall generate MOB_PAG-ADV including the following parameters:

Paging Group ID (16 bit)

One or more logical affiliation groupings of BS.

AMS MAC Address hash

This is a 24-bit field used to hash the AMS 48-bit MAC address. The hash value shall be the result of feeding following inputs into a hash function: (1) the AMS' MAC address, (2) an 8-bit nonce (which the ABS gives to the AMS during idle mode entry) and (3) the paging group ID (PG-ID).

Action Code

Paging action instruction to the AMS to perform the following action:

0b00 = No action required

0b01 = Perform ranging to establish location and acknowledge message

0b10 = Enter network

0b11 = Reserved

For OFDMA PHY, when a BS pages multiple AMSs, the BS may assign dedicated CDMA codes to one or more AMS being paged. The BS shall first list the MAC Address Hash of those AMSs that are assigned dedicated CDMA codes, followed by the AMSs that are not assigned dedicated CDMA codes.

For OFDMA PHY, the following TLV may be included in the MOB_PAG-ADV management message:

CDMA code and transmission opportunity assignment (11.17.1)

OFDMA-PHY-specific parameter used to indicate CDMA code and transmission opportunity assigned to one or more AMSs being paged in this message. One CDMA code and transmission opportunity assignment in the TLV corresponds to one AMS paged. The order of the assignments is the same as the order of appearance of AMS MAC address hash except with action code “No Action Required” in this message.

For OFDMA PHY, the following TLV may be included in the MOB_PAG-ADV management message. If a CDMA code assignment TLV is included, the Page-Response window TLV shall be included. There shall be no more than one occurrence of the Page-Response window TLV.

Page-Response window (11.17.2)

OFDMA-PHY-specific parameter used to indicate the time window (in unit of frames) during which the AMS shall transmit the CDMA code at the transmission opportunity assigned in the CDMA code and transmission opportunity assignment TLV. The start of the window is the next frame after receiving the MOB_PAG-ADV.

============================== End of Proposed Text ===============