| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Non-periodic real-time uplink scheduling service |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Xiao Xu, Hua Xu | Hua.xu@motorola.com |
| Re: | IEEE 802.16m-09/0012: Call for Contributions and Comments on Project 802.16m Amendments |  |
| Abstract | This contribution proposes an uplink scheduling scheme, namely non-periodic real-time uplink scheduling service. |  |
| Purpose | For review and adoption of proposed text in 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Non-periodic real-time uplink scheduling service

Xiao Xu, Hua Xu

Motorola

# Introduction

In this contribution, we propose an uplink scheduling service suitable for non-periodic applications with variable packet sizes such as interactive gaming. This scheduling service aims to provide fast response to bandwidth fluctuations in order to satisfy stringent delay requirements, while at the same time avoid significant over-provisioning.

# Text Proposal

================= Start of Proposed Text ========================

15.2.x Uplink request/grant scheduling

15.2.x.x Non-periodic real-time scheduling service (NPrts)

This scheduling service is designed to support real-time applications with non-periodic arrivals and variable packet sizes. An example application with this characteristic is interactive gaming. Since both the arrival patterns and the sizes of packets for these applications are unpredictable, and the delay requirements for these applications are very stringent in order to achieve a satisfactory end-user experience, the NPrts shall provide sufficient uplink bandwidth most of the time while providing mechanisms for AMS to request one-time additional bandwidth when needed.

As uplink bandwidth requests are costly in time, they should be made infrequently. During connection establishment, the ABS and the AMS shall negotiate and agree on a Minimum Reserved Traffic Rate, which represents the bandwidth that is sufficient for most (e.g., 90%) of the uplink packets. The ABS shall grant this bandwidth to the AMS periodically (potentially this period can be every one frame in order to minimize delays). In case the AMS observes more data in its buffer than can be transmitted by the Minimum Reserved Traffic Rate, it sends a request for additional bandwidth using the extended piggyback request field of the Grant Management Extended Header. This bandwidth request shall be incremental. The ABS makes a decision on whether to grant this request for additional bandwidth. If granted, the AMS shall be able to use this additional bandwidth over one or multiple frames. If for reasons of frame congestion or uplink power limitation the AMS is not able to use bandwidth in one frame, credit shall be carried over by the ABS and the AMS shall be granted remaining bandwidth in future frames until the original total additional bandwidth can be consumed. After that the AMS’s uplink bandwidth grants shall return to its normal level which is the Minimum Reserved Traffic Rate. This one-time feature of the additional bandwidth in NPrts eliminates the need for another bandwidth change request process between the ABS and the AMS to decrease the uplink bandwidth grants back to the Minimum Reserved Traffic Rate. Due to the highly variable packet sizes, these additional bandwidth requests tend to be one-time only and there is no need for them to be sustained.

The key QoS parameters for the non-periodic real-time scheduling service are the Traffic Rate Auto Tune-down, Minimum Reserved Traffic Rate, the Maximum Sustained Traffic Rate per flow, and the Maximum Latency.

============================== End of Proposed Text ===============