| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of HARQ functions for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Jason Junsung Lim, Yongbin Chang, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. 416 Maetan-3, Suwon, 443-770, Korea | +82-31-279-7467 Junsung.lim@samsung.com |
| Re: | IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: “HARQ functions” |  |
| Abstract | The contribution proposes text for HARQ functions to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of HARQ functions for the IEEE 802.16m Amendment

Jason Junsung Lim, Youngbin Chang, Jaeweon Cho, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

The purpose of this contribution is to propose table of contents (ToC) regarding overall HARQ functions for IEEE 802.16m Amendment Working Document. In addition, this contribution proposes text for the following sub-clauses in the said ToC, i.e. HARQ burst generation and HARQ signaling and timing. We note that the series of the following contribution or its last version provide the proposed text for the specific sub-clauses.

- C80216m-09_0539 : 15.2.x.2.2 A-MAP Relevance and HARQ timing

- C80216m-09_0540: 15.2.x.3 Persistent allocation HARQ signaling and timing

- C80216m-09_0541: 15.2.x.4 Group resource allocation HARQ signaling and timing

In particular, this contribution focus on text for the procedure of generating HARQ data burst in sub-clause 15.2.x.1. Furthermore, this contribution contains text for the generic HARQ signaling in sub-clause 15.2.x.2 which represents asynchronous HARQ scheme for downlink and synchronous HARQ scheme for uplink. Therefore, A-MAP IE should be transmitted for retransmission in downlink and A-MAP IE may or may not be transmitted for retransmission in uplink. Upon no receiving A-MAP IE at a given time, MS shall transmit HARQ data burst through the resources used to send last HARQ data burst with the same ACID at the same interlaced sub-frame.

- References

[1] IEEE 802.16m-08/003r7, IEEE 802.16m System Description Document (SDD)

[2] IEEE 802.16m-09/0050r1, IEEE 802.16m Amendment Working Document (AWD)

- Text proposal for inclusion in the 802.16m amendment

[Remedy 1: Insert the following new sub-clause and text into Section 15.2]

-------------------------------  Text Start  ---------------------------------------------------

### 15.2.x    HARQ Functions

HARQ is a mandatory for both downlink and uplink. BS and MS shall be capable of maintaining multiple HARQ channels where each channel is identified by HARQ channel identifier (ACID).

15.2.x.1   HARQ burst generation

15.2.x.2 Generic HARQ signaling and timing

15.2.x.2.1 HARQ signaling

15.2.x.2.1.1 Downlink

Detected the failure on initial transmission of HARQ data burst, BS shall transmit DL assignment IE for the retransmission which includes the corresponding ACID, sub-packet identification, the index of assigned resources and other parameters. Upon receiving the assignment IE, MS is responsible to decode information on the resource region indicated by DL assignment IE and transmits HARQ feedback to the BS. The timing for transmitting assignment IE, HARQ data burst and HARQ feedback are predefined as described in 15.2.x.2.2.

15.2.x.2.1.2 Uplink

For uplink HARQ transmission, the timing for transmitting UL assignment IE, HARQ data burst and HARQ feedback are predetermined as described in 15.2.x.2.2. Detected the failure on initial transmission of HARQ data burst, BS shall transmit Negative Acknowledgement to the MS. In further, BS may or may not transmit UL assignment IE for the retransmission. Upon receiving UL assignment MAP IE, MS shall transmit the retransmitted HARQ data burst through the resources indicated by the IE. Otherwise, MS shall transmit the retransmitted HARQ data burst through the resources used in the previous HARQ transmission at the same interlaced subframe with the same ACID. The ACID used in different interlaced subframe indexes different HARQ data burst, since retransmissions of HARQ data burst proceed in an interlaced subframe.

15.2.x.2.2 A-MAP Relevance and HARQ timing

15.2.x.3 Persistent allocation HARQ signaling and timing

15.2.x.4 Group resource allocation HARQ signaling and timing

-------------------------------  Text End  ---------------------------------------------------