| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for the 802.16m amendment (802.16m-09/0010) for section 11.1 on Power Control |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Ali T. Koc, Yang Rongzhen, Shilpa Talwar, Hujun Yin, Nageen Himayat, Guan, Wei; Papathanassiou, Apostolos; Choi, Yang-seok Intel Corporation | Ali.t.koc@intel.com |
| Re: | 802.16m amendment text”: IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: “Power Control”. |  |
| Abstract | The contribution proposes the text of uplink power control section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for the 802.16m amendment (802.16m-09/0010) for section 11.1.2 on Uplink Power Control

Intel Corporation

- Introduction

The contribution proposes the text on uplink power control schemes for 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D9 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

- References

[1] IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[2] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements”

[3] IEEE 802.16m-08/003r7, “IEEE 802.16m System Description Document [Draft]”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

------------------------------- Text Start ---------------------------------------------------

## Power Control

Uplink power control shall support the UL channel through both an initial calibration and periodic adjustment procedure without loss of data. The uplink power control algorithm determines the transmission power of an OFDM symbol to compensate for the pathloss, shadowing and fast fading. Uplink power control can be used to control inter-cell interference level. ABS shall transmit necessary information to AMSs through control channel or message to support uplink power control. The parameters of power control algorithm are optimized on system-wide basis by the ABS, and broadcasted periodically. AMS can transmit necessary information through control channel or message to ABS to support uplink power control. ABS can exchange necessary information with neighbor ABSs through backbone network to support uplink power control.

A transmitting AMS shall maintain the same transmitted power density, unless the maximum power level is reached. In other words, when the number of active LRU allocated to a user is reduced, the total transmitted power shall be reduced proportionally by the AMS, without additional power control messages. When the number of LRU is increased, the total transmitted power shall also be increased proportionally. However, the transmitted power level shall not exceed the maximum levels dictated by signal integrity considerations and regulatory requirements. The AMS shall interpret power control messages as the required changes to the transmitted power density.

### Open Loop Power Control

When the open-loop power control is used, the power per subcarrier and per transmission antenna shall be maintained for the UL transmission as follows (1).

P(dBm)=L+SINRTarget+NI+OffsetAMSperAMS+OffsetABSperAMS	(1)

Where

SINRTarget is the target uplink SINR received by ABS, the mode to calculate the value is signaled from power control message.

P is the TX power level (dBm) per subcarrier and per transmission antenna for the current transmission.

L is the estimated average current UL propagation loss. It shall include AMS’s Tx antenna gain and path loss.

NI is the estimated average power level (dBm) of the noise and interference per subcarrier at ABS, not including ABS’s Rx antenna gain.

OffsetAMSperAMS is the correction term for AMS-specific power offset. It is controlled by AMS. Its initial value is zero.

OffsetABSperAMS is the correction term for AMS-specific power offset. It is controlled by ABS. with power control messages.

The estimated average current UL propagation loss, L, shall be calculated based on the total power received on the active subcarriers of the frame preamble, and with reference to the BS_EIRP parameter sent by the ABS.

When the user connects to network, it can negotiate which SINRTarget calculation formula that it supports. It is defined as:

SINRtarget=CN−10log10(R), mode 1maxSINRMIN, SINROPT, mode 2	(2)

Where

C⁄N is the normalized C/N of the modulation/FEC rate for the current transmission, as appearing in Error: Reference source not found. Error: Reference source not found can be modified as TBD.

R is the number of repetitions for the modulation/FEC rate

SINRMIN: is the SINR requirement for the minimum rate expected by ABS, is set by the power control message. For uplink data sharing channel, control channel and sounding channel, SINRMIN have the different setting values if mode 2 is selected:

SINRMIN=DataSINRMINACKCHSINRMIN (fixed value, TBD)CQICHSINRMIN (fixed value, TBD)SoundingSINRMIN (fixed value, TBD)

DataSINRMIN has 4 bits to represent the dB value among {-3, -2.5, -2, -1.5, -1, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4, 4.5, 5} (TBD).

SINROPT is the target SINR value for IoT control and tradeoff between overall system throughput and cell edge performance, decided by control parameter γ and SINRMIN:

SINROPT=10log10 max10(SINRMIN10), γ×SIRDL−1Nr

Where

γ: is the fairness and IoT control factor, decided by ABS and set by the power control message. For uplink data sharing channel, control channel and sounding channel, γ have the different setting values if mode 2 is selected:

γ=γDataγACKCH (fixed value, TBD)rCQICH (fixed value, TBD)rSounding (fixed value or =γData, TBD)

γData has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}

Nr: is the number of receive antennas at ABS.

SIRDL: the ratio of downlink signal vs. interference power, measured by AMS

For uplink control channel, mode 1 is the default power control mode.

Table 1 – Normalized C/N per modulation

| Modulation/FEC rate | Required C/N |
| --- | --- |
| ACK/NAK | -3.0 |
| CQI | 0 |
| MAP ACK/NAK | 0 |
| Ranging code | 3 |
| QPSK 1/3 | 0.5 |
| QPSK 1/2 | 6 |
| QPSK 2/3 | 7.5 |
| QPSK 3/4 | 9 |
| 16-QAM-1/2 | 12 |
| 16-QAM-2/3 | 14.5 |
| 16-QAM-3/4 | 15 |
| 16-QAM-5/6 | 17.5 |
| 64-QAM-1/2 | 18 |
| 64-QAM-2/3 | 20 |
| 64-QAM-3/4 | 21 |
| 64-QAM-5/6 | 23 |

### Close Loop Power Control

To maintain at the ABS a power density consistent with the modulation and FEC rate used by each AMS, the ABS may change the AMS’s TX power, through power correction messages, as well as the AMS-assigned modulation and FEC rate. Upon transmission, the AMS shall use a temporary TX power value set according to (1) (in decibels).

Pnew=Plast+offset  (5)

Where

offset is the value indicated by Power correction value defined at UL power control IE:

Table 2 – Offset Value Table

| Power correction value | Offset (dB) |
| --- | --- |
| 0b00 | -0.5 |
| 0b01 | 0.0 |
| 0b10 | 0.5 |
| 0b11 | 1.0 |

Pnew= the power of the new UL burst in the current UL frame

Plast= the power of the burst in the most recently transmitted UL frame

------------------------------- Text End ---------------------------------------------------