| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multi-carrier Assignment Advanced MAP IE |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Kyounghwan Lee Kuk-Jin Song Yerang Hur POSDATA | Voice: +1-408-986-1143 E-mail: khlee@posdata-usa.com song@posdata.co.kr yehur@posdata-usa.com |
| Re: | 802.16m AWD |  |
| Abstract | This contribution proposes Multi-carrier Assignment Advanced MAP IE. |  |
| Purpose | To be discussed and adopted in 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Multi-carrier Assignment Advanced MAP IE

Kyounghwan Lee, Kuk-Jin Song, Yerang Hur

POSDATA

- Introduction

The assignment advanced MAP IE (A-A-MAP IE) in DL draft group text is only for single carrier operation [1]. To support seamless multi-carrier operation, we propose multi-carrier A-A-MAP IE complying with the latest A-A-MAP IE for single carrier operation included in the latest DL draft group text.

- Multi-carrier Assignment Advanced MAP IE

The followings are multi-carrier operation specific requirements and information [2].

- Multi-carrier AMS is controlled by a single fully-configured carrier called “primary carrier”. The AMS can have additional carrier as secondary carriers which are either fully-configured or partially-configured carrier.

- The secondary carrier is an additional carrier which the AMS may use for traffic, only per ABS’s specific allocation commands and rules typically on the primary carrier.

- A-MAP is present in a fully configured carrier. The presence and use of A-MAP on the partially configured carrier is FFS.

- The assigned secondary carriers for DL/UL asymmetrically based on system load and QoS, etc. This means that the number of active secondary carriers of DL is not necessarily same for UL.

Based on the above facts, there can be two types of burst allocations for a multi-carrier AMS: burst allocation on primary carrier and burst allocation on secondary carrier if it is assigned to the AMS.

In addition, since if present, the A-A-MAP IE for any active secondary carrier of a multi-carrier AMS provides burst allocation information for the carrier itself, the A-A-MAP IE for active secondary carrier would follow the same structure as single carrier A-A-MAP IE. Thus, the proposed multi-carrier A-A-MAP IE is transmitted only on the primary carrier.

The proposed multi-carrier A-A-MAP IE either allocates the burst for secondary carrier in the primary carrier directly or only provides A-MAP region information on the secondary carrier. In the latter case, AMS looks for its IE in the A-MAP region and finds the allocated burst information.

The multi-carrier AMS that supports multi-carrier operation decodes the proposed A-A-MAP IE transmitted on its primary carrier. By decoding it depending on DL or UL, AMS finds its allocated DL or UL burst. Secondary Carrier Allocation field in the IE acts as a flag such that if this field is set to 1, the IE provides burst allocation for its primary carrier as well as its active secondary carrier(s).

The Direct Allocation Bitmap provides information whether the burst allocation for active secondary carriers are directly provided or not. Bit 1 means the direct allocation. In this case, the AMS can directly access its allocated burst of the corresponding active secondary carrier. Bit 0 means that indirect allocation. In this case, the AMS does only know the A-MAP region that includes number of single carrier A-A-MAP IEs. The AMS locates its IE and find allocated burst.

Logical carrier index is the logical number for any active secondary carriers assigned to an AMS. The IE allocates burst allocation for the active secondary carriers from lowest index to highest index. To understand overall mechanism of the proposed IE, a high level flow chart describing overall procedure of proposed multi-carrier A-A-MAP IE for DL is shown in Figure 1.

Figure 1 Flow Chart for DL Multi-carrier Assignment Advanced MAP IE

- Proposed Text

---------------------------  Start of the proposed text  --------------------------------------------------------------

          - Multi-carrier A-A-MAP IE

Table x.x DL Multi-carrier A-A-MAP IE

| Syntax | Size [bits] | Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | The type is the DL Multi-carrier A-A-MAP IE |
| Secondary Carrier Allocation | 1 | 1: This IE contains burst allocation information on active DL secondary carrier(s) as well as the primary carrier. 0: This IE contains burst allocation information on the primary carrier only. |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| MM | [4] | MIMO Mode TBD MIMO modes for the basic MIMO IE include commonly used modes. Additional modes may be specified in extended IEs. |
| Resource Allocation | variable | Determines the start, length and number of subframes spanned the allocation. |
| Payload Size | TBD | Defines payload size if not included in resource allocation information. Clarification on the use of this field is required |
| Long TTI Length | TBD | Defines number of subframes spanned by allocations if not specified in resource allocation information. |
| Allocation Relevance | TBD | Defines allocation relevance if not specified in resource allocation information |
| Boosting | [3] | 000: Normal (not boosted) 001: +6dB; 010: –6dB 011: +9dB; 100: +3dB 101: –3dB; 110: –9dB 111: –12dB |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | [4] [5] | HARQ channel identifier |
| SPID/CoRe Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| If (Secondary Carrier Allocation == 1 ){ |  |  |
| Number of active DL secondary carriers | [2] | No. of active secondary carrier(s) of AMS that are allocated for DL traffic |
| Direct allocation bitmap | [4] | LSB is the lowest logical carrier index of active DL secondary carrier 1: The corresponding secondary carrier is directly allocated. 0: The corresponding secondary carrier is not directly allocated. |
| for (i=1:Number of active DL secondary carriers:i++){ |  |  |
| Logical index of active DL secondary carrier | TBD | The logical index of active secondary carrier of AMS that is allocated for DL traffic. The allocation is from the lowest logical index to the highest index. |
| If (Direct allocation bitmap[i] ==1){ |  |  |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| MM | [4] | MIMO Mode TBD MIMO modes for the basic MIMO IE include commonly used modes. Additional modes may be specified in extended IEs. |
| Resource Allocation | variable | Determines the start, length and number of subframes spanned the allocation. |
| Payload Size | TBD | Defines payload size if not included in resource allocation information. Clarification on the use of this field is required |
| Long TTI Length | TBD | Defines number of subframes spanned by allocations if not specified in resource allocation information. |
| Allocation Relevance | TBD | Defines allocation relevance if not specified in resource allocation information |
| Boosting | [3] | 000: Normal (not boosted) 001: +6dB; 010: –6dB 011: +9dB; 100: +3dB 101: –3dB; 110: –9dB 111: –12dB |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | [4] [5] | HARQ channel identifier |
| SPID/CoRe Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| } |  |  |
| else{ |  |  |
| A-MAP region information | TBD | The location of A-MAP region of the active DL secondary carrier |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |

Table x.x UL Multi-carrier A-A-MAP IE

| Syntax | Size [bits] | Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | The type is the UL Multi-carrier A-A-MAP IE |
| Secondary Carrier Allocation | 1 | 1: This IE contains burst allocation information on active UL secondary carrier(s) as well as the primary carrier. 0: This IE contains burst allocation information on the primary carrier only. |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| MM | [4] | MIMO Mode TBD MIMO modes for the basic MIMO IE include commonly used modes. Additional modes may be specified in extended IEs. |
| Resource Allocation | variable | Determines the start, length and number of subframes spanned the allocation. |
| Payload Size | TBD | Defines payload size if not included in resource allocation information. Clarification on the use of this field is required |
| Long TTI Length | TBD | Defines number of subframes spanned by allocations if not specified in resource allocation information. |
| Allocation Relevance | TBD | Defines allocation relevance if not specified in resource allocation information |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | [4] [5] | HARQ channel identifier |
| SPID/CoRe Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| If (Secondary Carrier Allocation == 1 ){ |  |  |
| Number of active UL secondary carriers | [2] | No. of active secondary carrier(s) of AMS that are allocated for UL traffic |
| Direct allocation bitmap | [4] | LSB is the lowest logical carrier index of active UL secondary carrier 1: The corresponding secondary carrier is directly allocated. 0: The corresponding secondary carrier is not directly allocated. |
| for (i=1:Number of active UL secondary carriers:i++){ |  |  |
| Logical index of active UL secondary carrier | TBD | The logical index of active secondary carrier of AMS that are allocated for UL traffic. The allocation is from the lowest logical index to the highest index. |
| If (Direct allocation bitmap[i] ==1){ |  |  |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| MM | [4] | MIMO Mode TBD MIMO modes for the basic MIMO IE include commonly used modes. Additional modes may be specified in extended IEs. |
| Resource Allocation | variable | Determines the start, length and number of subframes spanned the allocation. |
| Payload Size | TBD | Defines payload size if not included in resource allocation information. Clarification on the use of this field is required |
| Long TTI Length | TBD | Defines number of subframes spanned by allocations if not specified in resource allocation information. |
| Allocation Relevance | TBD | Defines allocation relevance if not specified in resource allocation information |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | [4] [5] | HARQ channel identifier |
| SPID/CoRe Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| } |  |  |
| else{ |  |  |
| A-MAP region information | TBD | The location of A-MAP region of the active UL secondary carrier |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |

--------------------------  End of the proposed text   ---------------------------------------------------------------

References

- DL Control Draft Group Text Version 2.7

- IEEE 802.16m-08/003r7. The Draft IEEE 802.16m System Description Document, February 2008.