| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text from DL PHY Ctrl Drafting group for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-xx |  |
| Source(s) | Xiaoyi Wang DL PHY Ctrl Drafting Group Chair | Xiaoyi.Wang@nsn.com |
| Re: | 802.16m AWD |  |
| Abstract | The contribution proposes the text of DL Control Structure section to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of DL Control Structure 
for the IEEE 802.16m Amendment

DL PHY Ctrl Drafting Group

- Introduction

This contribution proposes text to be included in the DL control section of the 802.16m amendment working document (AWD). The proposed text has been developed by the DL PHY Ctrl Drafting Group starting from session 59.

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

- 4. Abbreviations and acronyms

[Insert the following abbreviations and acronymns (Section 4):]

A-Preamble	advanced preamble

PA-Preamble	primary advanced preamble

SA-Preamble	secondary advanced preamble

P-SFH		primary superframe header

S-SFH 		secondary superframe header

A-MAP		advanced MAP

GRA		group resource allocation

PA		persistent allocation

IE		information element

[Editor’s note: Insert the following text into section 15.3.6 :]

# DL Control Structure

# Advanced Preamble

[Editor’s note:Place holder for synchronization DG output]

# DL Control Channels

[Editor’s note: This section describes the PHY resource allocated to the channel including physical bandwidth, frequency partition, DRU/CRUs used to construct the channel, tile structure with pilots and location of the channel within a subframe/superframe]

DL control channels convey information essential for system operation. Information on DL control channels is transmitted hierarchically over different time scales from the superframe level to the subframe level.

In mixed mode operation (WirelessMAN-OFDMA/Advanced Air interface), an AMS can access the system without decoding WirelessMAN-OFDMA FCH and MAP messages.

[  ------------------- Recommended AWD Text Proposal #1 --------------------]

# Superframe Header

The Superframe Header (SFH) carries essential system parameters and system configuration information. The SFH is located in the first subframe within a superframe. SFH occupies no more BW than 5 MHz. If SFH occupies narrower BW than system BW, SFH is FDM with data and control within the same subframe.

The SFH are TDM with A-Preamble.

The PHY structure for resource allocation of the SFH is described in 15.3.5. The SFH is transmitted within a predefined frequency partition called SFH frequency partition. The PRUs of the SFH frequency partition are selected within 5MHz physical bandwidth. The PRU in the whole SFH frequency partition uses the 2 stream pilot pattern defined in 15. 3. 5. The whole frequency partition is mapped to be distributed LRUs.

The SFH is divided into two parts: Primary Superframe Header (P-SFH) and Secondary Superframe Header (S-SFH).

Table 1 inludes the parameters and values for resource allocation of the SFH.

Table 1: Parameters and values for resource allocation of SFH

| Parameters | Description | Value |
| --- | --- | --- |
| NDLRU, SFH | The number of distributed LRUs which are occupied by SFH Note that NSFH = NP-SFH + NS-SFH | TBD (<= 24 (i.e. 5 MHz)) |
| N DLRU, P-SFH | The number of distributed LRUs which are occupied by P-SFH | Fixed (value is TBD) |
| N DLRU, S-SFH | The number of distributed LRUs which are occupied by S-SFH | Variable (maximum value is TBD) |

If N DLRU, SFH is less than 24, the other DLRUs of the SFH frequency partition are allocated for data or other control transmission.

# Primary Superframe Header

The Primary Superframe Header (P-SFH) shall be transmitted in every superframe.

The first N DLRU, P-SFH distributed LRUs of SFH FP are allocated for P-SFH transmission. N DLRU, P-SFH is a fixed value.

# Secondary Superframe Header

The Secondary Superframe Header (S-SFH) may be transmitted in every superframe

If S-SFH is present, the S-SFH shall be mapped to the N DLRU, S-SFH distributed LRUs following the N DLRU, P-SFH distributed LRUs P-SFH

The information transmitted in S-SFH is divided into different sub-packets.

[  ----------------End of Recommended AWD Text Proposal #1 -----------------]

[  ------------------- Recommended AWD Text Proposal #2 --------------------]

# Advanced MAP (A-MAP)

The Advanced MAP (A-MAP) carries unicast service control information. Unicast service control information consists of user-specific control information and non-user-specific control information. User-specific control information is further divided into assignment information, HARQ feedback information, and power control information. Different A-MAP IE can be carried by different A-MAP PHY channels.  All the physical channels in A-MAP share a region of physical resources called A-MAP region.

In the DL subframes where the A-MAP regions can be allocated, each frequency partition may contain an A-MAP region. An A-MAP region, if present, shall occupy the first few distributed LRUs in a frequency partition.

A-MAP regions shall be located 'n' IEEE 802.16m subframes apart. If an A-MAP region is allocated in subframe N, the next A-MAP region shall be allocated in subframe N+n of the same frame. DL data allocations corresponding to the A-MAP region can correspond to resources in any subframes between successive A-MAP regions.  The values of n can be 1 or 2. Other values of n (3 and 4) are FFS. For example, for n=2, an A-MAP region in subframe N can point to resource allocation in subframe N or N+1 and the next A-MAP region is in subframe N+2.

Figure 1 illustrates the location of a MAP region for n=1 and 2 cases in the TDD mode.

Figure 1: Example on the Location of A-MAP regions in a TDD system with a 4:4 subframe DL:UL split

In the FDD mode, the first IEEE 802.16m DL subframe of each frame shall contain at least one A-MAP region. In the TDD mode, the first IEEE 802.16m DL subframe after each UL to DL transition shall contain at least one A-MAP region.

A-MAP and data channels shall be multiplexed using FDM.

A-MAP is span all OFDM symbols in a sub-frame.

# Non-user Specific A-MAP

Non-user-specific A-MAP consists of information that is not dedicated to a specific user or a specific group of users. It includes information required to decode other A-MAPs.

The resource occupied by non-user specific information is of fixed size.

# Assignment A-MAP

Assignment A-MAP contains resource assignment information. There are multiple types of resource assignment IEs (assignment A-MAP IE) for different resource allocation schemes. Each assignment A-MAP IE is coded separately and carries information for one or a group of users.

The size of Assignment A-MAP is indicated by non-user-specific A-MAP.

The minimum logical resource unit in A-A-MAP is called MLRU, consisting of [48] data tones.

[Editor’s note: it is already agreed that MLRU is formed by selecting a certain number of data tones from the LRU, but the detail number is to be decided..]

The first MLRU is formed by selecting tones pair-wisely from the first distributed LRU in an A-MAP region. The second MLRU is formed by selecting the remaining tones in the distributed LRU.

When all tones in the first distributed LRU have been associated to MLRUs, the same process should continue with the next distributed LRU in the A-MAP region and so on.

The assignment A-MAP IE shall be transmitted with one MLRU or multiple concatenated MLRUs in the A-MAP region, the determination of the resource size is based on the IE size and channel coding rate, where channel coding rate is selected based on AMS’ link condition.

# HARQ Feedback A-MAP

HARQ feedback AMAP carries HARQ ACK/NACK information for uplink data transmission.

Within a subframe, DL ACK channel is FDM with other A-MAP and data.

# Power Control A-MAP

Power Control A-MAP carries fast power control command to AMS.

[  ----------------End of Recommended AWD Text Proposal #2 -----------------]

# Resource Mapping of DL Control Channels

[Editor’s note: This section describes the mapping of information bits to the DL control channel physical resource (subcarriers/symbols), coding, modulation etc.]

# Superframe Header

SFBC shall be used as the transmission scheme for SFH in the case of ABS with two Tx antennas. For more than two Tx antennas, SFBC with precoding, which is decoded by the MS without any information on the precoding and antenna configuration, is used as the transmission scheme for SFH.

[  ------------------- Recommended AWD Text Proposal #3 --------------------]

# Primary Superframe Header

The P-SFH IE is processed and then mapped to the P-SFH as shown in Figure 2.

Figure 2– Mapping of P-SFH IE to P-SFH

The P-SFH IE shall be appended with NCRC, P-SFH bits CRC.

The P-SFH IE shall be scrambled with a cell-specific sequence. The cell-specific sequence is indicated by A-preamble.

The P-SFH IE shall be encoded by the channel encoder. A coding rate of 1/2 is used.

The encoded sequences shall be repeated NRep, P-SFH times.

The resulting bit sequences shall be modulated using QPSK.

# Secondary Superframe Header

The S-SFH IE is processed and mapped to S-SFH as shown in Figure 3

Figure 3 – Mapping of S-SFH IE to S-SFH

The S-SFH IE shall be appended with 16-bit CRC.

The S-SFH IE shall be scrambled with a cell-specific sequence.

The resulting sequence of bits shall be encoded by the channel encoder. A code rate of 1/2 or 1/3 is used.

The encoded sequences shall be repeated NRep, S-SFH times.

The repeated bit sequences shall be modulated using QPSK.

[  ----------------End of Recommended AWD Text Proposal #3 -----------------]

# Advanced MAP (A-MAP)

SFBC with precoding shall be used for A-MAP region

[  ------------------- Recommended AWD Text Proposal #4 --------------------]

# Non-user Specific A-MAP

Figure 4 – Chain of Non-user specific A-MAP-IE to A-A-MAP symbols

The Non-user specific A-MAP bit sequence shall be encoded with a fixed MCS.

The encoded sequences shall be repeated NRep, NS-A-MAP times.

The repeateds bit sequences shall be modulated using QPSK.

# Assignment A-MAP

The Assignment A-MAP (A-A-MAP) shall include one or multiple A-A-MAP-IEs and each of them is encoded separately. Figure 5 describes the procedure for constructing A-A-MAP symbols.

Figure 5 – Chain of A-A-MAP-IE to A-A-MAP symbols

Each A-A-MAP IE shall be appended with 16-bit CRC.

The resulting sequence of bits shall be encoded by channel encoder.

Depending on the assignment IE size, a rate matching function may be applied.

After rate matching, the encoded bit sequences shall be modulated using QPSK.

# HARQ Feedback A-MAP

HARQ feedback A-MAP (HF-A-MAP) contains HARQ-feedback-IEs for ACK/NACK feedback information to uplink data transmission.

Figure 7 – Chain of HF-A-MAP IE to HF-A-MAP symbols

Figure 7 shows the construction procedure of HF-A-MAP symbols from HF-A-MAP-IE.

# Power Control A-MAP

Power Control A-MAP (PC-A-MAP) contains PC-A-MAP-IEs for closed-loop power control of the uplink transmission. ABS shall transmit PC-A-MAP-IE to every AMS which operates in closed-loop power control mode.

Figure 7 – Chain of PC-A-MAP IE to PC-A-MAP symbols

Figure 7 shows the construction procedure of PC-A-MAP symbols from PC-A-MAP-IE.

The i-th PC-A-MAP-IE shall have the size of 2 bits according to power correction value.

Then the repetition is performed with , where  is the number of repetition and explicitly signaled.

The repeated bit sequence shall be modulated as a QPSK symbol and be scaled by  (), where  is the number of PC-A-MAP-IEs and  is the value determined by management entity to satisfy the link performance.

[  ----------------End of Recommended AWD Text Proposal #4 -----------------]

# DL Control Information Elements

[Editor’s note: This section describes the information content in the DL control information elements.]

# Broadcast Control Information Elements

# P-SFH IE

[  ------------------- Recommended AWD Text Proposal #5 --------------------]

The P-SFH IE contains essential system information and it is mapped to the P-SFH.  The format of the P-SFH IE is shown in Table 2

Table 2 P-SFH IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| P-SFH IE format () { |  |  |
| LSB of Superframe number | TBD |  |
| System Configuration Description change count | TBD | System Configuration Descriptor Change Count |
| S-SFH Size | TBD |  |
| S-SFH Transmission Format | 2 | Indicate the transmission format (repetition) used for S-SFH. |
| } |  |  |

SCD Count

Incremented by one (modulo TBD) by the BS whenever any of the values of the S-SFH IEs changes. If the value of this count in a subsequent P-SFH IE remains the same, the AMS can quickly decide that the S-SFH IEs have not changed and may be able to disregard the S-SFH IEs.

[  ----------------End of Recommended AWD Text Proposal #5 -----------------]

# S-SFH IE

[  ------------------- Recommended AWD Text Proposal #6 --------------------]

The S-SFH IE is mapped to the S-SFH. Essential system parameters and system configuration information belonging to the S-SFH are categorized into multiple S-SFH IEs. The S-SFH IEs are transmitted in different timing and periodicity.

S-SFH SP1 IE presented in Table 3 indicate essential information needed for MS to select and access the network.

Table 3 S-SFH SP1 IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| MSBs of superframe number | TBD | Part of superframe number |
| S-SFH SP scheduling information | TBD | SP scheduling bitmap |
| S-SFH SP change bitmap | TBD | 1 or multiple bits per SP |
| } |  |  |

S-SFH SP2 contains information for network re-entry, see Table 4

Table 4 S-SFH SP2 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 2 | Sector ID Periodicity of A-MAP Sub-frame configuration (DL/UL ratio) DL permutation configuration (CRU, DRU partitioning and signaling related to that) UL permutation configuration (CRU, DRU partitioning and signaling related to that) FFR partitioning info for DL region (static) FFR partitioning info for UL region (static) FFR UL target IoT per partition # tx antenna MAC protocol revision Downlink burst profile BS EIRP UL carrier frequency UL bandwidth UL A-MAP relevance Uplink_Burst_Profile |  |

S-SFH SP3 contains information for initial network entry and network discovery, see Table 5

Table 5 S-SFH SP3 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 3 | Initial ranging channel information (initial ranging region location) Initial ranging channel format Initial ranging codes Initial ranging backoff start Initial ranging backoff end Minimum level of power offset adjustment Maximum level of power offset adjustment Duplex mode (TDD, FDD, HFDD) TTG RTG NSP IDs MSB bytes of BSID |  |

S-SFH SP4 contains information for maintaining communication with the ABS (e.g., periodic ranging, bandwidth request, sounding, HARQ parameters, and fast feedback parameters), see Table 6.

Table 6 S-SFH SP4 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 4 | Periodic ranging channel information (periodic ranging region location) Periodic ranging codes Periodic ranging backoff start Periodic ranging backoff end Bandwidth request channel information (bandwidth request region location) Bandwidth request backoff start Bandwidth request backoff end Sounding Region Fast Feedback Region HARQ Ack Region HARQ Ack delay |  |

S-SFH SP5 is transmitted in paging listening interval. It contains information for idle mode AMSs, see Table 7

Table 7: S-SFH SP5 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 5 | N_PGID Paging Indicator Usage Flag PGID List If( Paging Indicator Usage Flag ==1) ( Paging Indicator Bitmap } |  |

[  ----------------End of Recommended AWD Text Proposal #6 -----------------]

# Unicast Control Information Elements

A-MAP IE is defined as the basic element of unicast service control.

# Non-user-specific A-MAP IE

[  ------------------- Recommended AWD Text Proposal #7 --------------------]

Non-user-specific A-MAP IE consists of information that is not dedicated to a specific user or a specific group of users. It includes information required to decode assignment A-MAP IE The size of the A-MAP region is indicated in the Non-user-specific A-MAP IE, in the unit of [MLRU][LRU]. The detailed information included in non-user specific information is TBD.

[Editor’s note:it is undecided to use MLRU or LRU for non-user specific A-MAP IE. ]

Table 8: Non-user Specific A-MAP IE

| Syntax | Size [bits] | Notes |
| --- | --- | --- |
| A-MAP size | TBD | Indicate the size of A-MAP in the unit of [LRU][MLRU]. |

# Assignment A-MAP IE

Table 9 and Table 10 describe the fields in a DL Basic Assignment A-MAP IE and UL Basic Assignment A-MAP IE used for resource assignment in the DL and UL respectively.

Definitions of the fields in each IE are listed below the table.

[Editor’s note: the bracketed bits in following table is the estimated size based on SDD, not the final decision.]

Table 9: DL Basic Assignment A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | [4] | TBD A-MAP IE types distinguish between UL/DL, SU/MU, OL/CL MIMO operation, persistent/non-persistent allocation, basic/extended IEs |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| MM | [4] | MIMO Mode TBD MIMO modes for the basic MIMO IE include commonly used modes. Additional modes may be specified in extended IEs. |
| Resource Allocation | variable | Determines the start, length and number of subframes spanned the allocation. |
| Payload Size | TBD | Defines payload size if not included in resource allocation information. Clarification on the use of this field is required |
| Long TTI Length | TBD | Defines number of subframes spanned by allocations if not specified in resource allocation information. |
| Allocation Relevance | TBD | Defines allocation relevance if not specified in resource allocation information |
| Boosting | [3] | 000: Normal (not boosted) 001: +6dB; 010: –6dB 011: +9dB; 100: +3dB 101: –3dB; 110: –9dB 111: –12dB |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | [4] [5] | HARQ channel identifier |
| SPID/CoRe Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| Padding | variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |

* Estimated size based on currently defined features in SDD /AWD or proposed in AWD contributions

A-MAP IE Type: Defines the structure of the A-MAP IE for the bits in the A-MAP IE following the A-MAP IE type field.

A-MAP IE Type distinguishes between UL/DL, SU/MU OL/CL MIMO operation, persistent/non-persistent, single user/group resource allocation, basic/extended IE.

MM: MIMO Mode includes indicators for OL/CL operation, SU/MU allocations, rate and the number of streams.

RA: Resource Allocation determines the start, length and number of subframes spanned the allocation.

PS: Payload size defines the size of the allocation, units TBD.

Long TTI Length: Indicator to signal allocations span multiple subframes in time.

SPID/CoRe Version: Signaling for HARQ IR including HARQ subpacket identifier for IR and Constellation Rearrangement version.

MCRC: 16 bit CRC masked by Station ID/Flow ID.

Padding/Reserved Bits: padding to the nearest byte boundary.

Table 10: UL Basic Assignment A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | [4] | TBD types distinguish between UL/DL, MIMO/non-MIMO operation, persistent/non-persistent allocation, basic/extended IEs |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| MM | [4] | MIMO Mode TBD MIMO modes for the basic MIMO IE include commonly used modes. Additional modes may be specified in extended IEs. |
| Resource Allocation | variable | Determines the start, length and number of subframes spanned the allocation. |
| Payload Size | TBD | Defines payload size if not included in resource allocation information. Clarification on the use of this field is required |
| Long TTI Length | TBD | Defines number of subframes spanned by allocations if not specified in resource allocation information. |
| Allocation Relevance | TBD | Defines allocation relevance if not specified in resource allocation information |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | [4] [5] | HARQ channel identifier |
| Padding | variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |

* Estimated size based on currently defined features in SDD /AWD or proposed in AWD contributions

[  ----------------End of Recommended AWD Text Proposal #7 -----------------]

[  ------------------- Recommended AWD Text Proposal #8 --------------------]

      - Group Resource Assignment A-MAP IE

Group control information is used to allocate resources and/or configure resources to one or multiple mobile stations within a user group. The GRA A-MAP IE contains bitmaps to indicate scheduled user and also signal resource assignment, MCS, resource size.

ABS may use GRA A-MAP IE to allocate DL resources to one or multiple mobile stations within a user group.

[Editor’s Note: the bellowing IE is a placeholder for using a special type of A-A-MAP IE to configure a group, if tgm decide to use Mac Management Message for configuring a group, then this IE is not needed.].

      - [Group configuration A-MAP IE

Group configuration A-MAP IE is used for initiating and maintaining a group for resource assignment. ]

[Editor’s Note: the bellowing 2 IEs are placeholder for PA A-MAP IE, whether to have an independent PA IE is undecided ]

[  ----------------End of Recommended AWD Text Proposal #8 -----------------]

[  ------------------- Recommended AWD Text Proposal #9 --------------------]

      - [DL PA A-MAP IE

      - UL PA A-MAP IE]

# HARQ Feedback A-MAP IE

HARQ Feedback A-MAP IE includes one bit and corresponding value for HARQ ACK/NACK information is shown in Table xxx. If HF-A-MAP IE has the 0b0 or 0b1, it shall be interpreted as ACK information or NACK information, respectively.

Table xxx – HF-A-MAP-IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| HF-A-MAP IE format { |  |  |
| HF-A-MAP IE value | 1 | 0b0 : ACK feedback info. 0b1 : NACK feedback info. |
| } |  |  |

# Power Control A-MAP IE

PC-A-MAP IE includes two bits and corresponding value for power correction is shown in Table xxx. If PC-A-MAP IE has the 0b00, it shall be interpreted as tone power (power density) should be reduced by 0.5dB

Table xxx – PC-A-MAP IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| PC-A-MAP IE format { |  |  |
| Power correction value | 2 | 0b00 = -0.5 dB 0b01 = 0.0 dB 0b10 = 0.5 dB 0b11 = 1.0 dB |
| } |  |  |

[  ----------------End of Recommended AWD Text Proposal #9 -----------------]

-------------------------------  Text End  ---------------------------------------------------