| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text from DL PHY Ctrl Drafting group for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-12 |  |
| Source(s) | Xiaoyi Wang DL PHY Ctrl Drafting Group Chair | Xiaoyi.Wang@nsn.com |
| Re: | 802.16m AWD |  |
| Abstract | The contribution proposes the text of DL Control Structure section to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of DL Control Structure 
for the IEEE 802.16m Amendment

DL PHY Ctrl Drafting Group

- Introduction

This contribution proposes text to be included in the DL control section of the 802.16m amendment working document (AWD). The proposed text has been developed by the DL PHY Ctrl Drafting Group starting from session 59.

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

- 4. Abbreviations and acronyms

[Insert the following abbreviations and acronymns (Section 4):]

A-Preamble	advanced preamble

PA-Preamble	primary advanced preamble

SA-Preamble	secondary advanced preamble

P-SFH		primary superframe header

S-SFH 		secondary superframe header

A-MAP		advanced MAP

GRA		group resource allocation

PA		persistent allocation

IE		information element

[Editor’s note: Create a new section 15.3.5.3.4 and insert the following text]

15.3.5.3.4 Formation of MLRU

To form MLRUs for assignment A-MAP,

- Renumber all tone pairs in the distributed LRUs in the A-MAP region in a time first manner. Since each LRU has 8 tone-pairs per symbol, the renumbered A-MAP tone-pairs are denoted by RMP[u], where u ranges from 0 to LAMAP ∙ Nsym ∙ 8-1.

- A distributed tone-pair, SCLRUs,l,FPi[m], is mapped to RMP[u] where u=s ∙Nsym ∙8+m∙Nsym+l. SCLRUs,l,FPi[m] is the tone-pair index of the m-th tone-pair in the l-th OFDMA symbol in the s-th distributed LRU of frequency partition i as defined in Section 15.3.5.3.3.

- Suppose RMP[v] is the first tone-pair for allocation A-MAP. The k-th MLRU is formed by tone-pairs from RMP[v+k∙ NMLRU/2] to RMP[v+(k+1)∙NMLRU/2-1], where NMLRU is the size of MLRU.

[Editor’s note: Insert the following text into section 15.3.6 :]

# DL Control Structure

# Advanced Preamble

[Editor’s note:Place holder for synchronization DG output]

# DL Control Channels

[Editor’s note: This section describes the PHY resource allocated to the channel including physical bandwidth, frequency partition, DRU/CRUs used to construct the channel, tile structure with pilots and location of the channel within a subframe/superframe]

DL control channels convey information essential for system operation. Information on DL control channels is transmitted hierarchically over different time scales from the superframe level to the subframe level.

In mixed mode operation (WirelessMAN-OFDMA/Advanced Air interface), an AMS can access the system without decoding WirelessMAN-OFDMA FCH and MAP messages.

# Superframe Header

The Superframe Header (SFH) carries essential system parameters and system configuration information. The SFH is located in the first subframe within a superframe.

The SFH are TDM with A-Preamble.

The PHY structure for resource allocation of the SFH is described in 15.3.5. The SFH is transmitted within a predefined frequency partition called the SFH frequency partition. The SFH frequency partition consists of NPRU,SFH  PRUs within a 5 MHz physical bandwidth.

The PRUs in the SFH frequency partition uses the 2 stream pilot pattern defined in 15.3.5. The PRUs in the SFH frequency partition are permuted to generate NPRU,SFH  distributed LRUs.

The SFH is divided into two parts: Primary Superframe Header (P-SFH) and Secondary Superframe Header (S-SFH).

Error: Reference source not found inludes the parameters and values for resource allocation of the SFH.

Table 1: Parameters and values for resource allocation of SFH

| Parameters | Description | Value |
| --- | --- | --- |
| NDLRU, SFH | The number of distributed LRUs which are occupied by SFH Note that NDLRU, SFH = N DLRU,P-SFH + N DLRU,S-SFH | TBD (<= 24 (i.e. 5 MHz)) |
| N DLRU, P-SFH | The number of distributed LRUs which are occupied by P-SFH | Fixed (value is TBD) |
| N DLRU, S-SFH | The number of distributed LRUs which are occupied by S-SFH | Variable (maximum value is TBD) |

If N DLRU, SFH is less than 24, the other DLRUs of the SFH frequency partition are allocated for data or other control transmission.

Figure 1 illustrates the example of the subcarrier to resource unit mapping in the SFH frequency partition when assuming 10 MHz system bandwidth.

Figure 1: Example of subcarrier to resource unit mapping in the SFH frequency partition

# Primary Superframe Header

The Primary Superframe Header (P-SFH) shall be transmitted in every superframe.

The first N DLRU, P-SFH distributed LRUs of SFH frequency partition are allocated for P-SFH transmission. N DLRU, P-SFH is a fixed value.

# Secondary Superframe Header

The Secondary Superframe Header (S-SFH) may be transmitted in every superframe

If the S-SFH is present, the S-SFH shall be mapped to the N DLRU, S-SFH distributed LRUs following the N DLRU, P-SFH distributed LRUs.

The information transmitted in S-SFH is divided into different sub-packets.

# Advanced MAP (A-MAP)

The Advanced MAP (A-MAP) carries unicast service control information. Unicast service control information consists of user-specific control information and non-user-specific control information. User-specific control information is further divided into assignment information, HARQ feedback information, and power control information, and they are transmitted in the assignment A-MAP, HARQ feedback A-MAP, and power control A-MAP, respectively.. All the A-MAPs share a region of physical resources called A-MAP region.

A-MAP regions shall be located Nsubframe, A-MAP subframes apart in a frame. If an A-MAP region is allocated in subframe i, the next A-MAP region shall be allocated in subframe i+Nsubframe, A-MAP. In particular, for a frame with Nsubframe,DL DL subframes, A-MAP regions shall be present in subframe i, where i=j* Nsubframe, A-MAP and j is any non-negative integer such that j* Nsubframe, A-MAP< N subframe,DL. DL data allocations corresponding to the A-MAP region can correspond to resources in any subframes between successive A-MAP regions.  The values of Nsubframe, A-MAP can be 1 or 2. Other values of Nsubframe, A-MAP (3 and 4) are FFS. For example, for Nsubframe, A-MAP =2, an A-MAP region in subframe i can point to resource allocation in subframe i or i+1 and the next A-MAP region is in subframe i+2.

Figure 2 illustrates the location of a MAP region for Nsubframe, A-MAP=1 and 2 cases in the TDD mode.

Figure 2: Example on the Location of A-MAP regions in a TDD system with a 4:4 subframe DL:UL split

In the DL subframes where the A-MAP regions can be allocated, each frequency partition may contain an A-MAP region. An A-MAP region, if present, shall occupy the first few distributed LRUs in a frequency partition.

The structure of an A-MAP region is illustrated in the example in Figure 3. The resource occupied by each A-MAP physical channel may vary depending on the system configuration and scheduler operation.

An A-MAP region consists of LAMAP distributed LRUs and the LRUs are formed from PRUs with Nsym symbols.

Figure 3: The structure of an A-MAP region

# Non-user Specific A-MAP

Non-user-specific A-MAP consists of information that is not dedicated to a specific user or a specific group of users. It includes information required to decode other A-MAPs.

The resource occupied by non-user specific information is of fixed size.

# Assignment A-MAP

Assignment A-MAP contains resource assignment information which is categorized into multiple types of resource assignment IEs (assignment A-MAP IE). Each assignment A-MAP IE is coded separately and carries information for one or a group of users.

The size of the assignment A-MAP is indicated by non-user-specific A-MAP.

The minimum logical resource unit in the assignment A-MAP is called MLRU, consisting of [NMLRU =48] data tones.

[Editor’s note: it is already agreed that MLRU is formed by selecting a certain number of data tones from the LRU, but the detail number is to be decided..]

The assignment A-MAP IE shall be transmitted with one MLRU or multiple concatenated MLRUs in the A-MAP region. The number of logically contiguous MLRUs is determined based on the assignment IE size and channel coding rate, where channel coding rate is selected based on AMS’ link condition.

# HARQ Feedback A-MAP

HARQ feedback AMAP carries HARQ ACK/NACK information for uplink data transmission.

# Power Control A-MAP

Power Control A-MAP carries fast power control command to AMS.

# Resource Mapping of DL Control Channels

[Editor’s note: This section describes the mapping of information bits to the DL control channel physical resource (subcarriers/symbols), coding, modulation etc.]

# Superframe Header

# Primary Superframe Header

Figure 4 shows the physical processing block diagram for the P-SFH.

Figure 4: –Physical processing block diagram for the P-SFH

The P-SFH IE shall be appended with NCRC, P-SFH bits CRC followed by scrambling with a cell-specific sequence. The cell-specific sequence is determined from the A-Preamble.

The resulting sequence of bits shall be encoded by the convolutional encoder described in [refer to 15.3.x channel coding section]. A coding rate of 1/2 is used.

The encoded sequences shall be repeated NRep, P-SFH times.

The repeated bit sequences shall be modulated using QPSK.

The modulated symbols shall be mapped to two transmission streams using SFBC for two antennas. The two streams using SFBC may be precoded and mapped to more than two antennas.

Antenna specific symbols at the output of the MIMO encoder/precoder shall be mapped to the resource elements described in 15.3.6.2.1.1

# Secondary Superframe Header

Figure 5 shows the physical processing block diagram for the S-SFH.

Figure 5:  Physical processing block diagram for the S-SFH

The S-SFH IE shall be appended with 16-bit CRC followed by scrambling with a cell-specific sequence. The cell-specific sequence is determined from the A-Preamble.

The resulting sequence of bits shall be encoded by the channel encoder. The channel encoder for S-SFH is described in [refer to 15.3.x channel coding section]. A code rate of 1/2 or 1/3 is used.

The encoded sequences shall be repeated NRep, S-SFH times.

The repeated bit sequences shall be modulated using QPSK.

The modulated symbols shall be mapped to two transmission streams using SFBC for two antennas. The two stream using SFBC may be precoded and mapped to more than two antennas.

Antenna specific symbols at the output of the MIMO encoder/precoder shall be mapped to the resource elements described in 15.3.6.2.1.2.

# Advanced MAP (A-MAP)

SFBC with precoding shall be used for A-MAP region

# Non-user Specific A-MAP

Figure 6:  Chain of Non-user specific A-MAP-IE to A-A-MAP symbols

The Non-user specific A-MAP bit sequence shall be encoded with a fixed MCS.

The encoded sequences shall be repeated NRep, NS-A-MAP times.

The repeated bit sequences shall be modulated using QPSK.

# Assignment A-MAP

The Assignment A-MAP (A-A-MAP) shall include one or multiple A-A-MAP-IEs and each of them is encoded separately. Figure 7 describes the procedure for constructing A-A-MAP symbols.

Figure 7: Chain of A-A-MAP-IE to A-A-MAP symbols

Each A-A-MAP IE shall be appended with 16-bit CRC.

The resulting sequence of bits shall be encoded by a TBCC channel encoder.

Depending on the assignment IE size, a rate matching function may be applied.

Coded bits can be repeated to improve the robustness of an A-A-MAP channel based on the link condition of a particular AMS.

After rate matching and repetition, the encoded bit sequences shall be modulated using QPSK. For a given system configuration, assignment A-MAP IEs can be encoded with two different effective code rates. The exact code rates are TBD.

# HARQ Feedback A-MAP

HARQ feedback A-MAP (HF-A-MAP) contains HARQ-feedback-IEs for ACK/NACK feedback information to uplink data transmission.

Figure 8: Chain of HF-A-MAP IE to HF-A-MAP symbols

Figure 8 shows the construction procedure of HF-A-MAP symbols from HF-A-MAP-IE.

# Power Control A-MAP

Power Control A-MAP (PC-A-MAP) contains PC-A-MAP-IEs for closed-loop power control of the uplink transmission. ABS shall transmit PC-A-MAP-IE to every AMS which operates in closed-loop power control mode.

Figure 9 Chain of PC-A-MAP IE to PC-A-MAP symbols

Figure 9 shows the construction procedure of PC-A-MAP symbols from PC-A-MAP-IE.

The i-th PC-A-MAP-IE shall have the size of 2 bits according to power correction value.

Then the repetition is performed with , where  is the number of repetition and explicitly signaled.

The repeated bit sequence shall be modulated as a QPSK symbol and be scaled by  (), where  is the number of PC-A-MAP-IEs and  is the value determined by management entity to satisfy the link performance.

# DL Control Information Elements

[Editor’s note: This section describes the information content in the DL control information elements.]

# Broadcast Control Information Elements

# P-SFH IE

The P-SFH IE contains essential system information and it is mapped to the P-SFH.  The format of the P-SFH IE is shown in Table 2

Table 2 P-SFH IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| P-SFH IE format () { |  |  |
| LSB of Superframe number | TBD |  |
| System Configuration Description change count | TBD | System Configuration Descriptor Change Count |
| S-SFH Size | TBD |  |
| S-SFH Transmission Format | 2 | Indicate the transmission format (repetition) used for S-SFH. |
| [Reserved | TBD | Note the size of P-FSH should be fixed The reserved bits are for future extension] |
| } |  |  |

SCD Count

Incremented by one (modulo TBD) by the BS whenever any of the values of the S-SFH IEs changes. If the value of this count in a subsequent P-SFH IE remains the same, the AMS can quickly decide that the S-SFH IEs have not changed and may be able to disregard the S-SFH IEs.

# S-SFH IE

The S-SFH IE is mapped to the S-SFH. Essential system parameters and system configuration information belonging to the S-SFH are categorized into multiple S-SFH IEs. The S-SFH IEs are transmitted in different timing and periodicity.

S-SFH SP1 IE presented in Table 3 indicate essential information needed for MS to select and access the network.

Table 3 S-SFH SP1 IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| MSBs of superframe number | TBD | Part of superframe number |
| S-SFH SP scheduling information | TBD | SP scheduling bitmap |
| S-SFH SP change bitmap | TBD | 1 or multiple bits per SP |
| [Additional broadcast information indicator] | TBD |  |
| [HO information (Open BS/Close BS, UL load indicator, cell type) ] | TBD | For uncontrolled HO scenarios |
| [Cell bar info] | TBD | If Cell Bar bit=1, this cell is not allowed for any new initial entry |
| [Reserved] | TBD | The reserved bits are for future extension |
| } |  |  |

S-SFH SP2 contains information for network re-entry, see Table 4

Table 4 S-SFH SP2 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 2 | Sector ID Periodicity of A-MAP Sub-frame configuration (DL/UL ratio) DL permutation configuration (CRU, DRU partitioning and signaling related to that) UL permutation configuration (CRU, DRU partitioning and signaling related to that) FFR partitioning info for DL region (static) FFR partitioning info for UL region (static) FFR UL target IoT per partition # tx antenna MAC protocol revision Downlink burst profile BS EIRP UL carrier frequency UL bandwidth UL A-MAP relevance Uplink_Burst_Profile |  |

S-SFH SP3 contains information for initial network entry and network discovery, see Table 5

Table 5 S-SFH SP3 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 3 | Initial ranging channel information (initial ranging region location) Initial ranging channel format Initial ranging codes Initial ranging backoff start Initial ranging backoff end Minimum level of power offset adjustment Maximum level of power offset adjustment Duplex mode (TDD, FDD, HFDD) TTG RTG NSP IDs MSB bytes of BSID |  |

S-SFH SP4 contains information for maintaining communication with the ABS (e.g., periodic ranging, bandwidth request, sounding, HARQ parameters, and fast feedback parameters), see Table 6.

Table 6 S-SFH SP4 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 4 | Periodic ranging channel information (periodic ranging region location) Periodic ranging codes Periodic ranging backoff start Periodic ranging backoff end Bandwidth request channel information (bandwidth request region location) Bandwidth request backoff start Bandwidth request backoff end Sounding Region Fast Feedback Region HARQ Ack Region HARQ Ack delay |  |
|  | [SP scheduling periodicity (excluding SP1) information] | TBD |

S-SFH SP5 is transmitted in paging listening interval. It contains information for idle mode AMSs, see Table 7

Table 7: S-SFH SP5 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 5 | N_PGID Paging Indicator Usage Flag PGID List If( Paging Indicator Usage Flag ==1) ( Paging Indicator Bitmap } |  |

# Unicast Control Information Elements

A-MAP IE is defined as the basic element of unicast service control.

# Non-user-specific A-MAP IE

Non-user-specific A-MAP IE consists of information that is not dedicated to a specific user or a specific group of users. It includes information required to decode assignment A-MAP IE The size of the assignment A-MAP is indicated in the Non-user-specific A-MAP IE, in the unit of MLRU. The detailed information included in non-user specific information is TBD.

Table 8: Non-user Specific A-MAP IE

| Syntax | Size [bits] | Notes |
| --- | --- | --- |
| Assignment A-MAP size | TBD | Indicate the size of assignment A-MAP in the unit of one or multiple MLRU. |

# Assignment A-MAP IE

Table 9 and Table 10 describe the fields in a DL Basic Assignment A-MAP IE and UL Basic Assignment A-MAP IE used for resource assignment in the DL and UL respectively.

Definitions of the fields in each IE are listed below the table.

[Editor’s note: the bracketed bits in following table is the estimated size based on SDD, not the final decision.]

Table 9: DL Basic Assignment A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | [4] | TBD A-MAP IE types distinguish between UL/DL, SU/MU, OL/CL MIMO operation, persistent/non-persistent allocation, basic/extended IEs |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| MM | [4] | MIMO Mode TBD MIMO modes for the basic MIMO IE include commonly used modes. Additional modes may be specified in extended IEs. |
| Resource Allocation | variable | Determines the start, length and number of subframes spanned the allocation. |
| Payload Size | TBD | Defines payload size if not included in resource allocation information. Clarification on the use of this field is required |
| Long TTI Length | TBD | Defines number of subframes spanned by allocations if not specified in resource allocation information. |
| Allocation Relevance | TBD | Defines allocation relevance if not specified in resource allocation information |
| Boosting | [3] | 000: Normal (not boosted) 001: +6dB; 010: –6dB 011: +9dB; 100: +3dB 101: –3dB; 110: –9dB 111: –12dB |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | [4] [5] | HARQ channel identifier |
| SPID/CoRe Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| Padding | variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |

* Estimated size based on currently defined features in SDD /AWD or proposed in AWD contributions

A-MAP IE Type: Defines the structure of the A-MAP IE for the bits in the A-MAP IE following the A-MAP IE type field.

A-MAP IE Type distinguishes between UL/DL, SU/MU OL/CL MIMO operation, persistent/non-persistent, single user/group resource allocation, basic/extended IE.

MM: MIMO Mode includes indicators for OL/CL operation, SU/MU allocations, rate and the number of streams.

RA: Resource Allocation determines the start, length and number of subframes spanned the allocation.

PS: Payload size defines the size of the allocation, units TBD.

Long TTI Length: Indicator to signal allocations span multiple subframes in time.

SPID/CoRe Version: Signaling for HARQ IR including HARQ subpacket identifier for IR and Constellation Rearrangement version.

MCRC: 16 bit CRC masked by Station ID/Flow ID.

Padding/Reserved Bits: padding to the nearest byte boundary.

Table 10: UL Basic Assignment A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | [4] | TBD types distinguish between UL/DL, MIMO/non-MIMO operation, persistent/non-persistent allocation, basic/extended IEs |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| MM | [4] | MIMO Mode TBD MIMO modes for the basic MIMO IE include commonly used modes. Additional modes may be specified in extended IEs. |
| Resource Allocation | variable | Determines the start, length and number of subframes spanned the allocation. |
| Payload Size | TBD | Defines payload size if not included in resource allocation information. Clarification on the use of this field is required |
| Long TTI Length | TBD | Defines number of subframes spanned by allocations if not specified in resource allocation information. |
| Allocation Relevance | TBD | Defines allocation relevance if not specified in resource allocation information |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | [4] [5] | HARQ channel identifier |
| Padding | variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |

* Estimated size based on currently defined features in SDD /AWD or proposed in AWD contributions

      - Group Resource Allocation A-MAP IE

Group control information is used to allocate resources and/or configure resources to one or multiple mobile stations within a user group.

Group scheduling requires two operations

- Assignment of a user to a group. In order to add a user to a group in the DL or UL, the ABS shall transmit a [Group Configuration MAC management message] [Group Configuration A-MAP IE]

- Allocation of resources to users within a group. In order to assign resources to one or more users in a group, the ABS shall transmit the DL/UL Group Resource Allocation A-MAP IE. The DL/UL Group Resource Allocation A-MAP IE is included in user-specific resource assignment in an A-MAP region. The GRA A-MAP IE contains bitmaps to indicate scheduled users and signal resource assignment, MCS, resource size.

DL/UL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | [4] | TBD A-MAP IE types distinguish between UL/DL, SU/MU, OL/CL MIMO operation, persistent/non-persistent allocation, basic/extended IEs including GRA |
| Resource Offset | [6][8] | Indicates starting LRU for resource assignment to this group |
| ACID | TBD | TBD Required for explicit assignment of a single ACID group. Not required if implicit cycling of ACIDs starting from an initial ACID value is used as in 802.16e |
| HARQ ReTx Indicator | TBD | TBD Indicates whether this group resource assignment IE is for HARQ retransmissions or initial transmission |
| User Bitmap Size | [2][5] | TBD Size of the user bitmap; may not be needed if user bitmap size is included in configuration message/A-MAP IE |
| User Bitmap | Variable | Bitmap to indicate scheduled users in a group. The size of the bitmap is equal to the User Bitmap Size |
| Resource Allocation Bitmap descriptor | TBD | TBD MCS group type, packet/resource size type information or attributes required to decode resource assignment bitmap |
| Resource Assignment Bitmap | Variable | Bitmap to indicate MCS/resource size for each scheduled user |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit masked CRC |

[Editor’s Note: the bellowing IE is a placeholder for using a special type of A-A-MAP IE to configure a group, if tgm decide to use Mac Management Message for configuring a group, then this IE is not needed.].

      - [Group configuration A-MAP IE

Group configuration A-MAP IE is used for initiating and maintaining a group for resource assignment. ]

      - [DL PA A-MAP IE

      - UL PA A-MAP IE]

# HARQ Feedback A-MAP IE

HARQ Feedback A-MAP IE includes one bit and corresponding value for HARQ ACK/NACK information is shown in Table xxx. If HF-A-MAP IE has the 0b0 or 0b1, it shall be interpreted as ACK information or NACK information, respectively.

Table xxx – HF-A-MAP-IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| HF-A-MAP IE format { |  |  |
| HF-A-MAP IE value | 1 | 0b0 : ACK feedback info. 0b1 : NACK feedback info. |
| } |  |  |

# Power Control A-MAP IE

PC-A-MAP IE includes two bits and corresponding value for power correction is shown in Table xxx. If PC-A-MAP IE has the 0b00, it shall be interpreted as tone power (power density) should be reduced by 0.5dB

Table xxx – PC-A-MAP IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| PC-A-MAP IE format { |  |  |
| Power correction value | 2 | 0b00 = -0.5 dB 0b01 = 0.0 dB 0b10 = 0.5 dB 0b11 = 1.0 dB |
| } |  |  |

-------------------------------  Text End  ---------------------------------------------------