| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on A-MAP IE to Support A-MAP Occurrence Indication |  |
| Date Submitted | 2009-03-11 |  |
| Source(s) | Chieh-Yuan Ho, Ming-Hsien Wu, Ching-Yao Huang, I-Kang Fu, Yih-Shen Chen, Paul Cheng NCTU/MediaTek | IK.Fu@mediatek.com thorpe.ho@mediatek.com cyhuang@faculty.nctu.edu.tw |
| Re: | IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” – Comments on P802.16m AWD |  |
| Abstract | According to P802.16m SDD, assigned secondary carriers will not always be activated for saving unnecessary AMS processing power and signaling overheads. In addition, the AMS also need to know which sub-frame the AMS doesn’t need to decode A-MAP for sleep or co-located coexistence operations. This contribution suggests adding the relative parameters into the proper A-MAP IE. |  |
| Purpose | For member’s review and adoption into P802.16m Amendment Working Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on A-MAP IE to Support A-MAP Occurrence Indication

Chieh-Yuan Ho, Ming-Hsien Wu, Ching-Yao Huang, I-Kang Fu, Yih-Shen Chen, Paul Cheng

National Chiao Tung University (NCTU) / MediaTek

- Motivation

In the 802.16m System, Advanced Mobile Station (AMS) can support data transmission over multiple carriers. In order to best utilize the scarce radio resources, Advanced Base Station (ABS) may change carriers to serve data transmission for specific AMS from time to time, i.e. on a per frame basis. Therefore, an efficient method for carrier activation and de-activation will be necessary. Because AMS might consume more power while operating on more than one carrier, a detailed procedure of multi-carrier power management need to be carefully designed in order to further extend AMS battery life. The motivation of this contribution is to design an efficient carrier activation procedure and supporting message design to improve the performances of multi-carrier support in the 802.16m system.

- Introduction

In the latest P802.16m System Description Document (SDD), the carrier which may be potentially used by the AMS is defined as the assigned carriers. The assigned carriers are supposed to be determined based on ABS and AMS hardware capabilities. Among the assigned carriers, some of the carriers will be activated for data transmission based on system requirements and called the active carriers. Since the primary carrier will always be one of the active carriers, the secondary carriers of AMS may be activated or be de-activated from time to time.

In addition, the ABS should also let AMS know which sub-frames that AMS should decode the A-MAP. Otherwise AMS have to process the signals received during each sub-frame and decode all the A-MAP to see whether there exists its A-MAP IE. This will largely increase AMS complexity and power consumption. It will also disturb the multi-radio coexistence operation and possible sleep operation. Therefore, A-MAP should also transmit the proper IE to notify AMS in which sub-frames it should decode the A-MAP IEs.

In order to fulfill member’s classification on assigned carrier and active carrier, the stage 3 standard shall define proper procedure and messaging support for secondary carrier activation and de-activation. This contribution reviews the possible operations of secondary carriers and suggests the necessary input into SFH and A-MAP IE for enabling secondary carrier activation. The detail procedure will be further discussed and harmonized with TGm members.

- Proposed Text Input to P802.16m Amendment Working Document

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Add the following text into P802.16m Amendment Working Document]

# 15.3.6.2.2.5 Occurrence A-MAP

A-MAP Occurrence IE may be transmitted in the first sub-frame if transmitted and indicates the occurrence of the A-MAP IEs.

15.3.6.4.2.5 Occurrence A-MAP IE

A-MAP Occurrence IE indicates the occurrence of the A-MAP IEs by bitmap for subsequent 7 sub-frames. When the bitmap indicates “1”, at least one of the A-MAP IE defined in 15.3.6.5.2 will be occurred in the corresponding sub-frame. The order of the bitmap in A-MAP Occurrence IE is based on the sub-frame index.

Table xxx –A-MAP Occurrence IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| A-MAP Occurrence IE format { |  |  |
| A-MAP Occurrence Bitmap | 7 | 1: A-MAP IE is available 0: A-MAP IE is not available |
| } |  |  |

----------------------------------------------------------End of the Text-----------------------------------------------------------