| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal of SFH contents for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Yih-Shen Chen, I-Kang Fu, Kelvin Chou and Paul Cheng MediaTek Inc Ming-Hung Tao, Mamadou Kone, Ying-Chuan Hsiao ITRI | yihshen.chen@mediatek.com MHTao@itri.org.tw |
| Re: | IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) content”. Target topic: “Comment on 802.16m AWD”. |  |
| Abstract | The contribution proposes the text of SFH contents to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal of SFH contents for IEEE 802.16m Amendment

Yih-Shen Chen, I-Kang Fu, Kelvin Chou, Paul Cheng

MediaTek Inc.

Ming-Hung Tao, Mamadou Kone, Ying-Chuan Hsiao

ITRI

Introduction

In DL_CTRL DG amendment working document [1], SFH contents are categorized into subpackets (SPs) and broadcasted with different time periodicities. It is better to have a specific SP to announce the periodicities of the S-SFH SPs to facilitate scanning and power saving.

- Proposed text

Add the following text to [2]:

-------------------------------  Text Start  ---------------------------------------------------

# 15.3.6.5.1.2.6 S-SFH SP6 IE

S-SFH SP6 contains information for scheduling periodicities of SPs.

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 6 | S-SFH SP scheduling periodicities, except SP1 | TBD |

-------------------------------  Text End  ---------------------------------------------------

Reference

[1] IEEE 80216m-09/0558, Proposed text from DL PHY Ctrl Drafting group for IEEE 802.16m Amendment

[2] IEEE 802.16m-09/0010, IEEE 802.16m Amendment Working Document (AWD)