| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Sleep Mode operation in 16m |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Xin Qi, Shashikant Maheshwari, Yousuf Saifullah, Xiaoyi Wang Nokia Siemens Networks Zexian Li, Andrea Bacioccola, Jan Suumaki Nokia Maruti Gupta, Shantidev Mohanty, Muthaiah Venkatachalam Intel Corp. 2111 NE 25th Avenue, Hillsboro, OR 97124 Yih-Shen Chen, Paul Cheng MediaTek Inc. | E-mail: xin.qi@nsn.com Zexian.li@nokia.com shantidev.mohanty@intel.com Yihshen.chen@mediatek.com |
| Re: | “802.16m AWD”: IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: Power Management (Sleep Mode) |  |
| Abstract | Amendment Text for Sleep Mode operation in 16m |  |
| Purpose | Discuss and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Sleep Mode operation in 802.16m

Xin Qi, Shashikant Maheshwari, Yousuf Saifullah, Xiaoyi Wang

Nokia Siemens Networks

Zexian Li, Andrea Bacioccola, Jan Suumaki

Nokia

Maruti Gupta, Shantidev Mohanty, Muthaiah Venkatachalam

Intel Corp.

Yih-Shen Chen, Paul Cheng

MediaTek Inc

# Introduction

The contribution proposes the text of Sleep Mode section to be included in the 802.16m amendment [1]. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D9 [2], it is compliant to the 802.16m SRD [3] and the 802.16m SDD [4], and it follows the style and format guidelines in [5].

# Reference

- IEEE 802.16m-09/0010, “IEEE 802.16m Amendment Working Document”

- IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,” Jan. 2009.

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

# Text Proposal

================= Start of Proposed Text ========================

# 15.2.x.x. Sleep Mode

Sleep Mode is a state in which the AMS can remain absent for certain pre-negotiated intervals from the ABS air interface, while still actively connected to the ABS. The periods of AMS absence are characterized by AMS unavailability to the serving BS, to DL or UL traffic. In Sleep mode, the BS retains all the information related to the connections that currently belong to the AMS as it does during Normal Operations. Normal Operations is a mode where an AMS is always awake and the ABS or AMS can transmit to each other at anytime.

When an AMS is in Sleep mode, it alternates between pre-negotiated periods of unavailability known as Sleep window and availability known as Listen window. During Sleep Window in Sleep Mode, the ABS shall not transmit to the AMS; therefore the AMS may power down one or more physical operation components or perform other activities that do not require communication with the ABS. Such activities may include scanning for neighboring ABS, powering off components, or using sleep interval for co-located co-existence activities.

During Listen Window, the AMS is expected to receive all DL transmissions same way as in the state of normal operations. In addition, the AMS needs to ensure that it is able to verify synchronization and system configuration with the ABS prior to waking up in the Listen window. If the AMS wakes up to detect that there has been a change in the ABS, then it exits Sleep mode and performs network entry with the new ABS. Or if the AMS detects a changed system configuration, the AMS shall continue reception until receiving the corresponding updated information. In this case, the AMS does not exit from Sleep mode.

The length of successive sleep windows may remain constant or may be adaptive based on traffic conditions. Sleep windows and listening windows may also be dynamically adjusted for the purpose of data transportation as well as MAC control signaling transmission. AMS may send and receive data and MAC control signaling without deactivating the Sleep Mode.

For each involved AMS, the ABS keeps context known as Sleep Cycle settings which keeps track of all the parameters related to the AMS’ current sleep cycle. The Sleep cycle settings apply to all the connections of the AMS and is indicated by the Sleep Cycle ID (SCID).

## 15.2.x.x.1 Sleep mode initiation

Sleep Mode activation/entry is initiated by the MS and is supported by default. BS-initiated sleep can be negotiated as part of the capability parameters between the BS and the MS during network re-entry. This does not exclude the possibility of the BS terminating the sleep cycle at any time through the MOB_SLP-RSP message. When AMS is in active mode, parameters of the sleep cycle are negotiated between the AMS and ABS. ABS makes the final decision regarding the AMS request and instructs the AMS to enter sleep mode.The negotiation of sleep parameters is performed by using MAC management messages MOB_SLP-REQ and MOB_SLP-RSP. The AMS enters Sleep mode by first going into the Sleep window at the start of the Start Frame number, a parameter specified in the MOB_SLP-RSP message.

The AMS can initiate the negotiation by sending a MAC management message using a MOB_SLP-REQ message and receives a MOB_SLP-RSP message.

All Sleep mode messages must be acknowledged to avoid the falling out of sync when the MOB_SLP-RSP message is lost and it is assumed that Sleep mode messages use HARQ for transmission.

This signaling process takes place as shown below.. If the AMS does not receive a response, then the AMS repeats its request as shown in figure 1.

Alternatively, the ABS can also send a unsolicited MOB_SLP-RSP message to the AMS, and the AMS replies with MOB_SLP-ACK message as shown in figure 2 below.

Figure 1: AMS-initiated Sleep negotiation

Figure 2: ABS-initiated Sleep negotiation

## 15.2.x.x.2 Sleep mode operation

###### 15.2.x.x.2.1 Sleep cycle operations

The basic listening window length is a fixed value, which is set during the initiation of the Sleep Mode or during the PSC update. Listening window length could be dynamically extended by one or more frames, depending on traffic buffer status or HARQ status. The listening window extension is specified in section Error: Reference source not found. Sleep window length can be of fixed value or increase exponentially. A sleep cycle is the summation of adjacent listening window and sleep window. In the PSC with fixed sleep window, sleep cycle is of fixed length. In case of exponential sleep window, sleep cycle enlarges until the final sleep window length is arrived.

The definition of PSC could be updated based on the change of AMS traffic pattern.

16m Sleep mode operation specifies three different types of sleep mode operation, namely Fixed Sleep cycle with Fixed listen window, Fixed Sleep cycle with Extensible listen window and Variable Sleep cycle with Extensible listen window.

The parameters in the Sleep cycle (settings) are specified as follows:

- Listen Window: length of the Listen window

- Initial Sleep Window: length of initial Sleep window

- Final Sleep Window: length of initial Sleep window

- Listen Window Extension Flag (LWEF):

If LWEF = 0, indicates that the Listen window is of fixed duration.

If LWEF = 1, indicates that the Listen window can be extended and is of variable duration

- Traffic Indication Message Flag (TIMF)

If TIMF = 0, then a Traffic Indication Message is never sent

If TIMF = 1, then a Traffic Indication Message is sent every Listen window

###### 15.2.x.x.2.2 Sleep window operations

Figure 3: Sleep window operation when TIMF = 0 and LWEF = 0

### 15.2.x.x.2.2 Sleep Cycle Operation with Fixed Sleep and Extensible Listen Window.

In this type of operation, again the Initial and Final Sleep window lengths are the same. However,

the LWEF = 1. The TIMF can be set either 1 or 0. In this case, when there is DL traffic to be sent during Listen window, the ABS sends DL traffic during Listen window. If there is UL traffic, then it will be sent either during Listen windows (if MS already has UL allocations for UL data) or there may be a BW request message sent during the Sleep window.

### 15.2.x.x.2.3 Sleep Cycle Operation with Exponential Sleep and Extended Listen Window.

In this type of operation, the length of Initial Sleep window is not the same as the length of Final Sleep window and the Traffic Indication Message Flag is always set to 1 and the LWEF is also always set to 1.

If the Traffic Indication message is negative for the AMS or if there is no UL traffic during the Listening window (in short the conditions stated in section 15.2.x.x.2.4.2., the Sleep window doubles every sleep cycle using the following rule:

Sleep window = min (2*(Previous sleep window), Final-sleep window) --- (1)

Figure 5:  Sleep Cycle with Exponential Sleep and Extensible Listen Window

### 15.2.x.x.2.4 Listening window operations

### 15.2.x.x.2.4.1 Traffic Indication Message

Traffic Indication is sent for a group of AMS’s using the MOB_TRF-IND message and is placed at a

pre-determined fixed location, i.e. in the continuous NTRFIND distributed LRUs right following the A-MAP region in the 1st	subframe of a frame in the listening window. This message is sent if the Traffic Indication Message Flag is set to 1 and contains information in an hierarchical order, first at the group-level and then at the individual level. If the TIMF is set to 1 for a AMS with SLPID assigned, AMS needs to detect MOB_TRF-IND in each listening window. If the AMS finds that the traffic indication flag is negative, then it goes back to sleep for the rest of the Listening period. If it is positive, and if the LWEF is 1, then it extends its Listening windowas defined in section 15.2.x.x.2.4.2.

If MOB_TRF-IND can not be contained by two LRUs, then MOB_TRF-IND is segmented into two parts: MOB_TRF-IND_I and MOB_TRF-IND_II. MOB_TRF-IND_I is transmitted by 2 LRUs. If MOB_TRF-IND can be transmitted in two LRUs, then the segmentation will not be performed, and NTRFIND = 2. If segmented, MOB_TRF-IND_I and MOB_TRF-IND_II are separately encoded; the size of MOB_TRF-IND_II is indicated in MOB_TRF-IND_I; NTRFIND = (2+size of MOB_TRF-IND_II); MOB_TRF-IND_I is transmitted in the 1st two LRUs in the NTRFIND LRUs; MOB_TRF-IND_II is transmitted in the following NTRFIND-2 LRUs. MOB_TRF-IND is encoded by CTC with 0.5 coding rate plus 4 repetitions, and modulated by QPSK. The information format of MOB_TRF-IND is given below:

### 15.2.x.x.2.4.2  Listening window extension

The Listen window will be extended until the next Listen window if any of the following conditions is true:

- If the Listen window ends, but the ABS has pending HARQ retransmission retries and is waiting for HARQ ACK for DL data. The AMS will not go to sleep state if there are pending HARQ transmissions to be ACKed or until the next retransmission period of HARQ has elapsed after the ACK has been sent.

- If the Listen window ends, but the AMS has pending HARQ retransmission retries and is waiting for HARQ ACK for UL data.

- If the AMS has sent in a BW-REQ message during Listen window.

- If the TIMF = 1, and the ABS send a positive traffic indication message.

- If the AMS sends BW-REQ message during Sleep window, then the AMS is assumed to be awake until the Listen window following the sleep window; the listen window itself is not extended until the following Listen window.

### 15.2.x.x.2.4.3 Sleep Cycle Switching

The AMS or the ABS can switch the sleep cycle to a different sleep profile in response to many cases,

for example, if the AMS adds another transport connection to its existing connection or in response to change in traffic patterns, such as when the Listen window needs to be extended. Instead of the AMS resetting the Sleep Window to the initial Sleep window and then going through the process of exponentially increasing it again, the AMS may choose to change the sleep cycle to a fixed cycle duration with extensible Listen window or may change some parameters of existing sleep cycle.

The change in the sleep cycle can be accomplished by the AMS sending a MOB_SLP-REQ message with request to re-activate a previously defined sleep cycle or change the sleep parameters of existing PSCID. The ABS responds with a MOB_SLP-RSP message to confirm the change along with the start frame number of the next sleep cycle. At that point the sleep cycle changes to the new sleep cycle. Thus in figure 5, instead of the SW2 being reset to SW1, it may be reset to a different initial sleep window and listen window.

### 15.2.x.x.2.5 Periodic ranging in Sleep Mode

16m-MS performs CDMA code-based Periodic Ranging.

### 15.2.x.x.2.6 CQI Feedback during Sleep Mode

TBD

### 15.2.x.x.2.7 ARQ/HARQ Operations during Sleep Mode

TBD

## 15.2.x.x.2.8 Sleep mode termination

Sleep mode termination can be initiated by either the AMS or the ABS and uses HARQ messages. If AMS-initiated, then the AMS sends a MOB_SLP-REQ message with de-activation request and subsequently the ABS responds with the MOB_SLP-RSP message. The final decision to terminate rests with the ABS. The ABS can also send an unsolicited MOB_SLP-RSP message to de-activate and the MS needs to reply with MOB_SLP-ACK to confirm the de-activation message received. Receiving the MOB_SLP-RSP message with De-activation code causes the AMS to exit Sleep Mode with immediate effect.

============================== End of Proposed Text ===============