| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Parameter Reconfiguration for the Sleep Mode in IEEE 802.16m |  |
| Date Submitted | 2009-03-01 |  |
| Source(s) | Sunggeun Jin, Sungcheol Chang, Won-Ik Kim, and Chulsik Yoon ETRI 161 Gajeong-dong, Yuseong-gu, Daejeon, 305-700, Korea | Voice: +82-42-860-1757 E-mail: sgjin@etri.re.kr |
| Re: | TGm AWD : Contributions on Project 802.16m Amendment Working Document |  |
| Abstract | This contribution proposes parameter reconfiguration for sleep mode |  |
| Purpose | Adopt the proposed text into amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Parameter Reconfiguration for Sleep Mode in IEEE 802.16m

Sunggeun Jin, Sungcheol Chang, Won-Ik Kim, and Chulsik Yoon

ETRI

## 1 Introduction

In the 802.16m, the signaling for sleep mode should support creation and termination for a power saving class. Additionally, it should support reconfiguration of parameter changes for a power saving class. Since an AMS is allowed to have a single power saving class to manage two or more connections having different types of traffic, it is necessary to change PSM related parameters when a power saving class is activated. When a new connection is established in an AMS in the PSM, the AMS should reconfigure on-going power saving class in order to accommodate the new connection. For the purpose, we propose to append new parameters to MOB_SLP-REQ and MOB_SLP-RSP messages.

## 2 Text proposal for parameter reconfiguration

--------------------------------------- Start of Proposed Amendment Text -------------------------------------------------

ABS (or AMS) may request AMS (or ABS) to reconfigure the parameters for sleep mode by transmitting MOB_SLP-REQ message while sleep mode is already activated in the AMS. When ABS or AMS receives MOB_SLP-REQ message, it should reply with MOB_SLP-RSP.

Table xxx – MOB_SLP-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_SLP-REQ_Message_format () { |  |  |
| Sleep_request | 1 |  |
| If ( Sleep_request = 1 ) { |  |  |
| Definition | 1 |  |
| Listening_window_extension_for_HARQ_ | 1 |  |
| Listening_window_extension_for_traffic | 1 |  |
| Implicit_PSC_deactivation | 1 |  |
| Start_frame_number | TBD |  |
| TBD | TBD |  |
| If ( Definition = 1 ) { |  |  |
| TBD | TBD |  |
| } |  |  |
| } |  |  |

Sleep_request

0 = Deactivation of power saving class

1 = Activation/Reconfiguration of power saving class

Definition

0 = Definition of power saving class absent

1 = Definition of power saving class present

Listening_window_extension_for_HARQ

0 = Listening window extension for HARQ retransmission is not supported

1 = Listening window extension for HARQ retransmission is supported

Listening_window_extension_for_traffic

0 = Listening window extension for traffic is not supported

1 = Listening window extension for traffic retransmission is supported

Implicit_PSC_deactivation

0 = Implicit PSC deactivation is not supported

1 = Implicit PSC deactivation is supported

Table xxx – MOB_SLP-RSP

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_SLP-REQ_Message_format () { |  |  |
| Sleep_request_approved | 1 |  |
| If ( Sleep_request_approved = 0 ) { |  | If disapproved, the reasons are provided as follows |
| Definition | 1 |  |
| Listening_window_extension_for_HARQ_approved | 1 |  |
| Listening_window_extension_for_Traffic_approved | 1 |  |
| Implicit_PSC_deactivation_approved | 1 |  |
| Start_frame_number | TBD |  |
| TBD | TBD |  |
| If ( Definition = 1 ) { |  |  |
| TBD | TBD |  |
| } |  |  |
| } |  |  |

Sleep_request_approved

0 = Disapproves activation /deactivation

1 = Approves activation/reconfiguration of power saving class

Definition

0 = Definition of power saving class absent

1 = Definition of power saving class present

Listening_window_extension_for_HARQ_approved

0 = Disapproves listening window extension for HARQ retransmission request

1 = Approves listening window extension for HARQ retransmission request

Listening_window_extension_for_traffic_approved

0 = Disapproves listening window extension for traffic request

1 = Approves listening window extension for traffic retransmission request

Implicit_PSC_deactivation_approved

0 = Disapproves implicit PSC deactivation request

1 = Approves implicit PSC deactivation request

--------------------------------------- End of Proposed Amendment Text -------------------------------------------------