| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Sleep Mode Operations in IEEE 802.16m |  |
| Date Submitted | 2009-03-01 |  |
| Source(s) | Sunggeun Jin, Sungcheol Chang, Won-Ik Kim, and Chulsik Yoon ETRI 161 Gajeong-dong, Yuseong-gu, Daejeon, 305-700, Korea | Voice: +82-42-860-1757 E-mail: sgjin@etri.re.kr |
| Re: | TGm AWD : Contributions on Project 802.16m Amendment Working Document |  |
| Abstract | This contribution proposes signaling for sleep mode in IEEE 802.16m |  |
| Purpose | Adopt the proposed text into amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Sleep Mode Operations in IEEE 802.16m

Sunggeun Jin, Sungcheol Chang, Won-Ik Kim, and Chulsik Yoon

ETRI

## 1 Introduction

The Sleep Mode for the 802.16m is different from that of the 802.16e in several respects as summarized below:

- A single power saving class is allowed for a single MS

- The duration of sleep or listening windows changes depending on traffic patterns

- The characteristics of a power saving class can be reconfigured when necessary.

- The power saving mode can be stopped implicitly

In order to support the changes, we need to define new signaling messages accommodating the requirement. The proposed text gives procedures for the cases that power saving mode begins and stops.

## 2 Text proposal for sleep mode operations

--------------------------------------- Start of Proposed Amendment Text -------------------------------------------------

15.2.x.x Sleep Mode

15.2.x.x.x Introduction

Sleep mode is a state in which an AMS conducts pre-negotiated periods of absence from the serving ABS air interface under the conditions that (1) there is no traffic to be served for a long time; (2) periodic real time traffic or (3) lightly loaded traffic is served; and (4) an AMS scans neighboring ABSs. A single AMS shall support a single power saving class in order to handle all active connections belonging to an AMS. While an AMS is in the sleep mode, the AMS is provided with a series of alternate listening window and sleep windows. The listening window is the time in which the AMS is available to exchange control signaling as well as data between itself and the ABS. Sleep window including listening window is the time when the AMS can power down its device, and hence, it is not possible to exchange control signaling and data.

An AMS (or ABS) may request ABS (or AMS) to initiate sleep mode by transmitting MOB_SLP-REQ message when needed. AMS or ABS shall reply with MOB_SLP-RSP message in response to a MOB_SLP-REQ message. AMS and ABS negotiate the parameters for sleep mode by exchanging MOB_SLP-REQ. Furthermore, AMS and ABS may reconfigure the already-negotiated parameters by exchanging MOB_SLP-REQ/RSP messages even while sleep mode is activated. It is also possible to reconfigure the parameters of on-going power saving class when a new connection is established. In this case, the entity having requested the creation of the connection shall request to do that.

The duration of sleep windows and listening windows may be adjusted within a sleep cycle. For the purpose, AMS and ABS shall negotiate whether such adjustment should be provided or not prior to the beginning of sleep mode. The adjustment includes the operations summarized by:

- If HARQ retransmission is not completed in a listening window, the listening window may be extended until the completion of HARQ retransmission.

- If data transmission is not completed in a listening window, the listening window may be extended until transmitter’s buffer becomes empty.

- If transmission completes early within a listening window, the transmitter may request to terminate the listening window and begin sleep window.

- Activated PSC type-I may be deactivated with new traffic arrival.

Table xxx – MOB_SLP-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_SLP-REQ_Message_format () { |  |  |
| Sleep_request | 1 |  |
| If ( Sleep_request = 1 ) { |  |  |
| Definition | 1 |  |
| Listening_window_extension_for_HARQ_ | 1 |  |
| Listening_window_extension_for_traffic | 1 |  |
| Implicit_PSC_deactivation | 1 |  |
| Start_frame_number | TBD |  |
| TBD | TBD |  |
| If ( Definition = 1 ) { |  |  |
| TBD | TBD |  |
| } |  |  |
| } |  |  |

Sleep_request

0 = Deactivation of power saving class

1 = Activation of power saving class

Definition

0 = Definition of power saving class absent

1 = Definition of power saving class present

Listening_window_extension_for_HARQ

0 = Listening window extension for HARQ retransmission is not supported

1 = Listening window extension for HARQ retransmission is supported

Listening_window_extension_for_traffic

0 = Listening window extension for traffic is not supported

1 = Listening window extension for traffic retransmission is supported

Implicit_PSC_deactivation

0 = Implicit PSC deactivation is not supported

1 = Implicit PSC deactivation is supported

Table xxx – MOB_SLP-RSP

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_SLP-REQ_Message_format () { |  |  |
| Sleep_request_approved | 1 |  |
| If ( Sleep_request_approved = 0 ) { |  | If disapproved, the reasons are provided as follows |
| Definition | 1 |  |
| Listening_window_extension_for_HARQ_approved | 1 |  |
| Listening_window_extension_for_Traffic_approved | 1 |  |
| Implicit_PSC_deactivation_approved | 1 |  |
| Start_frame_number | TBD |  |
| TBD | TBD |  |
| If ( Definition = 1 ) { |  |  |
| TBD | TBD |  |
| } |  |  |
| } |  |  |

Sleep_request_approved

0 = Disapproves activation /deactivation

1 = Approves activation of power saving class

Definition

0 = Definition of power saving class absent

1 = Definition of power saving class present

Listening_window_extension_for_HARQ_approved

0 = Disapproves listening window extension for HARQ retransmission request

1 = Approves listening window extension for HARQ retransmission request

Listening_window_extension_for_traffic_approved

0 = Disapproves listening window extension for traffic request

1 = Approves listening window extension for traffic retransmission request

Implicit_PSC_deactivation_approved

0 = Disapproves implicit PSC deactivation request

1 = Approves implicit PSC deactivation request

--------------------------------------- End of Proposed Amendment Text -------------------------------------------------