| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Termination of Listening Window for the Sleep Mode in IEEE 802.16m |  |
| Date Submitted | 2009-03-01 |  |
| Source(s) | Sunggeun Jin, Sungcheol Chang, Won-Ik Kim, and Chulsik Yoon ETRI 161 Gajeong-dong, Yuseong-gu, Daejeon, 305-700, Korea | Voice: +82-42-860-1757 E-mail: sgjin@etri.re.kr |
| Re: | TGm AWD : Contributions on Project 802.16m Amendment Working Document |  |
| Abstract | This contribution proposes signaling in order to terminate listening window for sleep mode in the 802.16m. |  |
| Purpose | Adopt the proposed text into amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Termination of Listening Window for the Sleep Mode in IEEE 802.16m

Sunggeun Jin, Sungcheol Chang, Wonik Kim, and Chulsik Yoon

ETRI

## 1 Introduction

The 802.16m SDD signifies that “IEEE 802.16m provides a framework for dynamically adjusting the duration of sleep windows and listening windows within a sleep cycle.” For the purpose, we provide a signaling in order to terminate listening window within a sleep cycle when there is no buffered traffic in both ABS and AMS.

## 2 Text proposal for termination of listening window

--------------------------------------- Start of Proposed Amendment Text -------------------------------------------------

When there is no traffic buffered in either ABS or AMS for transmission, each ABS or AMS may transmit MOB_SLP-REQ message to its correspondent in order to terminate listening window. Upon reception of the MOB_SLP-REQ message, the correspondent shall begin sleep window and reply with MOB_SLP-RSP message.

Table xxx – MOB_SLP-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_SLP-REQ_Message_format () { |  |  |
| Operation | TBD |  |
| if ( Operation = 0bxx ) { |  | If the operation indicates listening window termination, detailed parameters are provided below |
| Start_frame_number | TBD | The number of frame when the listening window is terminated |
| } |  |  |
| } |  |  |

Operation

0bxx = indicates the listening window termination request

Table xxx – MOB_SLP-RSP

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_SLP-REQ_Message_format () { |  |  |
| Operation_approved | TBD |  |
| } |  |  |

Operation_approved

0bxx = indicates the listening window termination request is approved.

Otherwise = indicates the listening window termination request is not approved

--------------------------------------- End of Proposed Amendment Text -------------------------------------------------