| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of MIMO for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Alexei Davydov, Guangjie Li, Yang-seok, Gregory Morozov Intel Corporation David Mazzarese, Bruno Clerckx , Jiann-An Tsai, Jerry Pi, Heewon Kang, Hokyu Choi Samsung Electronics Yang Tang, Yang Tang, Zhigang Rong, Jianmin Lu Huawei Technologies | alexei.davydov@intel.com d.mazzarese@samsung.com YTang@huawei.com |
| Re: | TGm MIMO DG on MIMO Draft Amendment text |  |
| Abstract | The contribution proposes the text of MIMO section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of MIMO for the IEEE 802.16m Amendment

Alexei Davydov, Guangjie Li, Yang-seok, Gregory Morozov

Intel Corporation

David Mazzarese, Bruno Clerckx , Jiann-An Tsai, Jerry Pi, Heewon Kang, Hokyu Choi

Samsung Electronics

Yang Tang

Huawei Technologies

- Introduction

The contribution proposes the text of MIMO subclause to be included in the 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D7 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

- Modifications to the SDD text

The text proposed in this contribution is based on subclauses 11.8 and 11.12 in the IEEE 802.16m SDD [3]. The modifications to the SDD text are summarized below:

- Changed terminology … used in [ ][ ] as follows:

- Inserted an introduction subclause

- Added the almost same text

Updated the section of

- Left the section of … as TBD:

- Left the section of … as TBD:

- Not included the section of … of [ ]:

- Set …:

We propose to set…

- Limited the…, but left … as TBD:

It is proposed that…

Lastly, it is noted that, even though...

- References

[1] IEEE P802.16 Rev2/D7, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r5, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

Insert a new section 15:

- Advanced Air Interface

  - Physical layer

### Introduction

### OFDMA symbol description, symbol parameters and transmitted signal

### Frame Structure

### Reserved

### Downlink physical structure

### Downlink physical control

### Downlink MIMO transmission schemes

#### Downlink MIMO architecture and data processing

#### Transmission schemes for data channels

        - Encoding, precoding and mapping of SU-MIMO

        - Encoding, precoding and mapping of MU-MIMO

        - Mapping of data subcarriers

        - Mapping of pilot subcarriers

        - Usage of MIMO modes

        - Feedback mechanisms and operation

          - Downlink post-processing CINR measurement feedback

          - MIMO mode selection feedback

          - Types of MIMO feedback

          - Quantized MIMO feedback for closed-loop transmit precoding

            - Quantized feedback modes

            - Base mode for codebook-based feedback

The base codebook may be used for the feedback to support transmit precoding on the DL-MIMO, as instructed by the BS.

- The base codebook is a unitary codebook. A codebook is a unitary codebook if each of its matrices consists of columns of a unitary matrix.

The MS selects its preferred matrix from the base codebook based on the channel measurements. The MS feedbacks the index of the preferred codeword, and the BS computes the precoder W according to the index. Both BS and MS use the same codebook for correct operation.

- For the base mode, the PMI feedback from a mobile station shall represent an entry of the base codebook, where the base codebooks are defined as follows for two, four, and eight transmit antennas at the BS.

- The notation C(Nt, Mt, NB) denotes the codebook, which consists of 2NB complex matrices of dimension Nt by Mt, and Mt denotes the number of streams.

- The notation C(Nt, Mt, NB, i) denotes the i-th codebook entry of C(Nt, Mt, NB).

              - Base codebook for two transmit antennas

                - SU-MIMO base codebook

- The codebook for two transmit antenna is constructed using a similar methodology as described in section 8.4.11.15 of WirelessMAN-OFDMA, with the exception that the first codeword v1 is defined as an Nt by Mt unitary matrix

- v1=1Nt11⋯1ej2πNtej2πNt2j2πNtMt⋮⋮⋱ej2πNtNt−1ej2πNt2Nt−1ej2πNtMtNt−1			()

- All codeword matrices vi for i = 2,3…,2NB in V(Nt, Mt, NB) codebook can be derived from the first codeword matrix v1 using the following equations

- vi=HsQi−1HHsv1,				()

- vi=vi⋅e−jϕ1,					()

- where phase φ1 is a phase of the first entry of the first column of codeword vi. Operation H generates unitary matrix as follows

- H(v)=I, v=e1I−2wwHwHw, otherwise ,			()

- where w=v−e1, e1=1,0,...,0T.

- Matrix Qi is defined as following

- Qi=diage2π2Lu1i,e2π2Lu2i,...,e2π2LuNti,			()

- The parameters for generation of the codebooks for two transmit antenna Nt = 2 and number of streams Mt =1, 2 are listed in Table 1.

- Table 1. – Generating parameters for two transmit antenna codebook

| Nt | Mt | NB | L | u=u1,u2,...,uNt in Qi(u) | s in H(s) |
| --- | --- | --- | --- | --- | --- |
| 2 | 1 | 3 | 3 | [1, 2] | [1, 0] |
| 2 | 2 | 2 | 3 |  |  |

- The indexes from 4 to 7 are not used in 3-bits downlink PMI feedback for Mt =2 codebook.

                - MU-MIMO base codebook

The base codebook for MU-MIMO is same as the rank 1 base codebook for SU-MIMO, defined in 3.3.1.1.6.4.2.1.1.

              - Base codebook for four transmit antennas

                - SU-MIMO base codebook

The base codebooks of SU-MIMO with four transmit antennas consist of rank-1 codebook C(4,1,6), rank-2 codebook C(4,2,6), rank-3 codebook C(4,3,4) and rank-4 codebook C(4,4,3). Table 2, 3, 4 and 5 are included to illustrate the rank-1,2,3,4 base codebooks.

Table 2   C(4,1,6)

| # Binary Index | # m | C(4,1,6,m) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 000000 | 0 | 0.5000 | -0.5000 | 0.5000 | -0.5000 |
| 000001 | 1 | -0.5000 | -0.5000 | 0.5000 | 0.5000 |
| 000010 | 2 | -0.5000 | 0.5000 | 0.5000 | -0.5000 |
| 000011 | 3 | 0.5000 | 0.0000 - 0.5000i | 0.5000 | 0.0000 - 0.5000i |
| 000100 | 4 | -0.5000 | 0.0000 - 0.5000i | 0.5000 | 0.0000 + 0.5000i |
| 000101 | 5 | -0.5000 | 0.0000 + 0.5000i | 0.5000 | 0.0000 - 0.5000i |
| 000110 | 6 | 0.5000 | 0.5000 | 0.5000 | 0.5000 |
| 000111 | 7 | 0.5000 | 0.0000 + 0.5000i | 0.5000 | 0.0000 + 0.5000i |
| 001000 | 8 | 0.5000 | 0.5000 | 0.5000 | -0.5000 |
| 001001 | 9 | 0.5000 | 0.0000 + 0.5000i | -0.5000 | 0.0000 + 0.5000i |
| 001010 | 10 | 0.5000 | -0.5000 | 0.5000 | 0.5000 |
| 001011 | 11 | 0.5000 | 0.0000 - 0.5000i | -0.5000 | 0.0000 - 0.5000i |
| 001100 | 12 | 0.5000 | 0.3536 + 0.3536i | 0.0000 + 0.5000i | -0.3536 + 0.3536i |
| 001101 | 13 | 0.5000 | -0.3536 + 0.3536i | 0.0000 – 0.5000i | 0.3536 + 0.3536i |
| 001110 | 14 | 0.5000 | -0.3536 - 0.3536i | 0.0000 + 0.5000i | 0.3536 - 0.3536i |
| 001111 | 15 | 0.5000 | 0.3536 - 0.3536i | 0.0000 – 0.5000i | -0.3536 - 0.3536i |
| 010000 | 16 | 0.5000 | -0.4619 - 0.1913i | 0.3536 + 0.3536i | -0.1913 - 0.4619i |
| 010001 | 17 | 0.3117 | 0.6025 + 0.1995i | -0.4030 – 0.4903i | -0.1122 - 0.2908i |
| 010010 | 18 | 0.3117 | -0.6025 - 0.1995i | -0.1122 – 0.2908i | 0.4030 + 0.4903i |
| 010011 | 19 | 0.3058 | 0.1901 - 0.6052i | 0.1195 + 0.2866i | 0.4884 - 0.4111i |
| 010100 | 20 | 0.5000 | -0.1913 + 0.4619i | -0.3536 – 0.3536i | 0.4619 - 0.1913i |
| 010101 | 21 | 0.5000 | 0.1913 - 0.4619i | -0.3536 – 0.3536i | -0.4619 + 0.1913i |
| 010110 | 22 | 0.5000 | 0.4619 + 0.1913i | 0.3536 + 0.3536i | 0.1913 + 0.4619i |
| 010111 | 23 | 0.3082 | 0.0104 + 0.3151i | 0.4077 + 0.4887i | -0.4783 + 0.4145i |
| 011000 | 24 | 0.3117 | 0.3573 - 0.2452i | 0.6025 – 0.1995i | -0.1578 + 0.5360i |
| 011001 | 25 | 0.3117 | 0.2452 + 0.3573i | -0.6025 + 0.1995i | 0.5360 + 0.1578i |
| 011010 | 26 | 0.3082 | -0.3666 + 0.2426i | 0.6092 – 0.1842i | 0.1615 - 0.5298i |
| 011011 | 27 | 0.3117 | -0.2452 - 0.3573i | -0.6025 + 0.1995i | -0.5360 - 0.1578i |
| 011100 | 28 | 0.3117 | 0.4260 + 0.0793i | 0.1995 + 0.6025i | 0.2674 + 0.4906i |
| 011101 | 29 | 0.3117 | -0.0793 + 0.4260i | -0.1995 – 0.6025i | 0.4906 - 0.2674i |
| 011110 | 30 | 0.3117 | -0.4260 - 0.0793i | 0.1995 + 0.6025i | -0.2674 - 0.4906i |
| 011111 | 31 | 0.3117 | 0.0793 - 0.4260i | -0.1995 – 0.6025i | -0.4906 + 0.2674i |
| 100000 | 32 | 0.5636 | -0.3332 - 0.2672i | 0.1174 + 0.5512i | -0.3308 - 0.2702i |
| 100001 | 33 | 0.5587 | 0.3361 + 0.2735i | -0.3361 – 0.2735i | -0.1135 - 0.5471i |
| 100010 | 34 | 0.5587 | -0.3361 - 0.2735i | -0.1135 – 0.5471i | 0.3361 + 0.2735i |
| 100011 | 35 | 0.5587 | 0.2735 - 0.3361i | 0.1135 + 0.5471i | 0.2735 - 0.3361i |
| 100100 | 36 | 0.3082 | -0.4887 + 0.4077i | -0.6092 – 0.1842i | 0.2837 - 0.1205i |
| 100101 | 37 | 0.5636 | 0.2673 - 0.3331i | -0.1222 – 0.5501i | -0.2673 + 0.3331i |
| 100110 | 38 | 0.5636 | 0.3691 + 0.5142i | 0.3331 + 0.2673i | 0.0862 + 0.3032i |
| 100111 | 39 | 0.5587 | -0.2990 + 0.0880i | 0.3361 + 0.2735i | -0.5216 + 0.3616i |
| 101000 | 40 | 0.5587 | 0.0880 - 0.2990i | 0.3361 – 0.2735i | -0.3616 + 0.5216i |
| 101001 | 41 | 0.5587 | 0.2990 + 0.0881i | -0.3362 + 0.2735i | 0.5216 + 0.3616i |
| 101010 | 42 | 0.5587 | -0.0880 + 0.2990i | 0.3361 – 0.2735i | 0.3616 - 0.5216i |
| 101011 | 43 | 0.5587 | -0.2990 - 0.0880i | -0.3361 + 0.2735i | -0.5216 - 0.3616i |
| 101100 | 44 | 0.5636 | 0.2741 - 0.1559i | 0.2672 + 0.3332i | 0.1081 + 0.6236i |
| 101101 | 45 | 0.5636 | 0.1559 + 0.2741i | -0.2672 – 0.3332i | 0.6236 - 0.1081i |
| 101110 | 46 | 0.5587 | -0.2737 + 0.1492i | 0.2735 + 0.3361i | -0.1132 - 0.6245i |
| 101111 | 47 | 0.5587 | -0.1492 - 0.2737i | -0.2735 - 0.3361i | -0.6245 + 0.1132i |
| 110000 | 48 | 0.5000 | -0.4619 + 0.1913i | 0.3536 - 0.3536i | -0.1913 + 0.4619i |
| 110001 | 49 | 0.3117 | 0.4030 + 0.4903i | -0.6025 - 0.1995i | -0.1122 - 0.2908i |
| 110010 | 50 | 0.3117 | -0.4029 - 0.4904i | -0.1184 - 0.2883i | 0.6067 + 0.1865i |
| 110011 | 51 | 0.3082 | 0.4887 - 0.4077i | 0.1205 + 0.2837i | 0.1842 - 0.6092i |
| 110100 | 52 | 0.5000 | 0.1913 + 0.4619i | -0.3536 + 0.3536i | -0.4619 - 0.1913i |
| 110101 | 53 | 0.5000 | -0.1913 - 0.4619i | -0.3536 + 0.3536i | 0.4619 + 0.1913i |
| 110110 | 54 | 0.5000 | 0.4619 - 0.1913i | 0.3536 - 0.3536i | 0.1913 - 0.4619i |
| 110111 | 55 | 0.3117 | -0.2452 + 0.3573i | 0.6025 + 0.1995i | -0.5360 + 0.1578i |
| 111000 | 56 | 0.3117 | 0.3117 - 0.0000i | 0.4030 - 0.4903i | -0.4030 + 0.4903i |
| 111001 | 57 | 0.3117 | -0.0000 + 0.3117i | -0.4030 + 0.4903i | 0.4903 + 0.4030i |
| 111010 | 58 | 0.3082 | -0.3152 - 0.0036i | 0.4076 - 0.4888i | 0.4040 - 0.4872i |
| 111011 | 59 | 0.3082 | 0.0036 - 0.3152i | -0.4076 + 0.4888i | -0.4872 - 0.4040i |
| 111100 | 60 | 0.3117 | 0.2204 + 0.2204i | 0.4903 + 0.4030i | 0.0618 + 0.6317i |
| 111101 | 61 | 0.3117 | -0.2204 + 0.2204i | -0.4903 - 0.4030i | 0.6317 - 0.0618i |
| 111110 | 62 | 0.3082 | -0.2154 - 0.2302i | 0.4887 + 0.4077i | -0.0451 - 0.6313i |
| 111111 | 63 | 0.3082 | 0.2254 - 0.2204i | -0.4888 - 0.4076i | -0.6302 + 0.0588i |

Table 3   C(4,2,6)

| # C(4,2,6,m)=C(4,1,6,i)C(4,1,6,j) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary index | m | i,j | Binary index | m | i,j | Binary index | m | i,j |  | Binary index | m | i,j |
| 000000 | 0 | 6,0 | 010000 | 16 | 0,4 | 100000 | 32 | 12,14 | 110000 |  | 48 | 32,34 |
| 000001 | 1 | 6,1 | 010001 | 17 | 0,5 | 100001 | 33 | 12,15 | 110001 |  | 49 | 34,35 |
| 000010 | 2 | 6,2 | 010010 | 18 | 0,33 | 100010 | 34 | 12,31 | 110010 |  | 50 | 35,7 |
| 000011 | 3 | 0,1 | 010011 | 19 | 1,3 | 100011 | 35 | 13,14 | 110011 |  | 51 | 40,11 |
| 000100 | 4 | 0,2 | 010100 | 20 | 1,34 | 100100 | 36 | 5,23 | 110100 |  | 52 | 41,43 |
| 000101 | 5 | 1,2 | 010101 | 21 | 1,55 | 100101 | 37 | 14,15 | 110101 |  | 53 | 44,46 |
| 000110 | 6 | 7,4 | 010110 | 22 | 2,3 | 100110 | 38 | 14,47 | 110110 |  | 54 | 45,47 |
| 000111 | 7 | 7,5 | 010111 | 23 | 8,9 | 100111 | 39 | 17,2 | 110111 |  | 55 | 49,6 |
| 001000 | 8 | 3,4 | 011000 | 24 | 2,39 | 101000 | 40 | 17,3 | 111000 |  | 56 | 52,53 |
| 001001 | 9 | 3,5 | 011001 | 25 | 8,11 | 101001 | 41 | 18,19 | 111001 |  | 57 | 56,10 |
| 001010 | 10 | 6,4 | 011010 | 26 | 8,27 | 101010 | 42 | 18,6 | 111010 |  | 58 | 56,58 |
| 001011 | 11 | 6,5 | 011011 | 27 | 9,10 | 101011 | 43 | 24,9 | 111011 |  | 59 | 57,11 |
| 001100 | 12 | 7,1 | 011100 | 28 | 9,42 | 101100 | 44 | 25,10 | 111100 |  | 60 | 57,59 |
| 001101 | 13 | 7,2 | 011101 | 29 | 10,11 | 101101 | 45 | 28,13 | 111101 |  | 61 | 60,14 |
| 001110 | 14 | 8,10 | 011110 | 30 | 10,43 | 101110 | 46 | 29,14 | 111110 |  | 62 | 61,15 |
| 001111 | 15 | 13,15 | 011111 | 31 | 12,13 | 101111 | 47 | 30,15 | 111111 |  | 63 | 61,63 |

Table 4   C(4,3,4)

| Binary index | m | i,j,k | Binary index | m | i,j,k | Binary index | m | i,j,k |  | Binary Index | m | i,j,k |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000000 | 0 | 6,0,1 | 000100 | 4 | 7,3,4 | 001000 | 8 | 6,0,4 | 001100 |  | 12 | 8,9,10 |
| 000001 | 1 | 6,0,2 | 000101 | 5 | 7,3,5 | 001001 | 9 | 6,4,5 | 001101 |  | 13 | 8,10,11 |
| 000010 | 2 | 6,1,2 | 000110 | 6 | 7,4,5 | 001010 | 10 | 7,3,1 | 001110 |  | 14 | 12,13,15 |
| 000011 | 3 | 0,1,2 | 000111 | 7 | 3,4,5 | 001011 | 11 | 7,1,2 | 001111 |  | 15 | 13,14,15 |

Table 5   C(4,4,3)

| Binary index | m | i,j,k,p | Binary index | m | i,j,k,p | Binary index | m | i,j,k,p |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000000 | 0 | 6,0,1,2 | 000010 | 2 | 6,0,4,5 | 000100 | 4 | 8,9,10,11 |
| 000001 | 1 | 7,3,4,5 | 000011 | 3 | 7,3,1,2 | 000101 | 5 | 12,13,14,15 |

The chordal distance property of C(4,1,6) is shown in Fig 1. It is noted that the chordal distance between the codewords of different codeword clusters is much larger than the chordal distance between the codewords in Layer 2 and the codeword in Layer 1 of the same codeword cluster.

Figure 1  Chordal distance map of C(4,1,6)

                - MU-MIMO base codebook

The base codebook for MU-MIMO is same as the rank 1 base codebook for SU-MIMO, defined in 3.3.1.1.6.4.2.2.1.

              - Base codebook for eight transmit antennas

                - SU-MIMO base codebook

- The base codebook is constructed from two matrices V8(:,:,1) and V8(:,:,2), which are constructed as described below.

- Define

- ()

- ()

- The two rank-8 matrices used for rank-2 to rank-8 transmission for SU-MIMO are

- ()

- ()

- The third base matrix, V8(:,:,3), is used for rank-1 transmission for MU-MIMO. The j-th column vector of the base matrix V8(:,:,3) is given by

- where the set θj, j=1,...,16, is given by

- The 4bits 8Tx base codebook is constructed from the three 8×8 base matrices and is specified in Tables 6 and 7. Note that only the column indices of the corresponding base matrices are shown in Table 7 for brevity.

Table 6 – Rank 1 of SU MIMO 4bit 8Tx base codebook

| Codebook Matrix Index (CMI) | Base Matrix | C(8,1,4) |
| --- | --- | --- |
| 1 | V8(:,:,3) | V8(:,1,3) |
| 2 |  | V8(:,2,3) |
| 3 |  | V8(:,3,3) |
| 4 |  | V8(:,4,3) |
| 5 |  | V8(:,5,3) |
| 6 |  | V8(:,6,3) |
| 7 |  | V8(:,7,3) |
| 8 |  | V8(:,8,3) |
| 9 |  | V8(:,9,3) |
| 10 |  | V8(:,10,3) |
| 11 |  | V8(:,11,3) |
| 12 |  | V8(:,12,3) |
| 13 |  | V8(:,13,3) |
| 14 |  | V8(:,14,3) |
| 15 |  | V8(:,15,3) |
| 16 |  | V8(:,16,3) |

Table 7 – Ranks 2 to 8 of SU MIMO 4bit 8Tx base codebook

| Codebook Matrix Index (CMI) | Base Matrix | C(8,2,4) | C(8,3,4) | C(8,4,4) | C(8,5,4) | C(8,6,4) | C(8,7,4) | C(8,8,4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 1 5 | 1 3 5 | 1537 | 12357 | 123567 | 1234567 | 12345678 |
| 2 |  | 2 6 | 2 4 6 | 2648 | 12468 | 124568 | 1234568 | n/a |
| 3 |  | 3 7 | 2 3 7 | 3726 | 23467 | 234678 | 1234678 | n/a |
| 4 |  | 4 8 | 1 4 8 | 4815 | 13458 | 134578 | 1234578 | n/a |
| 5 |  | 5 3 | 3 5 7 | 5372 | 23567 | 234567 | 2345678 | n/a |
| 6 |  | 4 6 | 4 6 8 | 6481 | 14568 | 134568 | 1345678 | n/a |
| 7 |  | 2 7 | 2 6 7 | 7264 | 24678 | 124678 | 1245678 | n/a |
| 8 |  | 8 1 | 1 5 8 | 8153 | 13578 | 123578 | 1235678 | n/a |
| 9 |  | 1 3 | 1 2 3 | 1234 | 12345 | 123456 | 1234567 | 12345678 |
| 10 |  | 2 4 | 1 2 4 | 1246 | 12456 | 124567 | 1245678 | n/a |
| 11 |  | 2 3 | 2 3 4 | 2437 | 23478 | 123478 | 1234578 | n/a |
| 12 |  | 1 4 | 1 3 4 | 1348 | 13478 | 134678 | 1234678 | n/a |
| 13 |  | 5 8 | 5 7 8 | 3578 | 23578 | 235678 | 1235678 | n/a |
| 14 |  | 6 7 | 6 7 8 | 4678 | 14678 | 145678 | 1345678 | n/a |
| 15 |  | 5 7 | 5 7 6 | 5678 | 35678 | 345678 | 2345678 | n/a |
| 16 |  | 6 8 | 5 6 8 | 1568 | 13568 | 123568 | 1234568 | n/a |

                - MU-MIMO base codebook

The base codebook for MU-MIMO is same as the rank 1 base codebook for SU-MIMO, defined in section 15.3.7.2.6.4.2.3.

- 15.3.7.2.6.4.2.4. Codebook subset selection

- 15.3.7.2.6.4.2.4.1. OL MIMO subset

- 15.3.7.2.6.4.2.4.1.1. OL SU-MIMO subset for two transmit antennas

[TBD]

- 15.3.7.2.6.4.2.4.1.2. OL SU-MIMO subset for four transmit antennas

[TBD]

- 15.3.7.2.6.4.2.4.1.3. OL SU-MIMO subset for eight transmit antennas

[TBD]

- 15.3.7.2.6.4.2.4.2. CL SU-MIMO subset

- 15.3.7.2.6.4.2.4.2.1. CL SU-MIMO subset for four transmit antennas

Codebook subset selection for four transmit antennas is specified in Table 8.

Table 8 - Subset selection of the base codebook for four transmit antennas

| Rank | One | Two | Three | Four |
| --- | --- | --- | --- | --- |
| Subset selection | C(4,1,6,m) m = 0 to15 | C(4,2,6,m) m = 0 to15 | C(4,3,4,m) m = 0 to15 | C(4,4,3,m) m = 0 to 5 |

- 15.3.7.2.6.4.2.4.3. CL MU-MIMO subset

[TBD]

    - Uplink physical structure

    - Uplink physical structure

    - Uplink MIMO transmission schemes

      - Uplink MIMO architecture and data processing

      - Transmission schemes for data channels

      - Codebook for closed-loop transmit precoding

              - Base codebook for two transmit antennas

                - SU-MIMO base codebook

The base codebook of uplink 2 Tx is the same as the downlink 2 Tx base codebook (SU MIMO base codebook), defined in 3.3.1.1.6.4.2.1.

                - MU-MIMO base codebook

The base codebook for UL collaborative spatial multiplexing MIMO is same as base codebook for SU-MIMO, defined in 3.3.4.3.1.1.1.1.1

              - Base codebook for four transmit antennas

                - SU-MIMO base codebook

- The codebooks for UL MIMO with four transmit antenna MS are constructed using the methodology described in section 3.3.1.1.6.4.2.1 for two transmit antenna case.

- The parameters for the generation of codebooks for four transmit antenna Nt = 4 and number of streams Mt =1, 2, 3, 4 are listed in Table 9.

- Table 9. – Generating parameters for two transmit antenna codebook

| Nt | Mt | NB | L | u=u1,u2,...,uNt in Qi(u) | s in H(s) |
| --- | --- | --- | --- | --- | --- |
| 4 | 1 | 6 | 6 | [18, 55, 22, 6] | [1, 0, 0, 0] |
| 4 | 2 | 6 | 6 |  |  |
| 4 | 3 | 6 | 6 |  |  |
| 4 | 4 | 6 | 6 |  |  |

                - MU-MIMO base codebook

The base codebook for UL collaborative spatial multiplexing MIMO is same as the base codebook for UL SU-MIMO, defined in 3.3.4.3.1.1.1.2.1

-------------------------------  Text End  ---------------------------------------------------