| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Modifications on Signaling Protocol of DL/UL Subchannelization for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Taeyoung Kim, Jeongho Park, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics | E-mail: ty33.kim@samsung.com |
| Re: | IEEE 802.16m-09/0012, “Call for Comments on Amendment Working Document”. |  |
| Abstract | The contribution proposes modifications on IEEE 802.16m amendment working document IEEE 80216m-09/0010. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Modifications on Signaling Protocol of DL/UL Subchannelization for IEEE 802.16m Amendment

Taeyoung Kim, Jeongho Park, Jaeweon Cho, Hokyu Choi, Heewon Kang

Samsung Electronics

# Introduction

In this contribution, the signaling protocol for DL subchannelization in the current Amendment working document [1] is summarized and the revised signaling protocol is proposed to reduce the number of indication bits. Based on the modified signaling protocol, some equations and description for DL subchannelization are modified in the current Amendment working document.

# Current signaling protocol in Amendment working document [1]

In the current Amendment working document [1], many kinds of signaling protocols for DL/UL subchannelization are defined as shown in Table 1. Using these signaling indication fields, the allocation of subband, frequency partitioning for FFR operation are performed. The assumptions considered here are as follows:

- The number of frequency partitions is 4.

- The size of frequency partitions, FPi (i0) is equal.

- The number of subbands allocated to FPi (i0) is equal.

Table 1 Signaling protocol defined in current Amendment document

|  | Field |  | Bits | Description | Unit | Container |
| --- | --- | --- | --- | --- | --- | --- |
| Multi-cell | Subband Allocation Count (SAC or USAC) |  | 5 (TBD) | # of PRUs allocated to subbands, LSB | SB | (P- /S-) BCH |
|  | Freq. Partition Configuration (FPC or UFPC) |  | 12 |  |  | (P- /S-) BCH |
|  |  | Freq. Partition Count (FPCT) | 2 | # of freq. partitions (1~4) | - |  |
|  |  | Freq. Partition Size (FPS) | 6 | # of PRUs allocated to FPi, i>0 | MB |  |
|  |  | Freq. Partition Subband Count (FPSC) | 4 | # of subbands allocated to FPi, i>0 | SB |  |

# Proposed signaling protocol

In order to reduce the number of indication bits for DL/UL subchannelization, the concrete and feasible operation scenario is considered. In addition, the unnecessary assumptions defined in above section are removed, that is, the last assumption above causes the increase of the number of indication bits. The remained assumptions are still considered. In Table 2, the revised signaling protocol for DL/UL subchannelization is proposed.

Table 2 Proposed signaling protocol for DL subchannelization

|  | Field | Bits | Description | Unit | Container |
| --- | --- | --- | --- | --- | --- |
| Multi-cell | Subband Allocation Count (SAC) | 4 | # of PRUs allocated to subbands | SB | TBD |
|  | Freq. Partition Configuration (FPC) | 4 | Freq. partition configuration set | - | TBD |
|  | Freq. Partition Subband Count (FPSC) | 3 | # of subbands allocated to FPi, i>0 | SB | TBD |

- Subband Allocation Count (SAC) : 4 bits

Subband allocation count (SAC) field indicates the number of subbands allocated. This information is applied for all system bandwidths (e.g. 5~20MHz). For 20MHz bandwidth, the total number of PRUs is 96, which means the total number of subbands is 24. But, it is not necessary to indicate the all cases. As shown in Table 3, the number of bits required for SAC indication field can be reduced by mapping.

Table 3 Example of mapping between SAC and the number of subbands allocated

| SAC (4bits) | # of subbands allocated (KSB) | SAC (4bits) | # of subbands allocated (KSB) |
| --- | --- | --- | --- |
| 0 | 0 | 8 | 10 |
| 1 | 1 | 9 | 12 |
| 2 | 2 | 10 | 14 |
| 3 | 3 | 11 | 16 |
| 4 | 4 | 12 | 18 |
| 5 | 5 | 13 | 20 |
| 6 | 6 | 14 | 22 |
| 7 | 8 | 15 | 24 |

- Frequency Partition Configuration (FPC) : 4 bits

Frequency partition configuration field give the information about the frequency partitioning such as the number of frequency partitions and the size of each frequency partition. In the signaling protocol defined in Amendment working document, the explicit fields are given to divide the frequency partition, so the number of required indication bits for frequency partitioning is unnecessarily large. As shown in Table 4, it is available to give the information on frequency partition configuration with only small number of indication bits using the predetermined frequency partition set.

Table 4 Example of mapping between FPC and frequency partitioning

| FPC (4 bits) | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 3/6 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 5 | 9 : 1 : 1 : 1 | 4 | NPRU * 9/12 | NPRU * 1/12 |
| 6-15 | Reserved |  |  |  |

- Frequency Partition Subband Count (FPSC) : 3 bits

This is about the number of subbands for frequency partitions besides FP0. The maximum size of frequency partitions except FP0 is at least one third of total frequency band. So, the number of indication bits for FPSC field can be reduced compared to the case of FP0.

# Proposed Amendment Text

Adopt the following text changes for [1]:

Original Amendment text is shown in black. New proposed text is in blue. Removed Amendment text is shown in strike out red text.

------------------------------------------- Text Start ---------------------------------------------------------

[Remedy-1: Change the text from line 2 to 3 on the page 25, in 15.3.5.2.1, as follows:]

A 54 or 3-bit (TBD) field called Subband Allocation Count (SAC) field determines the value of KSB depending on system bandwidth.

[Remedy-2: Insert the table in line 8 on the page 25, in 15.3.5.2.1, as follows:]

Table XXX - Mapping between SAC and KSB for 10MHz or 20MHz

| SAC | # of subbands allocated (KSB) | SAC | # of subbands allocated (KSB) |
| --- | --- | --- | --- |
| 0 | 0 | 8 | 10 |
| 1 | 1 | 9 | 12 |
| 2 | 2 | 10 | 14 |
| 3 | 3 | 11 | 16 |
| 4 | 4 | 12 | 18 |
| 5 | 5 | 13 | 20 |
| 6 | 6 | 14 | 22 |
| 7 | 8 | 15 | 24 |

Table XXX - Mapping between SAC and KSB for 5 MHz

| SAC | # of subbands allocated (KSB) | SAC | # of subbands allocated (KSB) |
| --- | --- | --- | --- |
| 0 | 0 | 4 | 4 |
| 1 | 1 | 5 | 5 |
| 2 | 2 | 6 | 6 |
| 3 | 3 | 7 | N.A |

[Remedy-3: Change the text from line 13 to 14 on the page 25, in 15.3.5.2.1, as follows:]

Figure 399 illustrates the PRU to PRUSB and PRUMB mapping for a 5 MHz bandwidth with SACKSB equal to 3.

[Remedy-4: Change the text from line 52 on the page 26, in 15.3.5.2.1, as follows:]

Figure 399—PRU to PRUSB and PRUMB mapping for BW=5 MHz, SACKSB=3

[Remedy-5: Change the text from line 54 on the page 27, in 15.3.5.2.2, as follows:]

Figure 400—Mapping from PRUs to PRUSB and PPRUMB for BW=5 MHz, SACKSB=3

[Remedy-6: Change the equation (179) from line 15 to 16 on the page 28, in 15.3.5.2.3, as follows:]

[Remedy-7: Change the text in line 46 on the page 28, in 15.3.5.2.3, as follows:]

Figure 401 depicts the frequency partitioning BW=5 MHz, SACKSB=3, FPCT=2, FPS=12, and FPSC=1.

[Remedy-8: Change the text from line 64 on the page 27 to line 6 on the page 28, in 15.3.5.2.3, as follows:]

The frequency partition configuration is transmitted in the BCH in a 12 4(or 3)-bit called the Frequency Partition Configuration (FPC) depending on system bandwidth. Frequency Partition Count (FPCT) defines the number of frequency partitions. Frequency Partition Size (FPSi) defines the number of PRUs allocated to FPi. FPCT and FPSi are determined from FPC as shown in Table YYY. The FPC consists of a Frequency Partition Count (FPCT), Frequency Partition Size (FPS) and Frequency Partition Subband Count (FPSC).  The first two bits carry the FPCT that defines the number of frequency partitions (1 to 4).  The following 6 bits carry the FPS that defines the number of PRUs allocated to FPi, i>0 in the number of minibands (N2). The remaining 43 bits carry Frequency Partition Subband Count (FPSC) that define the number of subbands allocated to FPi, i>0.

[Remedy-9: Insert the table in line 7 on the page 28, in 15.3.5.2.3, as follows:]

Table YYY– Mapping between FPC and frequency partitioning for 10MHz or 20MHz

| FPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 3/6 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 5 | 9 : 1 : 1 : 1 | 4 | NPRU * 9/12 | NPRU * 1/12 |
| 6-15 | Reserved |  |  |  |

Table YYY– Mapping between FPC and frequency partitioning for 5MHz

| FPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 3/6 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 5-7 | Reserved |  |  |  |

[Remedy-10: Change the text from line 10 to 11 on the page 36, in 15.3.8.2.1, as follows:]

A 54 or 3-bit (TBD) field called Uplink Subband Allocation Count (USAC) determines the value of KSB depending on system bandwidth.

[Remedy-11: Insert the table in line 15 on the page 36, in 15.3.8.2.1, as follows:]

Table XXX - Mapping between USAC and KSB for 10MHz or 20MHz

| USAC | # of subbands allocated (KSB) | USAC | # of subbands allocated (KSB) |
| --- | --- | --- | --- |
| 0 | 0 | 8 | 10 |
| 1 | 1 | 9 | 12 |
| 2 | 2 | 10 | 14 |
| 3 | 3 | 11 | 16 |
| 4 | 4 | 12 | 18 |
| 5 | 5 | 13 | 20 |
| 6 | 6 | 14 | 22 |
| 7 | 8 | 15 | 24 |

Table XXX - Mapping between USAC and KSB for 5 MHz

| USAC | # of subbands allocated (KSB) | USAC | # of subbands allocated (KSB) |
| --- | --- | --- | --- |
| 0 | 0 | 4 | 4 |
| 1 | 1 | 5 | 5 |
| 2 | 2 | 6 | 6 |
| 3 | 3 | 7 | N.A |

[Remedy-12: Change the text from line 22 to 23 on the page 36, in 15.3.8.2.1, as follows:]

Figure 407 illustrates the PRU to PRUSBs and PRUMBs mapping for a 5 MHz bandwidth with SACKSB equal to 3.

[Remedy-13: Change the text from line 52 on the page 37, in 15.3.8.2.1, as follows:]

Figure 407—PRU to PRUSB and PRUMB mapping for BW=5 MHz, USACKSB=3

[Remedy-14: Change the text from line 50 on the page 38, in 15.3.8.2.2, as follows:]

Figure 408—Mapping of PRUs to PRUSBs and PPRUMBs for BW=5 MHz, USACKSB=3

[Remedy-15: Change the text in line 34 on the page 39, in 15.3.8.2.3, as follows:]

Figure 409 depicts the frequency partitioning BW=5 MHz, USACKSB=3, FPCT=2, FPS=12, and FPSC=1.

[Remedy-16: Change the text from line 59 on the page 38 to line 3 on the page 39, in 15.3.8.2.3, as follows:]

The frequency partition configuration is transmitted in the BCH in a 12 4 or 3-bit composite field called the Uplink Frequency Partition Configuration (UFPC) depending on system bandwidth. Frequency Partition Count (FPCT) defines the number of frequency partitions. Frequency Partition Size (FPSi) defines the number of PRUs allocated to FPi. FPCT and FPSi are determined from FPC as shown in Table YYY.

The UFPC consists of Frequency Partition Count (FPCT), Frequency Partition Size (FPS) and Frequency Partition Subband Count (FPSC) fields. The FPCT occupies the first 2 bits and defines the number of frequency partitions to be from 1 to 4 (TBD). The PFS occupies the next 6 bits and defines the number of PRUs, in minibands (N2) of PRUs, allocated to FPi for i>0. The Frequency Partition Subband Count (FPSC) occupies the remaining 43 bits and define the number of subbands allocated to FPi for i>0.

[Remedy-17: Insert the table in line 4 on the page 39, in 15.3.8.2.3, as follows:]

Table YYY– Mapping between UFPC and frequency partitioning for 10MHz or 20MHz

| UFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 3/6 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 5-15 | Reserved |  |  |  |

Table YYY– Mapping between FPC and frequency partitioning for 5MHz

| FPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 3/6 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 5-7 | Reserved |  |  |  |

------------------------------------------- Text End -----------------------------------------------------------

# Reference

[1] IEEE 802.16m-09/0010, “IEEE 802.16m Amendment Working Document”