| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Sleep Mode operation in 16m |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Maruti Gupta, Shantidev Mohanty, Muthaiah Venkatachalam Intel Corp. | E-mail: shantidev.mohanty@intel.com maruti.gupta@intel.com |
| Re: | “802.16m amendment text”: IEEE 802.16m-08/xxx, “Call for Contributions on Project 802.16m Draft Amendment Content”. Target topic: “UMAC-Sleep Mode operation”. |  |
| Abstract | Amendment Text for Sleep Mode operation in 16m |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Sleep Mode operation in 802.16m

Maruti Gupta, Shantidev Mohanty, Muthaiah Venkatachalam

Intel Corp.

# Introduction

# Text Proposal

================= Start of Proposed Text ========================

# 15.2.x.x. Sleep Mode

The start of the Listening Window is aligned at the sub-frame boundary.

# 15.2.x.x.2 Sleep mode operation

# 15.2.x.x.2.1 Sleep cycle operations

The parameters in the Sleep cycle (settings) are specified as follows:

- Listen Window: length of the Listen window in unit of sub-frames

- Initial Sleep Window: length of initial Sleep window in units of sub-frames

- Final Sleep Window: length of initial Sleep window in units of sub-frames

# 15.2.x.x.2.2 Sleep window operations

### 15.2.x.x.2.2.1 Sleep Cycle Operation with Fixed Listen and Sleep Window

In Fixed sleep cycle operation, both the listen and sleep windows are of fixed duration. Thus, the Initial and Final Sleep windows are of same length. The Traffic Indication Flag is set to 0, and the Extended Listen Window Flag is also set to 0.

In Fixed sleep cycle operation the AMS and the ABS exchange UL and DL traffic during the Listen Window only. The ABS buffers DL traffic during the Sleep window and sends it during Listen window. If the AMS has UL traffic, then the BW requests for UL traffic as well as UL data is also sent during Listen window only and the BS restricts UL allocations to Listen windows only. If the AMS sends requests for UL allocations during the Sleep window, then the request is also allocated during the Listen window. Otherwise, the Sleep cycle can be de-activated by the AMS sending a MOB_SLP-REQ message with De-activation request and the ABS responding with the MOB_SLP-RSP message with de-activation code. Receiving the MOB_SLP-RSP message with De-activation code causes the AMS to exit Sleep mode with immediate effect.

Figure 1: Sleep window operation when TIMF = 0 and LWEF = 0

# 15.2.x.x.2.3 Sleep Cycle Operation with Exponential Sleep and Extended Listen Window

If any of the conditions for extending the Listen window outlined in section 15.2.x.x.2.4.2, becomes true then the length of the Sleep Window is reset to the Initial Sleep window as shown in figure 5. Alternatively, the AMS may choose to send a MOB_SLP-REQ message to change the sleep cycle to different settings by activating a different SC instead as detailed in section 15.2.1.3.4. The Listen window can also be terminated using similar mechanisms as outlined in section 15.2.1.3.3

============================== End of Proposed Text ===============