| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Related to HARQ for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Sungkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | Voice : +82-42-860-6448 E-mail: cyrano@etri.re.kr |
| Re: | Re: IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” |  |
| Abstract | The contribution proposes the text of HARQ sections to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Related to HARQ for the IEEE 802.16m Amendment

Sungkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Introduction

This contribution includes the proposed text for the HARQ sections in the IEEE 802.16m Amendment. The technical text is inherited from the 802.16m SRD [1], the IEEE 802.16m SDD [2], and the IEEE P802.16 Rev2/D9 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4]. The text proposed in this contribution is based on subclause 10.2.2.2 and subclause 10.2.3 in the IEEE 802.16m SDD [3].

# HARQ operation for UL

When persistent allocation is applied to initial transmissions in uplink, HARQ retransmissions are supported in synchronous manner [2]. Fast link adaptation by AMC (Adaptive Modulation & Coding) is not easy for persistent allocation scheme. Also, the poor channel condition may cause frequent retransmissions. Figure 1 shows that an example of UL persistent allocation with HARQ retransmissions. In this figure, we assume that retransmission delay and persistent allocation period are equal to 1 frame and 4 frames respectively.

However, the resources that are locked by persistent allocation and synchronous retransmissions reduce the dynamic scheduling gain. As the persistent allocation frames are increased, there may be a shortage of resource for synchronous retransmissions. Figure 2 shows that an example of collision of the pre-allocated resources between synchronous retransmission of persistent allocation (A) and persistent allocation (B).

Figure 1. Example of UL persistent allocation with HARQ retransmissions

Figure 2. Collision of the pre-allocated resources between persistent allocation and synchronous retransmission

To avoid this matter, synchronous adaptive HARQ retransmissions can be considered. However, synchronous adaptive HARQ is required to indicate the allocation information for the HARQ retransmissions in like asynchronous scheme. Also, even if synchronous adaptive retransmission is applied, the resources for dynamic allocation or retransmission allocation can be limited because the sub-frame for synchronous retransmission doesn’t be changed. Therefore, we propose that asynchronous retransmission may be issued after N synchronous retransmissions (N=0,1,2,…). The number of synchronous retransmissions (N) may be negotiated during setup/change procedures for UL persistent allocation. In case that the number of synchronous retransmissions is equal to the maximum number of retransmissions, HARQ retransmissions of the corresponding flow are just supported in a synchronous manner. Figure 3 shows the proposed scheme in case of N=1.

Figure 3. UL HARQ scheme for Persistent Allocation

# HARQ and ARQ interactions

According to the System Description Document [2], to reduce the retransmission delay, HARQ and ARQ interactions have been dealt with. When an HARQ transmission is failed after maximum retransmission, ARQ transmitter may initiate retransmission of the corresponding ARQ Blocks. Then, MAC entity should keep a mapping table which presents the information between HARQ burst and ARQ Block (FID, ARQ Block Sequence Number, HARQ process ID, transmitted frame index, etc.). Also, an HARQ burst may include several ARQ blocks.

# Text proposal for HARQ in 802.16m amendment

============================== Start of Proposed Text ================================

Insert the following subsections at a new section 15.2:

# MAC

- 15.2.x HARQ

- 15.2.x.x HARQ Operation with Persistent and Group Allocation

For UL Persistent allocation, HARQ retransmissions are supported in a synchronous manner. To avoid collision of pre-allocated resources between persistent allocation and synchronous HARQ retransmissions, asynchronous retransmissions may be issued in uplink persistent allocation. In this case, after N synchronous retransmissions (N=0, 1, 2,...), HARQ scheme is automatically changed to the asynchronous adaptive HARQ scheme in uplink persistent allocation. When N is equal to the maximum number of retransmissions, asynchronous retransmission is not permitted. The number of synchronous retransmissions (N) may be negotiated during the setup/change procedures of persistent allocation.

- 15.2.x.y HARQ and ARQ interactions

When both ARQ and HARQ are applied for a flow, HARQ may assist ARQ transmitter to reduce the retransmission delay.

When the HARQ entity in the transmitter determines that the HARQ process was terminated with an unsuccessful outcome, the HARQ entity in the transmitter informs the ARQ entity in the transmitter about the failure of the HARQ burst. The ARQ entity in the transmitter can then initiate retransmission and re-segmentation of the ARQ blocks that correlate to the failed HARQ burst.

A mapping table, which can indicate the relation between HARQ burst and ARQ Block(s), should be managed between HARQ entity and ARQ entity internally. An HARQ burst may include several ARQ blocks.

=============================== End of Proposed Text ================================

# References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements.”

[2] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document.”

[3] IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Jan. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment.”

[5] IEEE 802.16m-08/454r1, “Hybrid ARQ Operation for IEEE 802.16.”

[6] IEEE 802.16m-08/1120r1, “ARQ operation for IEEE 802.16m.”