| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Inter-RAT Handovers in IEEE 802.16m |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Vivek Gupta, Shantidev Mohanty Intel Corporation | E-mail: vivekggupta@ieee.org |
| Re: | “802.16m amendment text”: IEEE 802.16m-08/xxx, “Call for Contributions on Project 802.16m Draft Amendment Content”. Target topic: “UMAC-Inter-RAT Handover operation”. |  |
| Abstract | This proposal describes stage-3 text for Inter-RAT Handovers in IEEE 802.16m. |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Inter-RAT Handovers in IEEE 802.16m

# Introduction

The IEEE 802.16m SDD [1] includes support for inter-RAT handovers (clause 10.3.6). The following features are supported:

- Topology acquisition of other RATs through inter-RAT advertisements and explicit unicast based query initiated by the mobile (AMS).

- Channel measurement procedures for target (non-IEEE 802.16m) RAT.

- Definition of a MAC container to support generic inter-RAT handover procedure

- Support for both dual radio (dual transmitted/dual receiver) and single radio (single transmitter/single receiver) handovers.

This proposal elaborates on these aspects and includes stage-3 text for inter-RAT handovers.

================= Start of Proposed Text ========================

### 2	Network Topology Acquisition

2.1 Inter-RAT Advertisements

The presence of other radio access technologies (RATs) is indicated using the Multi-RAT system bitmap. The Multi-RAT system bitmap is a bitmap where each bit is used to indicate the presence of a particular RAT type as described below

| Bit 0 | Bit 1 | Bit 2 | Bits 3-7 |
| --- | --- | --- | --- |
| WLAN | 3GPP | 3GPP2 | Reserved, set to 0 |

Multi-RAT System bitmap

The Multi-RAT bitmap is transmitted in the secondary super-frame header sub-packet 2 (S-SFH SP2) as shown in Figure 1. When a particular RAT type is present in the neighborhood of an IEEE 802.16m ABS, the corresponding bit in the Multi-RAT bitmap is set to 1 in the S-SFH SP2 transmitted by the ABS. When the Multi-RAT bitmap contains at least one non-zero bit indicating the presence of one or more other RATs, the S-SFH SP2 also contains a pointer known as Multi-RAT information pointer that indicates the frame number where system parameters and configuration information of the indicated RATs is present. When an AMS learns the presence of a particular RAT it receives the MAC management message containing RAT’s system parameters and configuration information in the frame specified by the Multi-RAT information pointer as shown in Figure 1. An AMS uses the information present in the Multi-RAT information MAC management message to decide whether or not to perform handoff to this RAT using RAT selection procedures.

If the AMS decides to handoff to one of the available RAT, then it uses scanning intervals or sleep intervals to perform measurements as well as other operations required to handoff to the RAT. The AMS uses the procedures of the corresponding RAT during measurement and other operations in the RAT.

Figure 1: Transmission of Inter-RAT information.

Multi-RAT Information:

The multi-RAT information includes the following:

- Network Type : the high level classification of network type. Please refer to Table F-14 in IEEE 802.21-2008 for further details.

- Network Subtype: includes release specific information and augments the network type

- Operator Identifier: Operator identifier for the network

- Frequency bands: The frequency bands supported by the RAT.

The format and information contents of the Multi-RAT information MAC management message are described below.

A single compound TLV, namely Inter-RAT Adv Type (value xxx) is used for this purpose.

The Inter-RAT TLV is a compound TLV list that specifies information for different radio access technologies defined by different standard bodies such as IEEE, 3GPP and 3GPP2.

| Type | Length | Value |
| --- | --- | --- |
| xxx | Variable | Compound |

2.1.1. Inter-RAT Adv TLV for IEEE 802.11 Radio Access Technology

This identifies the IEEE 802.11 radio access technology.

| Type | Length | Value |
| --- | --- | --- |
| xxx.1 | Variable | Compound |

2.1.1.1 Inter-RAT Adv TLV Parameters for IEEE 802.11 – Network Subtype

This specifies the network subtype for the IEEE 802.11 network.

| Type | Length | Value |
| --- | --- | --- |
| xxx.1.1 | 1 | Network Subtype as defined in IEEE 802.21-2008 Table F-14. See Annex. |

2.1.1.2 Inter-RAT Adv TLV Parameters for IEEE 802.11 - SSID

This specifies the service set identifier (SSID) of the IEEE 802.11 network.

| Type | Length | Value |
| --- | --- | --- |
| xxx.1.1 | Variable (1-32) | SSID as defined in IEEE 802.11 |

2.1.1.3 Inter-RAT Adv TLV Parameters for IEEE 802.11 – Data Rate

This specifies the maximum data rate currently used by the IEEE 802.11 network.

| Type | Length | Value |
| --- | --- | --- |
| xxx.1.4 | 4 | Maximum data rate in kb/s. |

2.1.2. Inter-RAT Adv TLV for 3GPP Radio Access Technologies

This identifies the entire 3GPP family of radio access technologies including GERAN (GSM, etc.), UTRA (UMTS, HSPA etc.) and E-UTRA (LTE).

| Type | Length | Value |
| --- | --- | --- |
| xxx.2 | Variable | Compound |

2.1.2.1 Inter-RAT Adv TLV Parameters for 3GPP -  Network Type

This specifies the Radio access technology type supported by the 3GPP network.

| Type | Length | Value |
| --- | --- | --- |
| xxx.2.1 | 1 | Network Type as specified in IEEE 802.21-2008 Table F-14. |

2.1.1.2 Inter-RAT Adv TLV Parameters for 3GPP – Network Subtype

This specifies the network subtype for the 3GPP network.

| Type | Length | Value |
| --- | --- | --- |
| xxx.2.2 | 1 | Network Subtype as defined in IEEE 802.21-2008 Table F-14. |

2.1.2.3 Inter-RAT Adv TLV Parameters for 3GPP -  Operator Id

This specifies the Public Land Mobile Network identifier for the 3GPP RAT. The PLMN Identifier is a combination of Mobile Country Code (MCC) and Mobile network Code (MNC).

| Type | Length | Value |
| --- | --- | --- |
| xxx.2.3 | Variable | As specified in 3GPP TS 23.003 |

2.1.2.4 Inter-RAT Adv TLV Parameters for 3GPP -  FrequencyBand

This specifies the frequency band used by the radio access technology.

| Type | Length | Value |
| --- | --- | --- |
| xxx.2.4 | LIST (4) | Frequency in KHz |

2.1.2.5 Inter-RAT Adv TLV Parameters for 3GPP -  ChannelBandwidth

This specifies the channel bandwidth in KHz

| Type | Length | Value |
| --- | --- | --- |
| xxx.2.5 | 4 | Channel bandwidth in KHz |

2.1.3. Inter-RAT Adv TLV for 3GPP2 Radio Access Technologies

This identifies the entire 3GPP2 family of radio access technologies including cdma2000 and cdma2000-HRPD

| Type | Length | Value |
| --- | --- | --- |
| xxx.3 | Variable | Compound |

2.1.3.1 Inter-RAT Adv TLV Parameters for 3GPP2 -  Network Type

This specifies the Radio access technology type supported by the 3GPP2 network.

| Type | Length | Value |
| --- | --- | --- |
| xxx.3.1 | 1 | Network Type as specified in IEEE 802.21-2008 Table F-14. See Annex |

2.1.3.2 Inter-RAT Adv TLV Parameters for 3GPP2 – Network Subtype

This specifies the network subtype for the 3GPP2 network.

| Type | Length | Value |
| --- | --- | --- |
| xxx.3.2 | 1 | Network Subtype as defined in IEEE 802.21-2008 Table F-14. See Annex. |

2.1.3.3 Inter-RAT Adv TLV Parameters for 3GPP2 -  Operator Id

This specifies the Public Land Mobile Network identifier for the 3GPP2 RAT. The PLMN Identifier is a combination of Mobile Country Code (MCC) and Mobile network Code (MNC).

| Type | Length | Value |
| --- | --- | --- |
| xxx.3.3 | Variable | As specified in 3GPP TS 23.003 |

2.1.3.4 Inter-RAT Adv TLV Parameters for 3GPP2 -  Band Class

This specifies the CDMA band class.

| Type | Length | Value |
| --- | --- | --- |
| xxx.3.4 | 1 | 0-9 |

2.1.3.4 Inter-RAT Adv TLV Parameters for 3GPP2 -  CarrierFrequency

This specifies the CDMA carrier frequency in KHz

| Type | Length | Value |
| --- | --- | --- |
| xxx.3.5 | 2 | 0-65535 |

2.1.3.6 Inter-RAT Adv TLV Parameters for 3GPP2 -  ChannelBandwidth

This specifies the channel bandwidth in KHz

| Type | Length | Value |
| --- | --- | --- |
| xxx.3.6 | 4 | Channel bandwidth in KHz |

2.2 Unicast based Discovery

The unicast based network discovery can be based on 802.21 MIH mechanism. MOB_MIH-MSG as described in IEEE 802.16 Rev2 can be used for this purpose. Please refer to clause 6.3.2.3.57 in IEEE 802.16 Rev2.

2.3 Target Network Discovery and Selection Procedure

During the target RAT selection process, the AMS may communicate with an information repository (such as ANDSF as defined in 3GPP [3] ) using its 16m connection to learn operator-defined rules and preferences that affect the inter-RAT handoff decisions. The handoff policy may be pre-provisioned in the AMS and may be updated when UE requests the information repository for network discovery and selection information. The target RAT discovery and selection procedure is shown as follows.

16m Access

Target-RAT Access

Information Repository

2. Target RAT information

1.AMS connected to 16m Access

3. Policy Request

4. Policy Response

5. Evaluate Target RAT(s) for HO

6. RAT(s) Information Request

7. RAT (s) Information Response

8. Negotiate scan/sleep interval with 16m ABS to perform Target-RAT(s) measurement

9. Turn on Target-RAT(s) radio, configure measurement reporting

10. Target-RAT Measurements

11. Target-RAT Selection and HO Decision

12. Handover Procedure

AMS

Figure 2. Target RAT Discovery and Selection

- The AMS is connected to 16m access network

- The AMS learns about the presence of other-RAT(s) using the Multi-RAT bitmap in S-SFH SP2 and then learns the system parameters and configuration information from the Multi-RAT information MAC management message.

- The AMS requests inter-RAT handover policy from the information repository.

- The information repository provides the updates inter-RAT handover policy to the AMS.

- The AMS evaluates target RATs for handover

- The AMS requests more information from the information repository. This can be a unicast information retrieval using MIH messages.

- The information repository provides information about target RATs as requested by the AMS.

- The AMS negotiates with the 802.16m BS about scan/sleep intervals so that it can evaluate the link connections at target RATs.

- The device turns on the other radios and configures measurement reporting for target RATs.

- The device conducts measurements and these reports are sent by the AMS to the 16m ABS for evaluation.

- The AMS selects a target RAT for handover.

- The AMS in conjunction with ABS and target access conducts the handover procedure.

3. Measurements

While the AMS is attached to the IEEE 802.16m network and is in active mode, the AMS may need to perform radio measurements on other RATs when directed by the IEEE 802.16m network. The IEEE 802.16m network will provide the AMS required neighbor cell list information and measurement controls. When needed the IEEE 802.16m ABS will be responsible for configuring and activating the measurements on the AMS via dedicated signaling message with appropriately defined IEs.

For single-radio AMSs, measurement gaps are needed to allow the AMS to switch to the other RAT and do radio measurements. These measurement gaps may be AMS controlled or network-controlled. In case of network-controlled scenarios the IEEE 802.16m ABS is responsible for configuring the gap pattern and providing it to the AMS through dedicated signaling. AMSs with a dual receiver can perform measurements on other RATs neighbor cells without tuning away from the IEEE 802.16m network.

In order to assist the IEEE 802.16m ABS, the AMS shall inform the system of its gap-related capabilities. This capability needs to be transferred along with other AMS capabilities. The AMS needs to indicate if it has a dual receiver. In cases that the measurement gaps are not required, the IEEE 802.16m ABS can configure measurements on cells of other RATs without the need to configure measurement gaps. No DL gap patterns will be required for AMSs which are capable of simultaneous reception on the involved frequency bands. No UL gap patterns will be required for AMSs which are capable of simultaneous transmission in one access and conducting measurements on another access.

3.1 Scanning

The AMS conducts scanning of neighboring target RAT cells for handover decision. Scanning is triggered by

- AMS: when serving serving channel quality on current RAT falls below a certain threshold

- ABS: the serving ABS may direct AMS to perform scanning via scanning control signaling

3.2 Measurement parameters

The AMS may measure the following parameters when considering handover to IEEE 802.11:

- RSSI: received Signal Strength Indicator

The AMS may measure the following parameters when considering handover to 3GPP/3GPP2 RATs:

- RSSI: received Signal Strength Indicator

- RSRP: Reference Signal Received Power

Please refer to [4] and [5] for further details.

3.3  Measurement Reporting

TBD.

4 Handover Procedure

The inter-RAT handover procedure will involve steps for handover preparation, handover execution and handover completion. The complete handover procedure would involve other network entities as well and would be defined by other SDOs.

4.1 Generic MAC Container for Handover Messages

The IEEE 802.16m system would provide a generic MAC container for transferring handover messages between he AMS and ABS. The messages carried by this container would be defined elsewhere.

4.1.1 Inter-RAT-HO Handover Message

The inter-RAT handover message would be transmitted on the primary connection. The message shall include the following parameters encoded as TLV tuples.

| Parameter | Length | Value |
| --- | --- | --- |
| Handover Type | 8 bits | 0 : WiFi – WiMAX Handover 1 : 3GPP – WiMAX Handover 2 : 3GPP2 – WiMAx Handover 3-7: Reserved |
| Handover Frame | Variable | As specified by WiMAX Forum |

| Syntax | Size | Notes |
| --- | --- | --- |
| Inter-RAT-HO_Message_Format() { |  |  |
| Management Message Type = xx | 8 bits |  |
| TLV encoded information | variable | TLV specific |
| } |  |  |

4.2 Dual Transmitter/Dual Receiver based Handovers

In this case a dual radio device can receive and transmit simultaneously on both the radio modules. Since both the radios can be active simultaneously in these types of devices, the target radio directly connects to the target network to prepare resources while maintaining the connection with source network during handover.

4.3 Single Transmitter/Single Receiver based Handovers

In this case a dual radio device can receive and transmit on only one radio at a time. This is usually the mode of operation when radio frequencies are close to each other (e.g. IMT 2000 bands). Since only one radio can be active at a time in these types of devices, they use the source radio and the back-end connection between the source and target network to prepare the target network for handover. The control signalling and flow for single transmitter/receiver based handovers are shown below.

Authentication

Pre-Registration

Context Transfer

AMS

802.16m ABS

WiMAX ASN

Handover Command

Control Signaling through MAC Container

Target Network

AMS

802.16 ABS

WiMAX ASN

Target Network

AS/ AAA

15. Release Source Connection and Resources

1. AMS Connected. Uplink/Downlink Traffic

7. Target Context Request

8. Target Context Response

13. Proxy Binding Update (PMIP)

14. Proxy Binding Ack (PMIP)

Common GW/ HA

12. Target Network Entry PHY/MAC Establishment

2. Target Radio ON

11. Handover to Target Command

Target Connected. Uplink/Downlink Traffic

9. Target Resource Preparation

10. Target Handover Response

5. Target Registration Request

6. Target Authentication

Authentication & Authorization

4. Decision to HO

3. Measurement Reporting

Figure xxx. Single radio handover procedure

============================== End of Proposed Text ===============

References

[1] IEEE 802.16m System Description Document (Draft r6), December 2008.

[2] IEEE Std. 802.16e-2005, IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems, Amendment 2: Physical and Medium Access Control Layers for Combined Fixed and Mobile Operation in Licensed Bands, and P802.16Rev2/D8 (December 2008)

[3] IEEE P802.21-2008: “IEEE Standard for Local and Metropolitan Area Networks:  Media Independent Handover Services ", IEEE LAN/MAN Standards committee of IEEE Computer Society, Dec, 2008

[4] 3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description.

[5] 3GPP TR 36.938: “Improved Network Controlled Mobility between E-UTRAN and 3GPP2/Mobile WiMAX Radio Technologies”.

[6] 3GPP TS 23.402: "Architecture Enhancements for non-3GPP accesses".

[7] 3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3".